Artifex ProCal — Project Progress Report

Professional Display Calibration Suite | April 25, 2026

# Executive Summary

Artifex ProCal is a professional-grade display calibration software suite competing with CalMAN Ultimate and Light Illusion's ColourSpace INF. The project follows a phased implementation approach, with Phases 0 through 3c now complete. The software is built using Tauri (Rust backend + React/TypeScript frontend) and targets cross-platform deployment on macOS, Windows, and Linux.

All backend calibration, color science, hardware abstraction, display protocol, and meter driver crates have been implemented and tested. The remaining work focuses on frontend integration, report generation, advanced calibration workflows, and hardware-in-the-loop validation.

# Phase Overview

| Phase | Name | Status | Key Deliverables |
| --- | --- | --- | --- |
| Phase 0 | Project Scaffold | Complete | Tauri 2.x, React 19, Rust workspace |
| Phase 1 | Color Science | Complete | XYZ/Lab/xyY, DeltaE 2000, Bradford adaptation, gamma curves, matrix transforms |
| Phase 2 | Hardware Abstraction Layer | Complete | Meter, DisplayController, PatternGenerator traits + mock implementations |
| Phase 3a | Calibration Engine | Complete | Session manager, patch sequencer, measurement loop, AutoCal flow, LUT generation |
| Phase 3b | Network Protocols (LG OLED) | Complete | SSAP WebSocket, SSDP discovery, pairing, calibration commands, iTPG, PGenerator |
| Phase 3c | Meter Drivers | Complete | i1 Display Pro (HID+unlock), i1 Pro 2 (USB bulk), ArgyllCMS adapter, init tracker |
| Phase 4 | Tauri IPC & Frontend | Pending | Tauri commands/events, React UI, CIE diagrams, wizard flows |
| Phase 5 | Report Engine | Pending | PDF/HTML report generation, templates, pre/post comparison |
| Phase 6 | Advanced Workflows | Pending | 3D LUT wizard, profiling, meter correction matrices |
| Phase 7 | Testing & QA | Pending | Hardware-in-the-loop validation, visual regression, CI |

# Phase 0 & 1: Project Scaffold & Color Science

Status: Complete | Commits: 16 | Tests: All Passing

## Phase 0: Tauri 2.x Project Scaffold

Tauri 2.x application with Rust backend and React/TypeScript frontend

React 19 + TypeScript + Tailwind CSS + Vite build pipeline

Three.js integrated for GPU-accelerated scientific visualization

SQLite via rusqlite for embedded database

Rust workspace with multi-crate architecture

## Phase 1: Color Science Crate

Color space types: XYZ, xyY, Lab, LCh, RGB (linear/gamma), WhitePoint

XYZ ↔ xyY bidirectional conversions

XYZ ↔ Lab (CIELAB) conversions with D65 reference white

CIEDE2000 DeltaE formula with Sharma reference test suite

Bradford chromatic adaptation (CAT02 preparation)

Gamma curves: power-law 2.2/2.4, BT.1886, PQ (ST.2084), HLG

RGB ↔ XYZ matrix transforms for BT.709, BT.2020, DCI-P3

Wired into Tauri IPC for frontend consumption

# Phase 2: Hardware Abstraction Layer (HAL)

Status: Complete | Commits: 10 | Tests: All Passing

## Core Traits

Meter trait: connect(), disconnect(), read_xyz(), set_mode(), model()

DisplayController trait: connect(), set_picture_mode(), upload_1d_lut(), upload_3d_lut(), set_white_balance()

PatternGenerator trait: connect(), display_patch(), display_pattern()

All traits require Send + Sync for multi-threaded measurement loops

## Supporting Types

Lut1D, Lut3D, RGBGain, PictureMode, MeterMode, DisplayError, PatternGenError

## Mock Implementations

MockMeter: returns programmable XYZ values for integration testing

MockDisplayController: records all uploaded LUTs and settings

MockPatternGenerator: logs all displayed patches

End-to-end measurement and upload test verifies full pipeline

## Skeleton Devices

LG OLED skeleton with network control placeholders

Sony VPL-VW385ES projector skeleton with RS-232 placeholders

PGenerator skeleton with HTTP API placeholders

X-Rite i1 Pro 2 skeleton with USB placeholders

LG internal pattern generator skeleton

# Phase 3a: Calibration Engine

Status: Complete | Commits: 12 | Tests: All Passing

## Crate Structure

calibration-core: state, events, errors, patches, measurement loop

calibration-storage: SQLite schema, SessionStore, ReadingStore

calibration-autocal: greyscale analyzer, 1D LUT generator

calibration-engine: event channel, AutoCal flow state machine

## Core Types (Task 1–3)

CalibrationState: enum for idle, measuring, analyzing, uploading, complete

CalibrationEvent: enum for device connected, patch displayed, reading taken, error occurred

Patch: RGB value + target XYZ + measured XYZ + dE + timestamp

PatchSet: greyscale ramp (21/51/101/256-point), primaries, secondaries, saturation sweeps

MeasurementLoop: configurable sample count (N=3–10), dark reading subtraction, outlier rejection

## Storage (Task 4–6)

SQLite schema: sessions, readings, displays, meters, profiles tables

SessionStore: CRUD for calibration sessions with metadata

ReadingStore: per-reading persistence with JSON blob for XYZ data

Migration system for schema versioning

## AutoCal (Task 7–8)

GreyscaleAnalyzer: computes RGB balance errors, gamma deviation, CCT drift per step

1D LUT Generator: per-channel curve fitting (spline, polynomial, lookup table)

Supports greyscale-only (RGB combined) and per-channel independent modes

LUT sizes: 256, 512, 1024 entries

## Engine (Task 9–10)

Tokio broadcast event channel for real-time frontend updates

AutoCal Flow State Machine: Display Selection → Meter Setup → Pattern Gen Choice → Pre-Measurement → Calibration → Post-Measurement

Pause/resume capability with SQLite state persistence

Rollback to any previous step

## Integration & Testing (Task 11–12)

Golden Path Integration Test: mock meter → measure greyscale → analyze → generate LUT → verify

Full test suite: all 4 calibration crates, 100% of unit tests passing

# Phase 3b: Network Protocols (LG OLED & PGenerator)

Status: Complete | Commits: 14 | Tests: All Passing

## hal-displays Crate: LG OLED

SSAP Protocol Types: WebSocket message structures for all calibration commands

SSDP Discovery: UDP multicast M-SEARCH to 239.255.255.250:1900

PIN-based Pairing: TV displays passcode → client sends via WebSocket → stores client key

Calibration Commands: start_calibration(), end_calibration(), upload_1d_lut(), upload_3d_lut(), set_tonemap_params()

DeviceControl HTTP Client: SSAP over WebSocket with automatic reconnection

Dual-Mode Controller: high-level API wrapping DeviceControl with picture mode switching

iTPG Control: internal pattern generator for 2019+ LG OLEDs (10-bit RGB, window/patch sizing)

Dolby Vision config upload support

3x3 gamut matrix upload support

## hal-patterns Crate: PGenerator

HTTP client for PGenerator 1.6 on Raspberry Pi 4

GET /patch?r=G=B= endpoint for 8-bit RGB patches

probe() method for connectivity verification

Dual-Mode Controller: switches between PGenerator and LG iTPG

Ted's LightSpace CMS Calibration Disk template catalog

XML protocol serialization for disk-based pattern sequences

## Integration & Testing

Mock network device tests for all protocol commands

Integration test with simulated LG OLED responses

Full test suite: all network protocol crates green

# Phase 3c: Meter Drivers

Status: Complete | Commits: 9 | Hardware Verified on macOS

## i1 Display Pro Rev.B

Native HID driver using hidapi crate (Linux/Windows)

Challenge-response unlock protocol reverse-engineered from ArgyllCMS

11 known OEM keys: Retail, ColorMunki, Generic OEM, NEC, HP, Wacom, Barco, Crysta, ViewSonic, Quato, TPA-1

User's device is Generic OEM variant (key 0xCAA62B2C / 0x30815B61) — verified on physical hardware

I1D3_ESCAPE env var for unknown OEM variants

macOS fallback: ArgyllCMS spotread -c 1 via PTY-based expect script

ArgyllCMS required on macOS due to IOKit HID driver quirk post-unlock

## i1 Pro 2 Spectrophotometer

Native USB bulk transfer using rusb crate (Linux/Windows)

Endpoints: 0x01 OUT (command), 0x82 IN (response), 0x84 IN interrupt, 0x85 IN bulk

Spectrophotometer trait: read_spectrum() returns 36-wavelength data (380–730nm)

initialize() for white reference calibration

macOS fallback: ArgyllCMS spotread -c 2 via same adapter

macOS requires ArgyllCMS because IOUSBHost claims the device

## Shared Infrastructure

SyncHidDevice wrapper with unsafe impl Send + Sync for Meter trait bounds

SyncUsbDevice wrapper with unsafe impl Send + Sync

send_command_u16() for 16-bit command codes (i1d3 unlock protocol)

ArgyllMeter struct with expect script generation for interactive spotread

Error types: NotFound, DeviceUnavailable, MeasurementFailed, CalibrationRequired, Timeout

## Init Tracker & Profiling

SQLite-backed InitTracker with meter_initializations table

3-hour white patch requirement tracking for i1 Pro 2

Meter profiling foundation: correction matrix generation (3x3) from spectrophotometer reference

Profiling test with identity matrix verification

# Remaining Phases & Future Work

## Phase 4: Tauri IPC & Frontend Integration

Wire the Rust backend into the React frontend via Tauri commands and events.

Tauri command handlers for: connect_meter(), start_calibration(), get_reading(), export_lut()

Tauri event streams for: measurement_progress, calibration_state_change, error_occurred

Zustand store for global frontend state

React Query for backend synchronization

CIE 1931/1976 diagram component with Three.js/WebGL

3D LUT cube visualization with rotation and error heatmap

Grayscale tracker: RGB balance bars, gamma curve overlay, CCT tracking

Real-time measurement monitor: live XYZ/xy readings, stability meter

## Phase 5: Report Engine

Generate professional calibration reports in PDF and HTML formats.

PDF generation via printpdf or headless Chrome

Built-in templates: Quick Report, Detailed, Netflix Validation

Pre/post comparison charts with dE analysis

Custom layout designer with drag-and-drop (future)

Export formats: PDF, HTML, CSV raw data

## Phase 6: Advanced Workflows

Implement the remaining calibration workflows beyond basic AutoCal.

3D LUT Wizard: 33³/65³ patch generation → measurement → tetrahedral/trilinear interpolation → export (.cube, .3dl, .xml, .dat)

Manual Calibration: live feedback for manual TV adjustment with before/after meter readings

Profiling Wizard: display characterization (native gamut, white/black points, gamma response)

Meter Profiling: correction matrix (.ccmx) generation from i1 Pro 2 reference + i1 Display Pro

Dolby Vision config generation and upload

HDR tone curve fitting and validation

## Phase 7: Testing & QA

Comprehensive validation before V1 release.

Hardware-in-the-loop validation with real LG OLED + i1 Display Pro

Visual regression testing for CIE diagrams and LUT cubes

CI pipeline with GitHub Actions for all Rust crates

Cross-platform build verification (macOS, Windows, Linux)

Beta testing with user's hardware lab

## Post-MVP Future Expansion

Additional Displays: Samsung QLED, Panasonic OLED, Flanders Scientific, EIZO ColorEdge

Additional Meters: Klein K-10, Jeti Specbos, Photo Research PR-series, Portrait C6 HDR2000

Additional Pattern Generators: VideoForge Pro, Murideo Six-G, DaVinci Resolve Pattern Generator

Video Wall Matching: multi-panel calibration for Planar/Barco walls

Netflix Validation Workflows: specific pre/post checks for post-production mastering

Cloud Sync: calibration history sync across workstations

Floating licenses for facility deployments

# Appendix: Current Repository Structure

calibration-suite/
├── crates/
│   ├── color-science/       # XYZ, Lab, DeltaE, gamut math, tone curves
│   ├── hal/                 # Hardware abstraction traits + mock impls
│   ├── hal_meters/          # i1 Display Pro, i1 Pro 2, ArgyllCMS adapter
│   ├── hal_displays/        # LG OLED (SSAP/WebSocket), Sony projector skeleton
│   ├── hal_patterns/        # PGenerator, LG iTPG, Ted's disk templates
│   ├── calibration-core/      # State, events, patches, measurement loop
│   ├── calibration-storage/   # SQLite schema, SessionStore, ReadingStore
│   ├── calibration-autocal/   # Greyscale analyzer, 1D LUT generator
│   ├── calibration-engine/    # Event channel, AutoCal flow state machine
│   └── ipc/                 # Tauri command handlers (skeleton)
├── src/                     # React 19 + TypeScript frontend
├── docs/superpowers/
│   ├── specs/               # Design specifications
│   └── plans/               # Implementation plans
├── tests/                   # Integration tests
├── Cargo.toml             # Rust workspace
├── package.json           # Frontend dependencies
└── tauri.conf.json        # Tauri configuration