# ArtifexProCal — Comprehensive Progress Report

Professional Display Calibration Suite — LG OLED & Sony Projector

Date: April 29, 2026

# Executive Summary

ArtifexProCal is a professional-grade display calibration software suite competing with CalMAN Ultimate and ColourSpace INF. The project has completed twelve major implementation phases spanning color science, hardware abstraction, calibration engine core, network protocols, meter drivers, Tauri IPC integration, dashboard shell, calibration wizard, visualization engine, 3D LUT generation, and session history management.

# Project Metrics

| Metric | Value |
| --- | --- |
| Total Rust Source Files (crates/) | 110 |
| Total Rust Lines of Code (crates/) | 8,267 |
| Total Rust Source Files (src-tauri/) | 13 |
| Total Rust Lines of Code (src-tauri/) | 1,628 |
| Total TypeScript/TSX Files (src/) | 55 |
| Total TypeScript Lines of Code (src/) | 3,799 |
| Rust Unit/Integration Tests Passing | 197 |
| Frontend Component Tests Passing | 8 |
| Crates in Workspace | 14 |
| Phases Completed | 12 of 18 |
| Git Branches Merged to Main | 7 |

# Completed Phases

## Phase 0: Project Scaffolding

Status: COMPLETED  |  Date: Early April 2026

Initial project setup with Tauri 2.x, React 19 + TypeScript, Tailwind CSS, and Rust workspace structure.

Key Deliverables:

Tauri 2.x app scaffold with Vite build system

Rust workspace with 14 crates

React 19 + TypeScript + Tailwind CSS frontend

Vitest testing framework

CLAUDE.md project instructions and design spec

## Phase 1: Color Science Foundation

Status: COMPLETED  |  Date: April 2026

Core color science library with XYZ/Lab/xyY/RGB conversions, DeltaE 2000, Bradford chromatic adaptation, and gamma curves.

Key Deliverables:

color_science crate with comprehensive type system (XYZ, xyY, Lab, LCh, RGB, WhitePoint)

CIELAB conversion pipeline (XYZ ↔ Lab, XYZ ↔ xyY)

DeltaE 2000 implementation with Sharma reference dataset validation (24 pairs)

Bradford chromatic adaptation (D65 ↔ D50)

RGB ↔ XYZ matrix transforms for BT.709 and BT.2020

Gamma curves: Power Law 2.2/2.4, BT.1886, sRGB, PQ (ST.2084), HLG

30 integration tests covering all conversion paths

## Phase 2: Hardware Abstraction Layer (HAL)

Status: COMPLETED  |  Date: April 2026

Trait-based plugin architecture for meters, displays, and pattern generators with mock implementations for testing.

Key Deliverables:

hal crate with Meter, DisplayController, and PatternGenerator traits

Mock implementations: FakeMeter, FakeDisplayController, FakePatternGenerator

Supporting types: Lut1D, Lut3D, RGBGain, PictureMode

Error types: MeterError, DisplayError, PatternGenError

Device skeletons: LG OLED, Sony Projector, i1 Display Pro, i1 Pro 2, PGenerator

Shared IP validation utility

End-to-end mock measurement and upload test

## Phase 3a: Calibration Engine Core

Status: COMPLETED  |  Date: April 2026

Core calibration engine with state machine, patch sequencing, measurement loop, storage, and AutoCal logic.

Key Deliverables:

calibration_core crate: CalibrationState, CalibrationEvent, CalibrationError, SessionConfig

Greyscale patch set generation with configurable count (5-33 points)

Measurement loop with dark reading subtraction, repeated samples, outlier rejection

Reading statistics (mean, std dev)

calibration_storage crate: SQLite schema, Storage struct, SessionStore, ReadingStore

calibration_autocal crate: GreyscaleAnalyzer with gamma estimation, white balance errors, per-channel corrections

1D LUT Generator (256-1024 entries) from per-channel corrections

calibration_engine crate: EventChannel (tokio broadcast), GreyscaleAutoCalFlow state machine

Golden path integration test: end-to-end calibration with mocks

Full test suite: 12+ tests across all 4 calibration crates

## Phase 3b: Network Protocols (LG OLED & PGenerator)

Status: COMPLETED  |  Date: April 2026

Complete LG OLED network calibration protocol implementation including SSAP over WebSocket, SSDP discovery, PIN pairing, and calibration commands. PGenerator HTTP API and Ted's LightSpace disk support.

Key Deliverables:

hal_displays crate: LG OLED SSAP protocol message types

SSDP discovery (M-SEARCH, response parsing)

PIN-based pairing flow with state machine

Calibration commands: start/end calibration, 1D/3D LUT upload, white balance, tone mapping

DeviceControl HTTP client for LG OLED

Dual-mode LG OLED controller (Direct WebSocket + DeviceControl)

iTPG (internal pattern generator) control for 2019+ models

hal_patterns crate: PGenerator HTTP API client

PGenerator DeviceControl mode support

Ted's LightSpace CMS Calibration Disk pattern catalog (26 patterns)

XML protocol serialization for pattern definitions

Integration tests with mock network devices

Full test suite: 25+ tests

## Phase 3c: Meter Drivers (X-Rite i1 Series)

Status: COMPLETED  |  Date: April 2026

Native HID drivers for X-Rite i1 Display Pro Rev.B and i1 Pro 2 spectrophotometer with cross-platform support.

Key Deliverables:

hal_meters crate with shared HID utilities

X-Rite command and status constants

i1 Display Pro driver with challenge-response unlock protocol

i1 Pro 2 driver with spectrophotometer trait

ArgyllCMS adapter for macOS/Windows cross-platform compatibility

SyncHidDevice wrapper for thread-safe HID communication

Initialization tracker for device warm-up management

Hardware test suite (ignored unless physical device present)

## Phase 4a: Tauri IPC + Dashboard Shell

Status: COMPLETED  |  Date: April 2026

Full Tauri IPC layer with type-safe commands/events via specta, CalibrationService, and React dashboard shell.

Key Deliverables:

src-tauri: IPC models with specta derives (AppState, DeviceInfo, SessionConfigDto, etc.)

CalibrationService with device inventory, session tracking, mock/real driver support

Tauri command handlers: connect_meter, connect_display, start_calibration, abort_calibration, etc.

Event emitters: calibration_progress, analysis_complete, lut_uploaded, verification_complete

tauri-specta bindings generation for TypeScript

Frontend: Zustand store with Tauri event hooks

DashboardShell with Sidebar (collapsible), TopBar, StatusFooter

React Router with routes: /dashboard, /calibrate, /devices, /history, /settings

Component tests: DashboardShell, Sidebar

Backend tests: CalibrationService unit tests, command tests

## Phase 4b: Calibration Wizard

Status: COMPLETED  |  Date: April 2026

Step-by-step calibration wizard with device selection, target configuration, live measurement, analysis, upload, and verification.

Key Deliverables:

CalibrationWizard shell with stepper UI (DeviceSelection → TargetConfig → Measurement → Analysis → Upload → Verify)

DeviceSelectionStep: display and meter selection with connection status

TargetConfigStep: color space, gamma, white point, patch count, tier selection

MeasurementStep: live chart (Recharts), RGB readout, dE2000 computation per patch

AnalysisStep: dE bar chart, gamma curve overlay, summary cards, CIE diagram integration

UploadStep: LUT upload progress, display verification

VerifyStep: post-calibration measurement, before/after comparison

ProfilingStep and MeterProfiler: meter correction matrix generation workflow

Shared chart components: LiveGammaChart, DeBarChart, PatchDataTable, YxyReadout

Integration into CalibrateView with wizard state management

## Phase 4c: Visualization Engine

Status: COMPLETED  |  Date: April 2026

GPU-accelerated scientific visualization with CIE diagrams, grayscale tracker, color checker grid, and Three.js scaffold.

Key Deliverables:

Backend: CIE 1931 spectral locus data (470 points) and 1976 u'v' conversion

Backend: get_spectral_locus and get_target_gamut IPC commands

Frontend: @react-three/fiber and @react-three/drei integration

CIEDiagram Canvas component with WebGL rendering

GrayscaleTracker SVG component for RGB balance visualization

ColorCheckerGrid component (24/140 patch display)

colorMath.ts utility library for frontend color calculations

Three.js scaffold for 3D LUT cube visualization

Integration into AnalysisStep for real-time display

## Phase 5: End-to-End Integration

Status: COMPLETED  |  Date: April 2026

Wiring the complete calibration pipeline from frontend wizard through backend service to engine execution with real event streaming.

Key Deliverables:

Engine: run_sync signature updated to accept trait objects and Storage

Backend: CalibrationService.run_calibration() with event bridge

Backend: PatternGenerator and Storage integration in CalibrationService

Backend: Abort flag for cancellation support

Backend: CalibrationEvent → Tauri event bridge (emit_engine_event)

IPC: Wire real start_calibration and abort_calibration commands

Frontend: MeasurementStep populates real RGB readings

Frontend: AnalysisStep listens for analysis-complete event

Integration test: full calibration with mocks (meter + display + pattern gen + storage)

## Phase 6: 3D LUT Generation

Status: COMPLETED  |  Date: April 2026

Complete 3D LUT pipeline from sparse measurement patches through tetrahedral interpolation to .cube/.3dl file export.

Key Deliverables:

Core: CalibrationTier enum (GrayscaleOnly, GrayscalePlus3D, Full3D)

Core: PatchStrategy with optimized subset generation (grayscale-only, grayscale+3D, full 3D)

calibration_autocal: TetrahedralInterpolator for 3D LUT lookup

calibration_autocal: Lut3DEngine for sparse-to-dense 3D LUT computation (33³)

calibration_autocal: 33³ → 17³ downsampling for Alpha 7 chipsets

calibration_autocal: .cube and .3dl file export formats

calibration_engine: Lut3DAutoCalFlow state machine

IPC: generate_3d_lut and export_lut commands

Frontend: Calibration tier selector in TargetConfigStep

Frontend: 3D LUT Analysis Tab with real data

Frontend: LutCubeScene updated with real LUT data props

Integration tests: end-to-end 3D LUT flow (grayscale-only, full3d, Alpha 7 downsampling)

## Phase 7a: Session History

Status: COMPLETED  |  Date: April 29, 2026

Full session history system with SQLite schema migrations, query layer, CSV/JSON export, and rich frontend views.

Key Deliverables:

Storage: SQLite migration v2 with ended_at, tier, patch_count columns; lut_3d_size, lut_3d_json in computed_results

Storage: SessionQuery with list/pagination, get_detail, and filter support

Storage: SessionExporter with streaming CSV and JSON writers

Storage: Query layer tests (7 tests: pagination, filtering, detail retrieval)

IPC: SessionSummaryDto, SessionDetailDto, SessionFilterDto, SessionListResponse

IPC: list_sessions, get_session_detail, export_session_data commands

Frontend: SessionTable component with sorting, pagination, tier badges

Frontend: SessionDetailView with summary cards, readings table, tabs

Frontend: SessionCompareView with side-by-side metric comparison and delta coloring

Frontend: ExportMenu dropdown for CSV/JSON export

Frontend: HistoryView with list/detail/compare routing, filters, IPC integration

Integration test: history_full_flow_list_detail_export (end-to-end with mocks)

# Remaining Phases

## Phase 7b: Report Generation (PDF/HTML)

Priority: HIGH

Professional calibration report generation from session data. Users can select templates (Quick Summary, Detailed, Pre/Post Comparison) and export as PDF or HTML. Reports include CIE diagram SVG embeds, grayscale plots, dE tables, and LUT metadata.

Planned Scope:

reporting crate with HTML template engine (inline CSS)

PDF generation via printpdf or genpdf

Built-in templates: Quick Summary, Detailed, Pre/Post Comparison

Report preview in SessionDetailView (HTML iframe)

IPC command: generate_report(session_id, template, format)

Embeds: CIE diagram SVG, grayscale plot, dE tables, LUT metadata

## Phase 8: Manual Calibration Mode

Priority: HIGH

Live feedback mode for manual TV adjustment. User changes settings on the display and sees real-time meter readings with before/after comparison. Essential for service menu calibration and CMS saturation adjustment.

Planned Scope:

Continuous measurement loop with configurable interval

Real-time XYZ/xy/dE readout display

Before/after snapshot comparison

Target overlay with tolerance bands

Support for RGB gain, RGB bias, CMS saturation/hue controls

## Phase 9: Device Profiling

Priority: MEDIUM

Display characterization and meter correction matrix generation. Measures native RGB primaries, white/black points, gamma response. Generates .ccmx and .ccss profiles.

Planned Scope:

Display characterization: native gamut measurement, peak luminance, gamma tracking

Meter profiling: correction matrix (.ccmx) generation using spectrophotometer reference

Profile storage and management

Profile export: .ccmx, .ccss, .icc formats

## Phase 10: Netflix / Post-Production Validation

Priority: MEDIUM

Specific validation workflows for post-production mastering. Includes Netflix-calibrated display validation, HDR10/Dolby Vision compliance checks, and custom target validation.

Planned Scope:

Netflix validation preset (BT.2020/PQ, specific dE thresholds)

HDR10 compliance checks (EOTF tracking, maxCLL, maxFALL)

Dolby Vision config validation

Custom target creation and validation

## Phase 11: Additional Hardware Support

Priority: LOW

Expand beyond LG OLED and Sony projector to additional displays, meters, and pattern generators.

Planned Scope:

Displays: Samsung QLED (2019-2026), Panasonic OLED, Flanders Scientific, EIZO ColorEdge

Meters: Klein K-10, Jeti Specbos, Photo Research PR-series, Portrait C6 HDR2000

Pattern Generators: VideoForge Pro, Murideo Six-G, DaVinci Resolve Pattern Generator

## Phase 12: Cloud Sync & Multi-Workstation

Priority: LOW

Optional cloud sync for calibration history across workstations. Floating licenses for facility deployments.

Planned Scope:

Calibration history cloud sync

Multi-workstation license management

Team collaboration features

# Workspace Crate Inventory

| Crate | Purpose | Status |
| --- | --- | --- |
| color-science | XYZ/Lab/xyY/RGB conversions, DeltaE, gamut math, tone curves, CIE diagrams | Complete |
| hal | Hardware abstraction traits (Meter, DisplayController, PatternGenerator), mocks | Complete |
| hal-displays | LG OLED controller, SSAP protocol, SSDP discovery, pairing, calibration commands | Complete |
| hal-patterns | PGenerator client, Ted's LightSpace disk catalog, XML protocol | Complete |
| hal_meters | X-Rite i1 Display Pro and i1 Pro 2 HID drivers, unlock protocols | Complete |
| calibration-core | CalibrationState, events, errors, SessionConfig, patches, measurement loop | Complete |
| calibration-storage | SQLite schema, SessionStore, ReadingStore, SessionQuery, SessionExporter | Complete |
| calibration-autocal | GreyscaleAnalyzer, Lut1DGenerator, TetrahedralInterpolator, Lut3DEngine | Complete |
| calibration-engine | EventChannel, GreyscaleAutoCalFlow, Lut3DAutoCalFlow | Complete |
| profiling | Meter profiling, correction matrix generation (foundation) | Foundation |
| reporting | PDF/HTML report generation | Not started |
| artifexprocal (src-tauri) | Tauri app, IPC commands, CalibrationService, event emitters | Complete |

# Test Coverage Summary

| Crate/Area | Tests | Passing | Notes |
| --- | --- | --- | --- |
| color-science | 35 | 35 | Property-based DeltaE, conversion roundtrips |
| hal | 26 | 26 | Mock end-to-end, trait compilation, error types |
| hal-displays | 29 | 29 | SSAP, SSDP, pairing, calibration, iTPG |
| hal-patterns | 9 | 9 | PGenerator, DeviceControl, catalog, XML |
| hal_meters | 19 | 19 | i1d3 unlock, command codes, HID utilities |
| calibration-core | 14 | 14 | State transitions, patches, measurement loop |
| calibration-storage | 16 | 16 | Schema, CRUD, query, export |
| calibration-autocal | 17 | 17 | Analyzer, LUT gen, 3D LUT, tetrahedral, export |
| calibration-engine | 18 | 18 | AutoCal flow, LUT3D flow, events, network, integration |
| artifexprocal (src-tauri) | 16 | 16 | Service state, IPC commands, integration |
| Frontend Components | 8 | 8 | SessionTable, DetailView, CompareView (Vitest) |

Total: 197 Rust tests + 8 frontend component tests = 205 tests passing

# Architecture Summary

The project follows a modular Rust workspace with a Tauri backend and React/TypeScript frontend. The architecture is designed for cross-platform deployment (macOS, Windows, Linux) with a hardware-first development approach using real devices for testing.

Key Architectural Decisions:

SQLite (embedded) for session persistence — no external database required

Tauri Commands + Events for IPC — type-safe, async, bidirectional

specta for TypeScript binding generation — eliminates manual type maintenance

Trait-based HAL plugin system — new devices can be added without modifying core

Event-driven calibration engine — frontend reacts to real-time backend events

Three.js/WebGL for scientific visualization — GPU-accelerated CIE diagrams and 3D LUT cubes

Zustand for frontend state — lightweight, no boilerplate

Offline-first — no account validation, entirely local operation

# Risk Assessment & Mitigation

| Risk | Impact | Mitigation |
| --- | --- | --- |
| webOS 7.3+ protocol changes | High | Monitor LG firmware updates; maintain protocol version detection |
| HID driver cross-platform compatibility | Medium | ArgyllCMS adapter for macOS/Windows; native HID for Linux |
| Measurement speed vs. accuracy tradeoff | Medium | Configurable integration time; fast default + precision mode |
| 3D LUT size memory constraints | Low | Streaming generation; downsampling for lower-end chipsets |
| PDF report generation performance | Low | Async generation with progress events; template caching |

# Recommended Next Steps

Based on the design specification and competitive analysis against CalMAN Ultimate and ColourSpace INF, the recommended priority order for remaining phases is:

1. Phase 7b: Report Generation — Highest impact for user value. Professional reports are a core differentiator.

2. Phase 8: Manual Calibration Mode — Essential for service menu work and CMS adjustment. Fills a gap in the current wizard-only workflow.

3. Phase 9: Device Profiling — Enables meter correction matrices and display characterization. Required for accurate colorimeter readings on OLED displays.

4. Phase 10: Netflix Validation — Post-production workflow validation. Medium priority until post-production use case is confirmed.

Generated by Claude Code (Anthropic) on April 29, 2026