# Phase 3b: Network Protocols Design Spec

Date: 2026-04-24

Scope: LG OLED AutoCal protocol (SSAP/WebSocket) and PGenerator 1.6 pattern generator (TCP XML / HTTP / DeviceControl)

Approach: reqwest::blocking, dual-mode for both devices

# 1. Goal

Implement real network protocol drivers for the two primary hardware devices in the calibration loop:

LG OLED TV — SSAP over WebSocket for calibration commands, LUT upload, and iTPG control

PGenerator 1.6 — TCP XML protocol (LightSpace Network Calibration Protocol) for patch display and pattern sequencing

Both devices support dual-mode operation:

Direct mode — Our app talks directly to the device (offline-first, no external dependencies)

DeviceControl mode — Our app talks to LightSpace DeviceControl running locally, which proxies commands to the device

# 2. Architecture

## New Crates

| Crate | Responsibility | Devices |
| --- | --- | --- |
| hal-displays | LG OLED display controller | LG OLED (direct + DeviceControl) |
| hal-patterns | Pattern generator controller | PGenerator 1.6 (direct + DeviceControl) |

# 3. PGenerator Pattern Generator

## 3.1 Dual-Mode Architecture

pub enum PGeneratorMode {
    Direct { ip: String, port: u16 },      // TCP port 85 (XML)
    DeviceControl { port: u16 },           // HTTP localhost:81
}

pub struct PGeneratorController {
    mode: PGeneratorMode,
    connected: bool,
    stream: Option<TcpStream>,  // Direct mode only
}

## 3.2 Direct Mode — TCP Port 85 (LightSpace XML Protocol)

Connection: TCP socket to pi-ip:85

XML Commands:

Display solid color patch:

<patch><r>128</r><g>64</g><b>32</b></patch>

Play pre-encoded disk pattern:

<pattern><name>21-Point Grayscale</name><chapter>1</chapter></pattern>

Black patch (disconnect):

<patch><r>0</r><g>0</g><b>0</b></patch>

## 3.3 DeviceControl Mode — HTTP localhost:81

POST /api/display_pattern — Body: {"name": "21-Point Grayscale"}

POST /api/display_patch — Body: {"r": 128, "g": 64, "b": 32}

GET /api/list_patterns — Returns full catalog

## 3.4 Ted's LightSpace Disk Pattern Catalog

Pre-Calibration Tools:

Brightness Patterns, Contrast Patterns, Color Temperature pattern, Black & White Pattern, Sharpness Pattern

Display Characterization:

21-Point Grayscale, 20-Steps per Primary Color (RGB), 20-Steps per Primary & Secondary Color (RGBCMY)

10-Point Cube (1,000 points), 10-Point Cube Hybrid 1D+3D (1,021 points)

17-Point Cube (4,913 points), 17-Point Cube Hybrid 1D+3D (4,934 points)

21-Point Cube (9,261 points)

Verification:

Color Checker Classic (24 Colors), Color Checker SG (96 Colors), Color Checker SG Fleshtones (19 Colors)

6-Point Near Black Patterns, Saturation Sweeps, Luminance Sweeps, Contrast Ratio Patterns

# 4. LG OLED Display Controller

## 4.1 Dual-Mode Architecture

pub enum LgOledMode {
    Direct { ip: String, port: u16 },       // WebSocket 3000/3001
    DeviceControl { port: u16 },            // HTTP localhost:81
}

## 4.2 Direct Mode — SSAP over WebSocket

Discovery: SSDP M-SEARCH to 239.255.255.250:1900

Pairing: PIN displayed on screen → send via WebSocket → store clientKey

Calibration Commands:

start_calibration(picMode=expert1) — Enter calibration mode

upload_1d_lut(data, picMode) — Upload 1D LUT

upload_3d_lut(data, picMode, colorSpace) — Upload 3D LUT

set_white_balance(rGain, gGain, bGain) — Set WB gains

end_calibration(picMode) — Exit calibration mode

## 4.3 DeviceControl Mode — HTTP localhost:81

DeviceControl handles all WebSocket/SSAP complexity. Our app sends high-level commands.

POST /api/lg/connect, /api/lg/start_calibration, /api/lg/upload_1d_lut, /api/lg/end_calibration

## 4.4 Calibration Mode Requirements

Calibration commands only work during calibration mode (between start_calibration and end_calibration)

HDR10 tone mapping is bypassed during calibration mode

Calibration data is picture-mode specific (SDR, HDR10, Dolby Vision are independent)

To upload HDR10 LUTs, TV must be receiving HDR10 signal

To upload Dolby Vision configs, TV must be playing DV content

Alpha 9 Gen 4+ chips use 33-point 3D LUTs; Alpha 7 uses 17-point

# 5. HAL Trait Implementations

Both controllers implement existing HAL traits so they drop into GreyscaleAutoCalFlow without changes.

PGeneratorController implements PatternGenerator trait

LgOledController implements DisplayController trait

Extension trait PatternGeneratorExt adds display_pattern() and list_patterns()

# 6. Error Handling

PGeneratorError: ConnectionFailed, XmlParseError, HttpError, PatternNotFound, DeviceControlUnavailable, InvalidResponse

LgOledError: DiscoveryFailed, PairingRejected, WebSocketError, CalibrationModeError, UploadFailed, DeviceControlUnavailable, InvalidResponse

# 7. Testing Strategy

Unit Tests:

Mock TCP server (port 85) responding to XML commands

Mock WebSocket server (port 3000) responding to SSAP JSON

Mock HTTP server (port 81) simulating DeviceControl API

Integration Tests:

test_pgenerator_direct_patch_display, test_pgenerator_devicecontrol_pattern

test_lg_discovery, test_lg_pairing_flow, test_lg_calibration_upload

test_dual_mode_switching

Hardware Tests (manual):

test_real_pgenerator_direct, test_real_lg_pairing, test_real_lg_calibration

# 8. UI Integration (Future Phase)

Settings panel exposes mode selector per device:

PGenerator: Direct (TCP) vs DeviceControl (HTTP)

LG OLED: Direct (WebSocket) vs DeviceControl (HTTP)

Both modes equally accessible. Default: Direct mode.

# 9. File Structure

crates/hal-patterns/
├── Cargo.toml
└── src/
    ├── lib.rs
    ├── pgenerator.rs          # PGeneratorController
    ├── xml_protocol.rs        # XML serialization for direct mode
    ├── devicecontrol_client.rs # HTTP client for DeviceControl mode
    └── patterns_catalog.rs    # Ted's disk pattern names

crates/hal-displays/
├── Cargo.toml
└── src/
    ├── lib.rs
    ├── lg_oled.rs             # LgOledController
    ├── ssap_protocol.rs       # SSAP JSON/WebSocket for direct mode
    ├── discovery.rs           # SSDP discovery
    ├── pairing.rs             # PIN-based pairing flow
    ├── calibration_commands.rs # start/end/upload LUT
    ├── itpg.rs                # Internal pattern generator control
    └── devicecontrol_client.rs # HTTP client for DeviceControl mode