LG OLED Calibration App

Complete Implementation Plan

A Calman-equivalent display calibration application for LG OLED TVs
2021–2026 model years · macOS · Python + PyQt6

| Sub-project | Scope | Status |
| --- | --- | --- |
| 1 — TV Control Layer | TV discovery, SSAP connection, Expert menu read/write, Keychain pairing, model detection 2021–2026 | Plan written Not yet implemented |
| 2 — LUT Upload Pipeline | 1D/3D LUT parsing & upload, Dolby Vision config upload, LUT management UI | Plan written Not yet implemented |
| 3 — Measurement Workflow | ArgyllCMS meter integration, iTPG + PGenerator, patch sequences, CGATS storage, LUT generation | Plan written Not yet implemented |

Generated: April 23, 2026

# Table of Contents

1  Project Overview

1.1  Goals and Hardware Targets

1.2  LG OLED Model / Chip / webOS Reference (2021–2026)

1.3  Full System Architecture

1.4  Calibration Workflow (End-to-End)

1.5  Technology Stack

2  Sub-project 1: TV Control Layer

2.1  Architecture and Components

2.2  Task 1: Project Scaffold

2.3  Task 2: TVSettingsSnapshot

2.4  Task 3: Discovery Service

2.5  Task 4: Keychain Utility

2.6  Task 5: Connection Manager

2.7  Task 6: LGTVSettings Extension

2.8  Task 7: PyQt6 App Shell + Sidebar

2.9  Task 8: Discovery Panel UI

2.10  Task 9: Settings Panel — All 5 Tabs

2.11  Task 10: Wire UI to Backend

3  Sub-project 2: LUT Upload Pipeline

3.1  bscpylgtv Upload API Reference

3.2  Task 1: LUT Data Models + .cube Parser

3.3  Task 2: LUT Uploader

3.4  Task 3: Dolby Vision Config Upload

3.5  Task 4: LUT Panel UI

3.6  Task 5: Hardware LUT Upload Tests

3.7  Task 6: Wire LUTPanel into MainWindow

4  Sub-project 3: Measurement Workflow

4.1  ArgyllCMS Primer

4.2  PGenerator 1.6 HTTP API

4.3  iTPG Internal Pattern Generator

4.4  Task 1: Measurement Data Models

4.5  Task 2: ArgyllCMS Subprocess Wrapper

4.6  Task 3: Pattern Generator Abstraction + iTPG

4.7  Task 4: PGenerator 1.6 HTTP Client

4.8  Task 5: Patch Sequences

4.9  Task 6: Measurement Session

4.10  Task 7: CGATS + JSON Storage

4.11  Task 8: LUT Generation from Measurements

4.12  Task 9: Measurement Panel UI

4.13  Task 10: Wire Measurement Workflow into MainWindow

4.14  Task 11: Hardware Integration Tests

Section 1

# Project Overview

Date: 2026-04-23

Platform: macOS desktop app — Python + PyQt6

Goal: A Calman-equivalent display calibration application for LG OLED TVs (2021–2026 model years), covering device control, LUT upload, colorimetric measurement, and LUT generation.

## What This App Does

The app connects to LG OLED TVs wirelessly and calibrates them end-to-end:

1. Connects to the TV over WebSocket/SSAP, reads and writes all Expert/Advanced picture settings, and uploads 1D/3D LUTs and Dolby Vision configs directly to the display pipeline

2. Drives test patterns via the TV's internal pattern generator (iTPG) or an external Raspberry Pi running PGenerator 1.6

3. Reads the display with an X-Rite i1 Display Pro or i1 Pro 2 via ArgyllCMS

4. Generates correction LUTs from the measurements and uploads them back to the TV

The result is a fully calibrated display with correct tone curve, accurate white point, and accurate color gamut — all without a commercial software license.

## Sub-project Map

Sub-project 1 — TV Control Layer          [PLAN WRITTEN, NOT YET IMPLEMENTED]

TV discovery (SSDP), WebSocket/SSAP connection, Expert menu read/write,

macOS Keychain pairing, model detection for 2021–2026 OLEDs

Sub-project 2 — LUT Upload Pipeline       [PLAN WRITTEN, NOT YET IMPLEMENTED]

.cube / .cal file parsing, 1D + 3D LUT upload via bscpylgtv,

Dolby Vision config (.cfg/.txt) upload, LUT management UI

Sub-project 3 — Measurement Workflow      [PLAN WRITTEN, NOT YET IMPLEMENTED]

ArgyllCMS meter integration (i1 Display Pro, i1 Pro 2),

iTPG + PGenerator 1.6 pattern generator control,

patch sequence automation, CGATS/JSON storage,

1D tone curve + 3D LUT generation from measurements

Dependencies flow in one direction: SP2 and SP3 both depend on SP1 being complete. SP3 optionally calls SP2 to upload generated LUTs directly.

## Hardware Targets

| Hardware | Role | Integration |
| --- | --- | --- |
| LG OLED C1 (2021) | Primary calibration target | bscpylgtv via SSAP |
| LG OLED C2 (2022) | Secondary calibration target | bscpylgtv via SSAP |
| LG OLED C3–C6 (2023–2026) | Extended targets (verified on connect) | bscpylgtv via SSAP |
| LG OLED G1–G5, B1–B5, Z1–Z5 | Gallery/Signature series | Same SSAP stack |
| X-Rite i1 Display Pro Rev. B | Colorimeter (fast, 2000 nit EDR) | ArgyllCMS |
| X-Rite i1 Pro 2 | Spectrophotometer (reference accuracy) | ArgyllCMS |
| Raspberry Pi 4 + PGenerator 1.6 | External pattern generator | HTTP API |
| LG OLED iTPG | Internal pattern generator | bscpylgtv SSAP |

## LG OLED Model / Chip / webOS Reference (2021–2026)

This table drives model detection in Sub-project 1 and conditions chip-specific API routing in Sub-projects 2 and 3.

| Year | Model Series | Chip | webOS | Calibration Status |
| --- | --- | --- | --- | --- |
| 2021 | C1, G1, Z1, B1 | Alpha 9 Gen 4 | 6.0 | Fully supported |
| 2022 | C2, G2, Z2, B2 | Alpha 9 Gen 5 | 22 | Fully supported |
| 2023 | C3, G3, Z3, B3 | Alpha 9 Gen 6 | 23 | Supported — verify firmware |
| 2024 | C4, G4, Z4, B4 | Alpha 9 Gen 7 | 24 | Supported — verify firmware |
| 2025 | C5, G5, Z5, B5 | Alpha 9 Gen 8 | 25 | Supported — verify firmware |
| 2026 | C6, G6, Z6, B6 | Alpha 9 Gen 9 (est.) | 26 (est.) | Unknown — test on connect |

webOS version note: LG changed version numbering from 6.x to calendar-year format (22, 23, ...) starting in 2022. The "webOS 7.3" breakage mentioned in SP1 refers to an interim firmware on 2021 C1 models with major_ver=7 in get_software_info — not the 2022+ calendar versioning system. The firmware check must use the raw major_ver/minor_ver strings from get_software_info, not assumed LG webOS branding versions.

## Full System Architecture

┌──────────────────────────────────────────────────────────────────┐

│                        PyQt6 App Shell                           │

│                                                                  │

│  Sidebar Nav           Content Area (QStackedWidget)            │

│  ─────────────         ────────────────────────────             │

│  ● C1 Connected  →     [Discovery] [Settings] [Calibrate]       │

│  ○ C2 Offline          [LUT Files] [Prefs]                      │

└───────────────┬───────────────┬──────────────────┬──────────────┘

│               │                  │

┌──────────▼──────┐  ┌─────▼──────┐  ┌───────▼──────────┐

│  Sub-project 1  │  │  SP2       │  │  Sub-project 3   │

│  TV Control     │  │  LUT Upload│  │  Measurement     │

│  ─────────────  │  │  ─────────-│  │  ────────────    │

│  ConnectionMgr  │  │  LUTParser │  │  MeterDevice     │

│  LGTVSettings   │  │  Uploader  │  │  ArgyllCMS       │

│  Discovery      │  │  DV Config │  │  iTPG/PGenerator │

│  TVSnapshot     │  │  LUT Panel │  │  PatchSequence   │

└────────┬────────┘  └─────┬──────┘  │  MeasurSession   │

│                 │         │  LUTGenerator    │

└────────┬────────┘         └───────┬──────────┘

│                          │

┌────────▼──────────────────────────▼──────────┐

│           bscpylgtv / WebOsClient             │

│           SSAP over WebSocket                  │

└───────────────────┬───────────────────────────┘

│

┌───────────────────▼───────────────────────────┐

│          LG OLED TV (2021–2026)               │

│          WebSocket ports 3000 / 3001           │

└───────────────────────────────────────────────┘

┌───────────────────────────────────────────────┐

│  Meter Layer (Sub-project 3)                  │

│  ArgyllCMS (spotread / dispread subprocess)   │

│  → i1 Display Pro Rev. B  (USB)               │

│  → i1 Pro 2               (USB)               │

└───────────────────────────────────────────────┘

┌───────────────────────────────────────────────┐

│  External Pattern Generator (Sub-project 3)   │

│  PGenerator 1.6 HTTP API on Raspberry Pi 4    │

└───────────────────────────────────────────────┘

## Calibration Workflow (End-to-End)

1. Connect TV (SP1)

↓

2. Select picture mode + color space + HDR format

↓

3. Select pattern generator (iTPG or PGenerator on Pi)

↓

4. Select meter (i1 Display Pro or i1 Pro 2)

↓

5. Run pre-calibration measurement (SP3)

→ Display 21-point grayscale + primaries + secondaries

→ Record XYZ readings per patch

↓

6. Generate correction LUTs (SP3)

→ 1D tone curve from grayscale

→ 3D LUT from full patch set

↓

7. Upload LUTs to TV (SP2)

→ 1D LUT via bscpylgtv

→ 3D LUT via bscpylgtv

→ Optional: DV config for Dolby Vision mode

↓

8. Run post-calibration verification measurement (SP3)

→ Compare against target (BT.709, DCI-P3, BT.2020)

→ Report ΔE2000 per patch

## Technology Stack

| Layer | Technology |
| --- | --- |
| UI framework | PyQt6 |
| Async bridge | qasync |
| TV communication | bscpylgtv |
| Meter communication | ArgyllCMS (subprocess) |
| Pattern generation | iTPG (SSAP) + PGenerator (HTTP) |
| Keychain storage | keyring (macOS Keychain) |
| Numeric processing | numpy |
| Color science | colour-science (for ΔE, gamut math) |
| Testing | pytest, pytest-asyncio, pytest-qt |

## Plan Files

| Sub-project | Spec | Plan |
| --- | --- | --- |
| 1 — TV Control Layer | `docs/superpowers/specs/2026-04-16-subproject1-tv-control-layer-design.md` | `docs/superpowers/plans/2026-04-16-subproject1-tv-control-layer.md` |
| 2 — LUT Upload Pipeline | *(see this overview)* | `docs/superpowers/plans/2026-04-23-subproject2-lut-upload-pipeline.md` |
| 3 — Measurement Workflow | *(see this overview)* | `docs/superpowers/plans/2026-04-23-subproject3-measurement-workflow.md` |

Section 2

# Sub-project 1: TV Control Layer

> For agentic workers: REQUIRED SUB-SKILL: Use superpowers:subagent-driven-development (recommended) or superpowers:executing-plans to implement this plan task-by-task. Steps use checkbox (- [ ]) syntax for tracking.

Goal: Build a macOS PyQt6 desktop app that discovers, connects to, and controls the full Expert/Advanced picture menu on LG OLED C1 and C2 TVs over WebSocket/SSAP.

Architecture: bscpylgtv handles SSAP transport and calibration commands; a custom LGTVSettings extension covers picture settings bscpylgtv doesn't expose (white balance, CMS, dynamic settings); TVSettingsSnapshot is an in-memory mirror of TV state populated on connect. The UI is a sidebar + 5-tab settings panel backed by PyQt6 with qasync bridging the asyncio event loop.

Tech Stack: Python 3.11+, PyQt6, bscpylgtv, qasync, keyring, websockets, pytest, pytest-asyncio

## File Map

| File | Responsibility |
| --- | --- |
| `app/main.py` | Entry point — starts qasync event loop, creates MainWindow |
| `app/tv/state.py` | `TVSettingsSnapshot` dataclass — in-memory mirror of TV settings |
| `app/tv/discovery.py` | SSDP M-SEARCH — finds LG TVs on local network |
| `app/utils/keychain.py` | macOS Keychain wrapper via `keyring` — stores/loads client keys by TV IP |
| `app/tv/connection.py` | `ConnectionManager` — wraps `bscpylgtv.WebOsClient`, pairing, model detection, reconnect |
| `app/tv/settings.py` | `LGTVSettings` — raw SSAP payloads for WB, CMS, dynamic settings |
| `app/ui/main_window.py` | `MainWindow` — sidebar with TV status badges, navigation |
| `app/ui/discovery_panel.py` | `DiscoveryPanel` — scan UI, TV list, connect/pair flow |
| `app/ui/settings_panel.py` | `SettingsPanel` — 5-tab settings UI, binds to `TVSettingsSnapshot` |
| `tests/conftest.py` | Shared fixtures: mock WebOsClient, sample snapshots |
| `tests/unit/test_state.py` | TVSettingsSnapshot unit tests |
| `tests/unit/test_discovery.py` | Discovery Service unit tests (mocked socket) |
| `tests/unit/test_keychain.py` | Keychain utility unit tests (mocked keyring) |
| `tests/unit/test_connection.py` | ConnectionManager unit tests (mocked bscpylgtv) |
| `tests/unit/test_settings.py` | LGTVSettings unit tests (mocked WebOsClient) |
| `tests/unit/test_settings_panel.py` | Settings panel binding tests (mock snapshot, no TV) |
| `tests/hardware/test_hardware.py` | `@pytest.mark.hardware` — real TV required |

## Task 1: Project Scaffold

Files:

Create: requirements.txt

Create: pytest.ini

Create: app/__init__.py, app/tv/__init__.py, app/ui/__init__.py, app/utils/__init__.py

Create: tests/__init__.py, tests/unit/__init__.py, tests/hardware/__init__.py

Modify: CLAUDE.md

☐  Step 1: Create requirements.txt

bscpylgtv>=0.9.0

PyQt6>=6.6.0

qasync>=0.27.0

keyring>=24.0.0

websockets>=12.0

pytest>=8.0.0

pytest-asyncio>=0.23.0

pytest-qt>=4.4.0

☐  Step 2: Create pytest.ini

[pytest]

asyncio_mode = auto

markers =

hardware: marks tests as requiring a real LG TV (deselect with -m "not hardware")

☐  Step 3: Create all __init__.py files

mkdir -p app/tv app/ui app/utils tests/unit tests/hardware

touch app/__init__.py app/tv/__init__.py app/ui/__init__.py app/utils/__init__.py

touch tests/__init__.py tests/unit/__init__.py tests/hardware/__init__.py

☐  Step 4: Create virtual environment and install dependencies

python3 -m venv .venv

source .venv/bin/activate

pip install -r requirements.txt

Expected: all packages install without error.

☐  Step 5: Update CLAUDE.md

Replace contents with:

# CLAUDE.md

This file provides guidance to Claude Code (claude.ai/code) when working in this repository.

## Commands

```bash

# Install dependencies

python3 -m venv .venv && source .venv/bin/activate && pip install -r requirements.txt

# Run all unit tests (no TV required)

pytest tests/unit/ -v

# Run hardware tests (requires LG TV on local network)

pytest tests/hardware/ -v -m hardware

# Run a single test

pytest tests/unit/test_discovery.py::test_discover_returns_found_tvs -v

# Launch the app

python -m app.main

## Architecture

Python + PyQt6 macOS desktop app for LG OLED C1/C2 calibration.

app/tv/ — TV communication layer: discovery (SSDP), connection (bscpylgtv wrapper), settings extension (raw SSAP), state cache

app/ui/ — PyQt6 UI: main window with sidebar, discovery panel, 5-tab settings panel

app/utils/ — macOS Keychain wrapper for client key storage

bscpylgtv handles SSAP transport and calibration commands; LGTVSettings (app/tv/settings.py) adds picture settings bscpylgtv doesn't cover

qasync bridges asyncio and the PyQt6 event loop

## Sub-projects

Sub-project 1 (this): TV discovery, connection, full expert picture menu read/write

Sub-project 2: LUT upload pipeline (1D/3D, SDR/HDR10/DV)

Sub-project 3: Measurement workflow + LUT generation (X-Rite meters, LightSpace Pi)

- [ ] **Step 6: Commit**

```bash

git init

git add requirements.txt pytest.ini CLAUDE.md app/ tests/

git commit -m "feat: project scaffold — PyQt6 + bscpylgtv structure"

## Task 2: TVSettingsSnapshot

Files:

Create: app/tv/state.py

Create: tests/unit/test_state.py

☐  Step 1: Write failing tests

# tests/unit/test_state.py

from app.tv.state import TVSettingsSnapshot, ChipGeneration

def test_snapshot_has_defaults():

snap = TVSettingsSnapshot()

assert snap.oled_light == 50

assert snap.contrast == 85

assert snap.brightness == 50

assert snap.pic_mode == "expert1"

assert snap.chip_generation == ChipGeneration.UNKNOWN

def test_snapshot_update():

snap = TVSettingsSnapshot()

snap.oled_light = 70

assert snap.oled_light == 70

def test_snapshot_wb_20pt_has_20_entries():

snap = TVSettingsSnapshot()

assert len(snap.wb_20pt_red) == 20

assert len(snap.wb_20pt_green) == 20

assert len(snap.wb_20pt_blue) == 20

def test_snapshot_cms_colors():

snap = TVSettingsSnapshot()

for color in ("red", "green", "blue", "cyan", "magenta", "yellow"):

assert hasattr(snap, f"cms_{color}_hue")

assert hasattr(snap, f"cms_{color}_saturation")

assert hasattr(snap, f"cms_{color}_luminance")

☐  Step 2: Run tests to verify they fail

pytest tests/unit/test_state.py -v

Expected: ImportError or ModuleNotFoundError

☐  Step 3: Implement TVSettingsSnapshot

# app/tv/state.py

from dataclasses import dataclass, field

from enum import Enum

from typing import List

class ChipGeneration(Enum):

UNKNOWN = "unknown"

ALPHA9_GEN4 = "alpha9_gen4"   # C1 (2021)

ALPHA9_GEN5 = "alpha9_gen5"   # C2 (2022)

ALPHA9_GEN6 = "alpha9_gen6"   # C3 (2023)

ALPHA9_GEN7 = "alpha9_gen7"   # C4 (2024)

ALPHA9_GEN8 = "alpha9_gen8"   # C5 (2025)

ALPHA9_GEN9 = "alpha9_gen9"   # C6 (2026, estimated)

@dataclass

class TVSettingsSnapshot:

# Identity

chip_generation: ChipGeneration = ChipGeneration.UNKNOWN

webos_version: str = ""

pic_mode: str = "expert1"

# Tab 1 — Picture

oled_light: int = 50

contrast: int = 85

brightness: int = 50

sharpness: int = 10

color: int = 50

tint: int = 0

color_temperature: str = "warm2"  # warm2/warm1/natural/cool/manual

# Tab 3 — Gamma / Color Space

gamma: str = "bt1886"   # 1.8/2.0/2.2/2.4/bt1886/srgb

color_space: str = "auto"  # auto/native/bt709/bt2020/dcip3

black_level: str = "low"   # low/high/auto

trumotion: str = "off"

# Tab 2 — White Balance 2-point

wb_2pt_red_gain: int = 0

wb_2pt_green_gain: int = 0

wb_2pt_blue_gain: int = 0

wb_2pt_red_offset: int = 0

wb_2pt_green_offset: int = 0

wb_2pt_blue_offset: int = 0

# Tab 2 — White Balance 20-point (one value per IRE step 5%–100%)

wb_20pt_red: List[int] = field(default_factory=lambda: [0] * 20)

wb_20pt_green: List[int] = field(default_factory=lambda: [0] * 20)

wb_20pt_blue: List[int] = field(default_factory=lambda: [0] * 20)

# Tab 4 — CMS per-color (Hue/Saturation/Luminance)

cms_red_hue: int = 0

cms_red_saturation: int = 0

cms_red_luminance: int = 0

cms_green_hue: int = 0

cms_green_saturation: int = 0

cms_green_luminance: int = 0

cms_blue_hue: int = 0

cms_blue_saturation: int = 0

cms_blue_luminance: int = 0

cms_cyan_hue: int = 0

cms_cyan_saturation: int = 0

cms_cyan_luminance: int = 0

cms_magenta_hue: int = 0

cms_magenta_saturation: int = 0

cms_magenta_luminance: int = 0

cms_yellow_hue: int = 0

cms_yellow_saturation: int = 0

cms_yellow_luminance: int = 0

# Tab 5 — HDR / Dynamic

dynamic_contrast: str = "off"   # off/low/medium/high

dynamic_color: str = "off"      # off/low/high

asbl: bool = False

hdr_tone_mapping: bool = True

peak_luminance: int = 1000      # nits, HDR10 target

dv_picture_mode: str = "dark"   # bright/dark/vivid

local_dimming: str = "high"     # off/low/medium/high

energy_saving: str = "off"      # off/min/med/max/auto/screen_off

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_state.py -v

Expected: 4 tests pass.

☐  Step 5: Commit

git add app/tv/state.py tests/unit/test_state.py

git commit -m "feat: TVSettingsSnapshot dataclass with all expert settings fields"

## Task 3: Discovery Service

Files:

Create: app/tv/discovery.py

Create: tests/unit/test_discovery.py

☐  Step 1: Write failing tests

# tests/unit/test_discovery.py

import asyncio

from unittest.mock import AsyncMock, patch, MagicMock

from app.tv.discovery import DiscoveredTV, discover_tvs

def test_discovered_tv_has_ip_and_name():

tv = DiscoveredTV(ip="192.168.1.101", name="[LG] webOS TV OLED65C1")

assert tv.ip == "192.168.1.101"

assert tv.name == "[LG] webOS TV OLED65C1"

SSDP_RESPONSE = (

"HTTP/1.1 200 OK\r\n"

"LOCATION: http://192.168.1.101:3000/\r\n"

"FRIENDLY-NAME: [LG] webOS TV OLED65C1\r\n"

"USN: uuid:abc123\r\n\r\n"

).encode()

async def test_discover_returns_found_tvs():

mock_sock = MagicMock()

mock_sock.recvfrom = MagicMock(side_effect=[

(SSDP_RESPONSE, ("192.168.1.101", 1900)),

TimeoutError(),

])

with patch("app.tv.discovery.socket.socket", return_value=mock_sock):

tvs = await discover_tvs(timeout=0.1)

assert len(tvs) == 1

assert tvs[0].ip == "192.168.1.101"

assert "C1" in tvs[0].name

async def test_discover_returns_empty_on_timeout():

mock_sock = MagicMock()

mock_sock.recvfrom = MagicMock(side_effect=TimeoutError())

with patch("app.tv.discovery.socket.socket", return_value=mock_sock):

tvs = await discover_tvs(timeout=0.1)

assert tvs == []

☐  Step 2: Run tests to verify they fail

pytest tests/unit/test_discovery.py -v

Expected: ImportError

☐  Step 3: Implement Discovery Service

# app/tv/discovery.py

import asyncio

import socket

import re

from dataclasses import dataclass

from typing import List

SSDP_ADDR = "239.255.255.250"

SSDP_PORT = 1900

SSDP_MX = 3

SSDP_ST = "urn:lge-com:service:webos-second-screen:1"

SSDP_REQUEST = (

"M-SEARCH * HTTP/1.1\r\n"

f"HOST: {SSDP_ADDR}:{SSDP_PORT}\r\n"

"MAN: \"ssdp:discover\"\r\n"

f"MX: {SSDP_MX}\r\n"

f"ST: {SSDP_ST}\r\n"

"\r\n"

).encode()

@dataclass

class DiscoveredTV:

ip: str

name: str

def _parse_ssdp_response(data: bytes) -> dict:

text = data.decode(errors="ignore")

result = {}

for line in text.splitlines():

if ":" in line:

key, _, value = line.partition(":")

result[key.strip().upper()] = value.strip()

ip_match = re.search(r"http://(\d+\.\d+\.\d+\.\d+)", result.get("LOCATION", ""))

result["_IP"] = ip_match.group(1) if ip_match else ""

return result

async def discover_tvs(timeout: float = 5.0) -> List[DiscoveredTV]:

found: dict[str, DiscoveredTV] = {}

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM, socket.IPPROTO_UDP)

sock.setsockopt(socket.IPPROTO_IP, socket.IP_MULTICAST_TTL, 2)

sock.settimeout(timeout)

try:

sock.sendto(SSDP_REQUEST, (SSDP_ADDR, SSDP_PORT))

while True:

try:

data, _ = sock.recvfrom(4096)

parsed = _parse_ssdp_response(data)

ip = parsed.get("_IP", "")

name = parsed.get("FRIENDLY-NAME", ip)

if ip and ip not in found:

found[ip] = DiscoveredTV(ip=ip, name=name)

except TimeoutError:

break

finally:

sock.close()

return list(found.values())

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_discovery.py -v

Expected: 3 tests pass.

☐  Step 5: Commit

git add app/tv/discovery.py tests/unit/test_discovery.py

git commit -m "feat: SSDP discovery service for LG webOS TVs"

## Task 4: Keychain Utility

Files:

Create: app/utils/keychain.py

Create: tests/unit/test_keychain.py

☐  Step 1: Write failing tests

# tests/unit/test_keychain.py

from unittest.mock import patch

from app.utils.keychain import save_client_key, load_client_key, delete_client_key

SERVICE = "lg-oled-cal"

def test_save_and_load_key():

with patch("app.utils.keychain.keyring") as mock_kr:

mock_kr.get_password.return_value = "abc123"

save_client_key("192.168.1.101", "abc123")

mock_kr.set_password.assert_called_once_with(SERVICE, "192.168.1.101", "abc123")

key = load_client_key("192.168.1.101")

mock_kr.get_password.assert_called_once_with(SERVICE, "192.168.1.101")

assert key == "abc123"

def test_load_missing_key_returns_none():

with patch("app.utils.keychain.keyring") as mock_kr:

mock_kr.get_password.return_value = None

assert load_client_key("192.168.1.102") is None

def test_delete_key():

with patch("app.utils.keychain.keyring") as mock_kr:

delete_client_key("192.168.1.101")

mock_kr.delete_password.assert_called_once_with(SERVICE, "192.168.1.101")

☐  Step 2: Run tests to verify they fail

pytest tests/unit/test_keychain.py -v

Expected: ImportError

☐  Step 3: Implement Keychain utility

# app/utils/keychain.py

import keyring

from typing import Optional

_SERVICE = "lg-oled-cal"

def save_client_key(tv_ip: str, client_key: str) -> None:

keyring.set_password(_SERVICE, tv_ip, client_key)

def load_client_key(tv_ip: str) -> Optional[str]:

return keyring.get_password(_SERVICE, tv_ip)

def delete_client_key(tv_ip: str) -> None:

keyring.delete_password(_SERVICE, tv_ip)

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_keychain.py -v

Expected: 3 tests pass.

☐  Step 5: Commit

git add app/utils/keychain.py tests/unit/test_keychain.py

git commit -m "feat: macOS Keychain wrapper for TV client key storage"

## Task 5: Connection Manager

Files:

Create: app/tv/connection.py

Create: tests/unit/test_connection.py

Create: tests/conftest.py

☐  Step 1: Write shared fixtures

# tests/conftest.py

import pytest

from unittest.mock import AsyncMock, MagicMock

from app.tv.state import TVSettingsSnapshot, ChipGeneration

@pytest.fixture

def mock_webos_client():

client = AsyncMock()

client.connect = AsyncMock()

client.disconnect = AsyncMock()

client.get_software_info = AsyncMock(return_value={

"model_name": "OLED65C1PUB",

"major_ver": "6",

"minor_ver": "0",

})

client.client_key = "test-key-abc"

return client

@pytest.fixture

def c2_mock_webos_client(mock_webos_client):

mock_webos_client.get_software_info = AsyncMock(return_value={

"model_name": "OLED65C2PUA",

"major_ver": "7",

"minor_ver": "0",

})

return mock_webos_client

@pytest.fixture

def sample_snapshot():

return TVSettingsSnapshot(

oled_light=70,

contrast=85,

chip_generation=ChipGeneration.ALPHA9_GEN4,

)

☐  Step 2: Write failing tests

# tests/unit/test_connection.py

import pytest

from unittest.mock import AsyncMock, patch, MagicMock

from app.tv.connection import ConnectionManager

from app.tv.state import ChipGeneration

async def test_connect_stores_client_key(mock_webos_client):

with patch("app.tv.connection.WebOsClient", return_value=mock_webos_client), \

patch("app.tv.connection.save_client_key") as mock_save:

mgr = ConnectionManager("192.168.1.101")

await mgr.connect()

mock_save.assert_called_once_with("192.168.1.101", "test-key-abc")

async def test_connect_detects_c1_chip(mock_webos_client):

with patch("app.tv.connection.WebOsClient", return_value=mock_webos_client):

mgr = ConnectionManager("192.168.1.101")

await mgr.connect()

assert mgr.snapshot.chip_generation == ChipGeneration.ALPHA9_GEN4

async def test_connect_detects_c2_chip(c2_mock_webos_client):

with patch("app.tv.connection.WebOsClient", return_value=c2_mock_webos_client):

mgr = ConnectionManager("192.168.1.101")

await mgr.connect()

assert mgr.snapshot.chip_generation == ChipGeneration.ALPHA9_GEN5

async def test_connect_detects_c4_chip():

mock_client = AsyncMock()

mock_client.connect = AsyncMock()

mock_client.client_key = "key"

mock_client.get_software_info = AsyncMock(return_value={

"model_name": "OLED65C4PUA",

"major_ver": "24",

"minor_ver": "0",

})

with patch("app.tv.connection.WebOsClient", return_value=mock_client), \

patch("app.tv.connection.save_client_key"):

mgr = ConnectionManager("192.168.1.101")

await mgr.connect()

assert mgr.snapshot.chip_generation == ChipGeneration.ALPHA9_GEN7

async def test_connect_unknown_model_returns_unknown():

mock_client = AsyncMock()

mock_client.connect = AsyncMock()

mock_client.client_key = "key"

mock_client.get_software_info = AsyncMock(return_value={

"model_name": "OLEDXXXNEW99",

"major_ver": "99",

"minor_ver": "0",

})

with patch("app.tv.connection.WebOsClient", return_value=mock_client), \

patch("app.tv.connection.save_client_key"):

mgr = ConnectionManager("192.168.1.101")

await mgr.connect()

assert mgr.snapshot.chip_generation == ChipGeneration.UNKNOWN

async def test_firmware_warning_on_webos_73(mock_webos_client):

mock_webos_client.get_software_info = AsyncMock(return_value={

"model_name": "OLED65C1PUB",

"major_ver": "7",

"minor_ver": "3",

})

with patch("app.tv.connection.WebOsClient", return_value=mock_webos_client):

mgr = ConnectionManager("192.168.1.101")

await mgr.connect()

assert mgr.firmware_warning is True

async def test_uses_stored_client_key_on_reconnect(mock_webos_client):

with patch("app.tv.connection.WebOsClient", return_value=mock_webos_client), \

patch("app.tv.connection.load_client_key", return_value="stored-key"), \

patch("app.tv.connection.save_client_key"):

mgr = ConnectionManager("192.168.1.101")

await mgr.connect()

_, kwargs = mock_webos_client.__class__.call_args if hasattr(mock_webos_client.__class__, 'call_args') else (None, {})

# WebOsClient should be initialized with the stored client_key

assert mgr.client_key == "stored-key" or mgr.client_key == "test-key-abc"

☐  Step 3: Run tests to verify they fail

pytest tests/unit/test_connection.py -v

Expected: ImportError

☐  Step 4: Implement ConnectionManager

# app/tv/connection.py

import asyncio

import logging

from typing import Optional, Callable

from bscpylgtv import WebOsClient

from app.tv.state import TVSettingsSnapshot, ChipGeneration

from app.utils.keychain import save_client_key, load_client_key, delete_client_key

logger = logging.getLogger(__name__)

_WEBOS_73_MAJOR = 7

_WEBOS_73_MINOR = 3

_CHIP_MAP = {

"C1": ChipGeneration.ALPHA9_GEN4,   # 2021

"G1": ChipGeneration.ALPHA9_GEN4,

"Z1": ChipGeneration.ALPHA9_GEN4,

"B1": ChipGeneration.ALPHA9_GEN4,

"C2": ChipGeneration.ALPHA9_GEN5,   # 2022

"G2": ChipGeneration.ALPHA9_GEN5,

"Z2": ChipGeneration.ALPHA9_GEN5,

"B2": ChipGeneration.ALPHA9_GEN5,

"C3": ChipGeneration.ALPHA9_GEN6,   # 2023

"G3": ChipGeneration.ALPHA9_GEN6,

"Z3": ChipGeneration.ALPHA9_GEN6,

"B3": ChipGeneration.ALPHA9_GEN6,

"C4": ChipGeneration.ALPHA9_GEN7,   # 2024

"G4": ChipGeneration.ALPHA9_GEN7,

"Z4": ChipGeneration.ALPHA9_GEN7,

"B4": ChipGeneration.ALPHA9_GEN7,

"C5": ChipGeneration.ALPHA9_GEN8,   # 2025

"G5": ChipGeneration.ALPHA9_GEN8,

"Z5": ChipGeneration.ALPHA9_GEN8,

"B5": ChipGeneration.ALPHA9_GEN8,

"C6": ChipGeneration.ALPHA9_GEN9,   # 2026 (estimated)

"G6": ChipGeneration.ALPHA9_GEN9,

"Z6": ChipGeneration.ALPHA9_GEN9,

"B6": ChipGeneration.ALPHA9_GEN9,

}

def _detect_chip(model_name: str) -> ChipGeneration:

model_upper = model_name.upper()

for suffix, gen in _CHIP_MAP.items():

if suffix in model_upper:

return gen

return ChipGeneration.UNKNOWN

def _is_firmware_incompatible(major: str, minor: str) -> bool:

try:

return int(major) > _WEBOS_73_MAJOR or (

int(major) == _WEBOS_73_MAJOR and int(minor) >= _WEBOS_73_MINOR

)

except ValueError:

return False

class ConnectionManager:

def __init__(self, ip: str):

self.ip = ip

self.snapshot = TVSettingsSnapshot()

self.firmware_warning: bool = False

self.client_key: Optional[str] = load_client_key(ip)

self._client: Optional[WebOsClient] = None

self._on_disconnect: Optional[Callable] = None

async def connect(self) -> None:

self._client = WebOsClient(self.ip, client_key=self.client_key)

await self._client.connect()

self.client_key = self._client.client_key

save_client_key(self.ip, self.client_key)

await self._post_connect()

async def _post_connect(self) -> None:

info = await self._client.get_software_info()

model = info.get("model_name", "")

major = info.get("major_ver", "0")

minor = info.get("minor_ver", "0")

self.snapshot.chip_generation = _detect_chip(model)

self.snapshot.webos_version = f"{major}.{minor}"

self.firmware_warning = _is_firmware_incompatible(major, minor)

if self.firmware_warning:

logger.warning("webOS %s.%s detected — calibration API may be incompatible", major, minor)

async def disconnect(self) -> None:

if self._client:

await self._client.disconnect()

self._client = None

async def reconnect_with_backoff(self, max_attempts: int = 3) -> bool:

delays = [2, 4, 8]

for attempt, delay in enumerate(delays[:max_attempts], 1):

try:

await self.connect()

return True

except Exception as e:

logger.warning("Reconnect attempt %d failed: %s", attempt, e)

if attempt < max_attempts:

await asyncio.sleep(delay)

return False

def clear_stored_key(self) -> None:

delete_client_key(self.ip)

self.client_key = None

@property

def is_connected(self) -> bool:

return self._client is not None

@property

def client(self) -> Optional[WebOsClient]:

return self._client

☐  Step 5: Run tests to verify they pass

pytest tests/unit/test_connection.py -v

Expected: 5 tests pass.

☐  Step 6: Commit

git add app/tv/connection.py tests/unit/test_connection.py tests/conftest.py

git commit -m "feat: ConnectionManager with pairing, model detection, firmware check, reconnect"

## Task 6: LGTVSettings Extension

Files:

Create: app/tv/settings.py

Create: tests/unit/test_settings.py

☐  Step 1: Write failing tests

# tests/unit/test_settings.py

import pytest

from unittest.mock import AsyncMock, call

from app.tv.settings import LGTVSettings

from app.tv.state import TVSettingsSnapshot

@pytest.fixture

def mock_client():

client = AsyncMock()

client.request = AsyncMock(return_value={"returnValue": True})

return client

@pytest.fixture

def settings(mock_client):

return LGTVSettings(client=mock_client, pic_mode="expert1")

async def test_set_white_balance_2pt(settings, mock_client):

await settings.set_white_balance_2pt(

red_gain=10, green_gain=0, blue_gain=-5,

red_offset=0, green_offset=0, blue_offset=0

)

mock_client.request.assert_called_once()

uri, payload = mock_client.request.call_args[0]

assert uri == "ssap://externalpq/setExternalPqData"

assert payload["picMode"] == "expert1"

assert payload["data"]["whiteBalanceRedGain"] == 10

async def test_set_white_balance_20pt(settings, mock_client):

red = [0] * 20

green = [0] * 20

blue = [5] * 20

await settings.set_white_balance_20pt(red=red, green=green, blue=blue)

mock_client.request.assert_called_once()

uri, payload = mock_client.request.call_args[0]

assert uri == "ssap://externalpq/setExternalPqData"

assert len(payload["data"]["whiteBalance20ptBlue"]) == 20

async def test_set_cms_color(settings, mock_client):

await settings.set_cms_color("red", hue=5, saturation=-3, luminance=0)

mock_client.request.assert_called_once()

uri, payload = mock_client.request.call_args[0]

assert uri == "ssap://externalpq/setExternalPqData"

assert payload["data"]["colorManagementRedHue"] == 5

async def test_set_dynamic_contrast(settings, mock_client):

await settings.set_dynamic_contrast("medium")

mock_client.request.assert_called_once()

uri, payload = mock_client.request.call_args[0]

assert payload["data"]["dynamicContrast"] == "medium"

async def test_set_cms_invalid_color_raises(settings):

with pytest.raises(ValueError, match="Unknown color"):

await settings.set_cms_color("purple", hue=0, saturation=0, luminance=0)

☐  Step 2: Run tests to verify they fail

pytest tests/unit/test_settings.py -v

Expected: ImportError

☐  Step 3: Implement LGTVSettings

# app/tv/settings.py

from typing import List

from bscpylgtv import WebOsClient

_SET_PQ_URI = "ssap://externalpq/setExternalPqData"

_GET_PQ_URI = "ssap://externalpq/getExternalPqData"

_CMS_COLOR_KEYS = {

"red":     ("colorManagementRedHue",     "colorManagementRedSaturation",     "colorManagementRedLuminance"),

"green":   ("colorManagementGreenHue",   "colorManagementGreenSaturation",   "colorManagementGreenLuminance"),

"blue":    ("colorManagementBlueHue",    "colorManagementBlueSaturation",    "colorManagementBlueLuminance"),

"cyan":    ("colorManagementCyanHue",    "colorManagementCyanSaturation",    "colorManagementCyanLuminance"),

"magenta": ("colorManagementMagentaHue", "colorManagementMagentaSaturation", "colorManagementMagentaLuminance"),

"yellow":  ("colorManagementYellowHue",  "colorManagementYellowSaturation",  "colorManagementYellowLuminance"),

}

class LGTVSettings:

def __init__(self, client: WebOsClient, pic_mode: str = "expert1"):

self._client = client

self.pic_mode = pic_mode

async def _set(self, data: dict) -> None:

await self._client.request(_SET_PQ_URI, {"picMode": self.pic_mode, "data": data})

async def set_white_balance_2pt(

self,

red_gain: int, green_gain: int, blue_gain: int,

red_offset: int, green_offset: int, blue_offset: int,

) -> None:

await self._set({

"whiteBalanceRedGain": red_gain,

"whiteBalanceGreenGain": green_gain,

"whiteBalanceBlueGain": blue_gain,

"whiteBalanceRedOffset": red_offset,

"whiteBalanceGreenOffset": green_offset,

"whiteBalanceBlueOffset": blue_offset,

})

async def set_white_balance_20pt(

self,

red: List[int],

green: List[int],

blue: List[int],

) -> None:

if len(red) != 20 or len(green) != 20 or len(blue) != 20:

raise ValueError("20-point white balance requires exactly 20 values per channel")

await self._set({

"whiteBalance20ptRed": red,

"whiteBalance20ptGreen": green,

"whiteBalance20ptBlue": blue,

})

async def set_cms_color(

self, color: str, hue: int, saturation: int, luminance: int

) -> None:

if color not in _CMS_COLOR_KEYS:

raise ValueError(f"Unknown color '{color}'. Must be one of: {list(_CMS_COLOR_KEYS)}")

hue_key, sat_key, lum_key = _CMS_COLOR_KEYS[color]

await self._set({hue_key: hue, sat_key: saturation, lum_key: luminance})

async def set_dynamic_contrast(self, value: str) -> None:

await self._set({"dynamicContrast": value})

async def set_dynamic_color(self, value: str) -> None:

await self._set({"dynamicColor": value})

async def set_asbl(self, enabled: bool) -> None:

await self._set({"autoStaticBrightnessLimit": "on" if enabled else "off"})

async def set_local_dimming(self, value: str) -> None:

await self._set({"localDimming": value})

async def set_energy_saving(self, value: str) -> None:

await self._set({"energySaving": value})

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_settings.py -v

Expected: 5 tests pass.

☐  Step 5: Commit

git add app/tv/settings.py tests/unit/test_settings.py

git commit -m "feat: LGTVSettings extension for white balance, CMS, dynamic settings via raw SSAP"

## Task 7: PyQt6 App Shell + Sidebar

Files:

Create: app/main.py

Create: app/ui/main_window.py

Create: tests/unit/test_main_window.py

☐  Step 1: Write failing tests

# tests/unit/test_main_window.py

import pytest

from unittest.mock import MagicMock

from PyQt6.QtWidgets import QApplication

from app.ui.main_window import MainWindow

from app.tv.state import TVSettingsSnapshot, ChipGeneration

@pytest.fixture(scope="module")

def app():

return QApplication.instance() or QApplication([])

def test_main_window_creates(app):

win = MainWindow()

assert win is not None

win.close()

def test_sidebar_shows_tv_connected(app):

win = MainWindow()

snap = TVSettingsSnapshot(chip_generation=ChipGeneration.ALPHA9_GEN4)

win.update_tv_status("192.168.1.101", "C1", connected=True)

# Check the sidebar label text contains "C1"

found = False

for label in win.findChildren(__import__("PyQt6.QtWidgets", fromlist=["QLabel"]).QLabel):

if "C1" in label.text():

found = True

break

assert found

win.close()

def test_sidebar_shows_tv_offline(app):

win = MainWindow()

win.update_tv_status("192.168.1.102", "C2", connected=False)

from PyQt6.QtWidgets import QLabel

labels = [l.text() for l in win.findChildren(QLabel)]

assert any("C2" in t for t in labels)

win.close()

☐  Step 2: Run tests to verify they fail

pytest tests/unit/test_main_window.py -v

Expected: ImportError

☐  Step 3: Implement MainWindow

# app/ui/main_window.py

from PyQt6.QtWidgets import (

QMainWindow, QWidget, QHBoxLayout, QVBoxLayout,

QLabel, QPushButton, QFrame, QStackedWidget

)

from PyQt6.QtCore import Qt

from PyQt6.QtGui import QFont

class TVStatusWidget(QWidget):

def __init__(self, name: str, connected: bool, parent=None):

super().__init__(parent)

layout = QHBoxLayout(self)

layout.setContentsMargins(8, 4, 8, 4)

dot = QLabel("●" if connected else "○")

dot.setStyleSheet(f"color: {'#4fc3f7' if connected else '#666'};")

label = QLabel(name)

label.setStyleSheet(f"color: {'#fff' if connected else '#666'}; font-size: 12px;")

layout.addWidget(dot)

layout.addWidget(label)

layout.addStretch()

class MainWindow(QMainWindow):

def __init__(self, parent=None):

super().__init__(parent)

self.setWindowTitle("LG OLED Calibration")

self.setMinimumSize(900, 600)

self._tv_status_widgets: dict[str, TVStatusWidget] = {}

self._build_ui()

def _build_ui(self):

central = QWidget()

self.setCentralWidget(central)

root_layout = QHBoxLayout(central)

root_layout.setContentsMargins(0, 0, 0, 0)

root_layout.setSpacing(0)

# Sidebar

sidebar = QFrame()

sidebar.setFixedWidth(160)

sidebar.setStyleSheet("background: #1a1a2e; border-right: 1px solid #333;")

sidebar_layout = QVBoxLayout(sidebar)

sidebar_layout.setContentsMargins(0, 12, 0, 12)

sidebar_layout.setSpacing(0)

self._tv_status_area = QVBoxLayout()

sidebar_layout.addLayout(self._tv_status_area)

divider = QFrame()

divider.setFrameShape(QFrame.Shape.HLine)

divider.setStyleSheet("color: #333;")

sidebar_layout.addWidget(divider)

nav_items = [("📺  Settings", "settings"), ("🎨  Calibrate", "calibrate"),

("📁  LUT Files", "luts"), ("⚙️  Prefs", "prefs")]

for label, key in nav_items:

btn = QPushButton(label)

btn.setStyleSheet(

"QPushButton { text-align: left; padding: 8px 12px; background: transparent; "

"color: #aaa; border: none; font-size: 12px; }"

"QPushButton:hover { background: #2a2a3e; color: #fff; }"

)

sidebar_layout.addWidget(btn)

sidebar_layout.addStretch()

# Main content area

self.content_stack = QStackedWidget()

self.content_stack.setStyleSheet("background: #111;")

root_layout.addWidget(sidebar)

root_layout.addWidget(self.content_stack, 1)

def update_tv_status(self, ip: str, name: str, connected: bool):

if ip in self._tv_status_widgets:

old = self._tv_status_widgets[ip]

self._tv_status_area.removeWidget(old)

old.deleteLater()

widget = TVStatusWidget(name, connected)

self._tv_status_widgets[ip] = widget

self._tv_status_area.addWidget(widget)

def set_content(self, widget: QWidget):

self.content_stack.addWidget(widget)

self.content_stack.setCurrentWidget(widget)

☐  Step 4: Create entry point

# app/main.py

import sys

import asyncio

from PyQt6.QtWidgets import QApplication

import qasync

from app.ui.main_window import MainWindow

def main():

app = QApplication(sys.argv)

app.setStyle("Fusion")

loop = qasync.QEventLoop(app)

asyncio.set_event_loop(loop)

window = MainWindow()

window.show()

with loop:

loop.run_forever()

if __name__ == "__main__":

main()

☐  Step 5: Run tests to verify they pass

pytest tests/unit/test_main_window.py -v

Expected: 3 tests pass.

☐  Step 6: Verify the app launches

python -m app.main

Expected: Window opens with dark sidebar, closes cleanly with Cmd+Q.

☐  Step 7: Commit

git add app/main.py app/ui/main_window.py tests/unit/test_main_window.py

git commit -m "feat: PyQt6 MainWindow with sidebar, TV status badges, navigation"

## Task 8: Discovery Panel UI

Files:

Create: app/ui/discovery_panel.py

Create: tests/unit/test_discovery_panel.py

☐  Step 1: Write failing tests

# tests/unit/test_discovery_panel.py

import pytest

from unittest.mock import AsyncMock, patch, MagicMock

from PyQt6.QtWidgets import QApplication

from app.ui.discovery_panel import DiscoveryPanel

from app.tv.discovery import DiscoveredTV

@pytest.fixture(scope="module")

def app():

return QApplication.instance() or QApplication([])

def test_panel_shows_found_tvs(app):

panel = DiscoveryPanel(on_connect=AsyncMock())

tvs = [DiscoveredTV("192.168.1.101", "C1 OLED"), DiscoveredTV("192.168.1.102", "C2 OLED")]

panel.show_discovered(tvs)

from PyQt6.QtWidgets import QListWidget

list_widgets = panel.findChildren(QListWidget)

assert any(lw.count() == 2 for lw in list_widgets)

panel.close()

def test_panel_shows_no_tvs_message(app):

panel = DiscoveryPanel(on_connect=AsyncMock())

panel.show_discovered([])

from PyQt6.QtWidgets import QLabel

labels = [l.text() for l in panel.findChildren(QLabel)]

assert any("No TVs" in t or "not found" in t.lower() for t in labels)

panel.close()

☐  Step 2: Run tests to verify they fail

pytest tests/unit/test_discovery_panel.py -v

Expected: ImportError

☐  Step 3: Implement DiscoveryPanel

# app/ui/discovery_panel.py

from typing import Callable, List, Optional

from PyQt6.QtWidgets import (

QWidget, QVBoxLayout, QHBoxLayout, QPushButton,

QLabel, QListWidget, QListWidgetItem, QLineEdit, QMessageBox

)

from PyQt6.QtCore import Qt, pyqtSignal

from app.tv.discovery import DiscoveredTV

class DiscoveryPanel(QWidget):

tv_selected = pyqtSignal(str, str)  # ip, name

def __init__(self, on_connect: Callable, parent=None):

super().__init__(parent)

self._on_connect = on_connect

self._build_ui()

def _build_ui(self):

layout = QVBoxLayout(self)

layout.setContentsMargins(24, 24, 24, 24)

title = QLabel("Connect to TV")

title.setStyleSheet("color: #fff; font-size: 18px; font-weight: bold;")

layout.addWidget(title)

scan_row = QHBoxLayout()

self._scan_btn = QPushButton("Scan Network")

self._scan_btn.setStyleSheet(

"background: #4fc3f7; color: #000; padding: 8px 16px; border-radius: 4px; font-weight: bold;"

)

scan_row.addWidget(self._scan_btn)

scan_row.addStretch()

layout.addLayout(scan_row)

self._status_label = QLabel("")

self._status_label.setStyleSheet("color: #aaa; font-size: 12px;")

layout.addWidget(self._status_label)

self._tv_list = QListWidget()

self._tv_list.setStyleSheet(

"background: #1a1a2e; color: #ccc; border: 1px solid #333; border-radius: 4px;"

)

layout.addWidget(self._tv_list)

# Manual IP entry

manual_row = QHBoxLayout()

manual_label = QLabel("Manual IP:")

manual_label.setStyleSheet("color: #aaa; font-size: 12px;")

self._ip_input = QLineEdit()

self._ip_input.setPlaceholderText("192.168.1.xxx")

self._ip_input.setStyleSheet("background: #1a1a2e; color: #fff; border: 1px solid #333; padding: 4px; border-radius: 4px;")

connect_btn = QPushButton("Connect")

connect_btn.setStyleSheet("background: #2a2a3e; color: #aaa; padding: 4px 12px; border-radius: 4px;")

connect_btn.clicked.connect(self._on_manual_connect)

manual_row.addWidget(manual_label)

manual_row.addWidget(self._ip_input, 1)

manual_row.addWidget(connect_btn)

layout.addLayout(manual_row)

self._tv_list.itemDoubleClicked.connect(self._on_item_double_clicked)

self._scan_btn.clicked.connect(self._on_scan_clicked)

def show_discovered(self, tvs: List[DiscoveredTV]):

self._tv_list.clear()

if not tvs:

self._status_label.setText("No TVs found — ensure TV is on the same Wi-Fi network")

return

self._status_label.setText(f"{len(tvs)} TV(s) found")

for tv in tvs:

item = QListWidgetItem(f"{tv.name}  ({tv.ip})")

item.setData(Qt.ItemDataRole.UserRole, tv)

self._tv_list.addItem(item)

def set_scanning(self, scanning: bool):

self._scan_btn.setEnabled(not scanning)

self._scan_btn.setText("Scanning…" if scanning else "Scan Network")

def _on_scan_clicked(self):

self.tv_selected.emit("__scan__", "")

def _on_item_double_clicked(self, item: QListWidgetItem):

tv: DiscoveredTV = item.data(Qt.ItemDataRole.UserRole)

self.tv_selected.emit(tv.ip, tv.name)

def _on_manual_connect(self):

ip = self._ip_input.text().strip()

if ip:

self.tv_selected.emit(ip, ip)

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_discovery_panel.py -v

Expected: 2 tests pass.

☐  Step 5: Commit

git add app/ui/discovery_panel.py tests/unit/test_discovery_panel.py

git commit -m "feat: DiscoveryPanel UI with SSDP scan results, TV list, manual IP entry"

## Task 9: Settings Panel — All 5 Tabs

Files:

Create: app/ui/settings_panel.py

Create: tests/unit/test_settings_panel.py

☐  Step 1: Write failing tests

# tests/unit/test_settings_panel.py

import pytest

from PyQt6.QtWidgets import QApplication, QTabWidget, QSlider, QComboBox

from app.ui.settings_panel import SettingsPanel

from app.tv.state import TVSettingsSnapshot, ChipGeneration

@pytest.fixture(scope="module")

def app():

return QApplication.instance() or QApplication([])

@pytest.fixture

def panel(app):

snap = TVSettingsSnapshot(

oled_light=70,

contrast=85,

gamma="bt1886",

chip_generation=ChipGeneration.ALPHA9_GEN4,

)

p = SettingsPanel(snapshot=snap, on_write=lambda coro: None)

yield p

p.close()

def test_panel_has_five_tabs(panel):

tabs = panel.findChildren(QTabWidget)

assert len(tabs) == 1

assert tabs[0].count() == 5

def test_oled_light_slider_reflects_snapshot(panel):

sliders = panel.findChildren(QSlider)

# The OLED light slider should be set to 70

oled_slider = next((s for s in sliders if s.value() == 70), None)

assert oled_slider is not None

def test_gamma_combobox_reflects_snapshot(panel):

combos = panel.findChildren(QComboBox)

gamma_combo = next((c for c in combos if c.findText("BT.1886") >= 0), None)

assert gamma_combo is not None

assert "1886" in gamma_combo.currentText().upper() or "BT.1886" in gamma_combo.currentText()

def test_panel_calls_on_write_when_slider_changes(app):

wrote = []

snap = TVSettingsSnapshot(oled_light=50)

panel = SettingsPanel(snapshot=snap, on_write=lambda coro: wrote.append(coro))

sliders = panel.findChildren(QSlider)

oled_slider = next((s for s in sliders if s.objectName() == "oled_light"), None)

if oled_slider:

oled_slider.setValue(80)

assert len(wrote) > 0

panel.close()

☐  Step 2: Run tests to verify they fail

pytest tests/unit/test_settings_panel.py -v

Expected: ImportError

☐  Step 3: Implement SettingsPanel with all 5 tabs

# app/ui/settings_panel.py

from typing import Callable, Coroutine

from PyQt6.QtWidgets import (

QWidget, QVBoxLayout, QHBoxLayout, QTabWidget, QLabel,

QSlider, QComboBox, QPushButton, QGroupBox, QGridLayout,

QSpinBox, QCheckBox, QScrollArea, QFrame

)

from PyQt6.QtCore import Qt

from app.tv.state import TVSettingsSnapshot

_STYLE_SLIDER = "QSlider::groove:horizontal { height: 6px; background: #2a2a3e; border-radius: 3px; } QSlider::handle:horizontal { background: #4fc3f7; width: 14px; height: 14px; margin: -4px 0; border-radius: 7px; } QSlider::sub-page:horizontal { background: #4fc3f7; border-radius: 3px; }"

_STYLE_COMBO = "background: #1a1a2e; color: #ccc; border: 1px solid #333; padding: 4px 8px; border-radius: 4px;"

_STYLE_LABEL = "color: #ccc; font-size: 12px;"

_STYLE_SECTION = "color: #fff; font-size: 13px; font-weight: bold; background: transparent;"

def _labeled_slider(label: str, min_v: int, max_v: int, value: int, obj_name: str, on_change: Callable) -> QWidget:

row = QWidget()

layout = QHBoxLayout(row)

layout.setContentsMargins(0, 0, 0, 0)

lbl = QLabel(label)

lbl.setFixedWidth(130)

lbl.setStyleSheet(_STYLE_LABEL)

slider = QSlider(Qt.Orientation.Horizontal)

slider.setRange(min_v, max_v)

slider.setValue(value)

slider.setObjectName(obj_name)

slider.setStyleSheet(_STYLE_SLIDER)

val_label = QLabel(str(value))

val_label.setFixedWidth(32)

val_label.setStyleSheet("color: #4fc3f7; font-size: 12px;")

slider.valueChanged.connect(lambda v: val_label.setText(str(v)))

slider.valueChanged.connect(on_change)

layout.addWidget(lbl)

layout.addWidget(slider, 1)

layout.addWidget(val_label)

return row

def _labeled_combo(label: str, options: list[str], current: str, on_change: Callable) -> QWidget:

row = QWidget()

layout = QHBoxLayout(row)

layout.setContentsMargins(0, 0, 0, 0)

lbl = QLabel(label)

lbl.setFixedWidth(130)

lbl.setStyleSheet(_STYLE_LABEL)

combo = QComboBox()

combo.setStyleSheet(_STYLE_COMBO)

for opt in options:

combo.addItem(opt)

idx = combo.findText(current, Qt.MatchFlag.MatchFixedString)

if idx >= 0:

combo.setCurrentIndex(idx)

combo.currentTextChanged.connect(on_change)

layout.addWidget(lbl)

layout.addWidget(combo, 1)

return row

class SettingsPanel(QWidget):

def __init__(self, snapshot: TVSettingsSnapshot, on_write: Callable[[Coroutine], None], parent=None):

super().__init__(parent)

self._snap = snapshot

self._on_write = on_write

self._build_ui()

def _build_ui(self):

layout = QVBoxLayout(self)

layout.setContentsMargins(0, 0, 0, 0)

self._tabs = QTabWidget()

self._tabs.setStyleSheet(

"QTabWidget::pane { background: #111; border: none; }"

"QTabBar::tab { background: #1a1a2e; color: #aaa; padding: 8px 16px; border: none; }"

"QTabBar::tab:selected { background: #2a2a3e; color: #fff; border-bottom: 2px solid #4fc3f7; }"

)

self._tabs.addTab(self._build_picture_tab(), "Picture")

self._tabs.addTab(self._build_white_balance_tab(), "White Balance")

self._tabs.addTab(self._build_gamma_tab(), "Gamma / Color Space")

self._tabs.addTab(self._build_cms_tab(), "Color Management")

self._tabs.addTab(self._build_hdr_tab(), "HDR / Dynamic")

layout.addWidget(self._tabs)

def _scrollable(self, inner: QWidget) -> QScrollArea:

scroll = QScrollArea()

scroll.setWidgetResizable(True)

scroll.setStyleSheet("background: #111; border: none;")

scroll.setWidget(inner)

return scroll

def _build_picture_tab(self) -> QWidget:

w = QWidget()

w.setStyleSheet("background: #111;")

layout = QVBoxLayout(w)

layout.setContentsMargins(24, 20, 24, 20)

layout.setSpacing(12)

s = self._snap

layout.addWidget(_labeled_slider("OLED Light", 0, 100, s.oled_light, "oled_light",

lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_slider("Contrast", 0, 100, s.contrast, "contrast",

lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_slider("Brightness", 0, 100, s.brightness, "brightness",

lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_slider("Sharpness", 0, 50, s.sharpness, "sharpness",

lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_slider("Color", 0, 100, s.color, "color",

lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_slider("Tint", -10, 10, s.tint, "tint",

lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_combo("Color Temperature",

["Warm 50", "Warm", "Natural", "Cool", "Manual"],

s.color_temperature, lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_combo("Picture Mode",

["Expert1", "Expert2", "Cinema", "ISF Bright", "ISF Dark"],

s.pic_mode.capitalize(), lambda v: self._on_write(self._noop())))

layout.addStretch()

return self._scrollable(w)

def _build_white_balance_tab(self) -> QWidget:

w = QWidget()

w.setStyleSheet("background: #111;")

layout = QVBoxLayout(w)

layout.setContentsMargins(24, 20, 24, 20)

layout.setSpacing(12)

s = self._snap

toggle = _labeled_combo("Method", ["2-Point", "20-Point"], "2-Point",

lambda v: self._on_write(self._noop()))

layout.addWidget(toggle)

layout.addWidget(QLabel("2-Point White Balance:").also(

lambda l: l.setStyleSheet(_STYLE_SECTION)) if False else

self._section_label("2-Point White Balance"))

for label, obj, val in [

("Red Gain", "wb_2pt_red_gain", s.wb_2pt_red_gain),

("Green Gain", "wb_2pt_green_gain", s.wb_2pt_green_gain),

("Blue Gain", "wb_2pt_blue_gain", s.wb_2pt_blue_gain),

("Red Offset", "wb_2pt_red_offset", s.wb_2pt_red_offset),

("Green Offset", "wb_2pt_green_offset", s.wb_2pt_green_offset),

("Blue Offset", "wb_2pt_blue_offset", s.wb_2pt_blue_offset),

]:

layout.addWidget(_labeled_slider(label, -50, 50, val, obj,

lambda v: self._on_write(self._noop())))

btn_row = QHBoxLayout()

reset_btn = QPushButton("Reset to Defaults")

reset_btn.setStyleSheet("background: #2a2a3e; color: #aaa; padding: 6px 12px; border-radius: 4px;")

copy_btn = QPushButton("Copy C1 → C2")

copy_btn.setStyleSheet("background: #2a2a3e; color: #aaa; padding: 6px 12px; border-radius: 4px;")

btn_row.addWidget(reset_btn)

btn_row.addWidget(copy_btn)

btn_row.addStretch()

layout.addLayout(btn_row)

layout.addStretch()

return self._scrollable(w)

def _build_gamma_tab(self) -> QWidget:

w = QWidget()

w.setStyleSheet("background: #111;")

layout = QVBoxLayout(w)

layout.setContentsMargins(24, 20, 24, 20)

layout.setSpacing(12)

s = self._snap

gamma_map = {"1.8": "1.8", "2.0": "2.0", "2.2": "2.2", "2.4": "2.4",

"BT.1886": "bt1886", "sRGB": "srgb"}

gamma_display = next((k for k, v in gamma_map.items() if v == s.gamma), "BT.1886")

layout.addWidget(_labeled_combo("Gamma", list(gamma_map.keys()), gamma_display,

lambda v: self._on_write(self._noop())))

cs_map = {"Auto": "auto", "Native": "native", "BT.709": "bt709",

"BT.2020": "bt2020", "DCI-P3": "dcip3"}

cs_display = next((k for k, v in cs_map.items() if v == s.color_space), "Auto")

layout.addWidget(_labeled_combo("Color Space", list(cs_map.keys()), cs_display,

lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_combo("Black Level", ["Low", "High", "Auto"],

s.black_level.capitalize(), lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_combo("TruMotion", ["Off", "Smooth", "Clear", "User"],

s.trumotion.capitalize(), lambda v: self._on_write(self._noop())))

layout.addStretch()

return self._scrollable(w)

def _build_cms_tab(self) -> QWidget:

w = QWidget()

w.setStyleSheet("background: #111;")

layout = QVBoxLayout(w)

layout.setContentsMargins(24, 20, 24, 20)

layout.setSpacing(12)

color_selector = _labeled_combo("Color", ["Red", "Green", "Blue", "Cyan", "Magenta", "Yellow"],

"Red", lambda v: self._on_write(self._noop()))

layout.addWidget(color_selector)

s = self._snap

layout.addWidget(_labeled_slider("Hue", -30, 30, s.cms_red_hue, "cms_hue",

lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_slider("Saturation", -30, 30, s.cms_red_saturation, "cms_sat",

lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_slider("Luminance", -30, 30, s.cms_red_luminance, "cms_lum",

lambda v: self._on_write(self._noop())))

btn_row = QHBoxLayout()

reset_color = QPushButton("Reset Color")

reset_color.setStyleSheet("background: #2a2a3e; color: #aaa; padding: 6px 12px; border-radius: 4px;")

reset_all = QPushButton("Reset All Colors")

reset_all.setStyleSheet("background: #2a2a3e; color: #aaa; padding: 6px 12px; border-radius: 4px;")

btn_row.addWidget(reset_color)

btn_row.addWidget(reset_all)

btn_row.addStretch()

layout.addLayout(btn_row)

layout.addStretch()

return self._scrollable(w)

def _build_hdr_tab(self) -> QWidget:

w = QWidget()

w.setStyleSheet("background: #111;")

layout = QVBoxLayout(w)

layout.setContentsMargins(24, 20, 24, 20)

layout.setSpacing(12)

s = self._snap

layout.addWidget(_labeled_combo("Dynamic Contrast", ["Off", "Low", "Medium", "High"],

s.dynamic_contrast.capitalize(), lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_combo("Dynamic Color", ["Off", "Low", "High"],

s.dynamic_color.capitalize(), lambda v: self._on_write(self._noop())))

asbl_row = QHBoxLayout()

asbl_lbl = QLabel("ASBL")

asbl_lbl.setFixedWidth(130)

asbl_lbl.setStyleSheet(_STYLE_LABEL)

asbl_check = QCheckBox()

asbl_check.setChecked(s.asbl)

asbl_check.stateChanged.connect(lambda v: self._on_write(self._noop()))

asbl_row.addWidget(asbl_lbl)

asbl_row.addWidget(asbl_check)

asbl_row.addStretch()

layout.addLayout(asbl_row)

layout.addWidget(_labeled_combo("HDR Tone Mapping", ["On", "Off"],

"On" if s.hdr_tone_mapping else "Off", lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_slider("Peak Luminance (nits)", 100, 4000, s.peak_luminance,

"peak_luminance", lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_combo("DV Picture Mode", ["Bright", "Dark", "Vivid"],

s.dv_picture_mode.capitalize(), lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_combo("Local Dimming", ["Off", "Low", "Medium", "High"],

s.local_dimming.capitalize(), lambda v: self._on_write(self._noop())))

layout.addWidget(_labeled_combo("Energy Saving", ["Off", "Min", "Med", "Max", "Auto", "Screen Off"],

s.energy_saving.capitalize(), lambda v: self._on_write(self._noop())))

layout.addStretch()

return self._scrollable(w)

def _section_label(self, text: str) -> QLabel:

lbl = QLabel(text)

lbl.setStyleSheet(_STYLE_SECTION)

return lbl

async def _noop(self):

pass

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_settings_panel.py -v

Expected: 4 tests pass.

☐  Step 5: Verify visually

python -m app.main

Expected: Window launches. Click "📺 Settings" — tab panel shows all 5 tabs with sliders and dropdowns. Sliders are draggable.

☐  Step 6: Commit

git add app/ui/settings_panel.py tests/unit/test_settings_panel.py

git commit -m "feat: SettingsPanel with 5 tabs — Picture, White Balance, Gamma/CS, CMS, HDR"

## Task 10: Wire UI to Backend

Files:

Modify: app/ui/main_window.py

Modify: app/main.py

Create: tests/hardware/test_hardware.py

☐  Step 1: Add write dispatch helpers to SettingsPanel

The _noop() calls in Task 9 are scaffolding. Replace them with real coroutine dispatch by updating app/ui/settings_panel.py. Add a set_connection method and replace _noop() references with actual bscpylgtv / LGTVSettings calls:

# Add to SettingsPanel in app/ui/settings_panel.py

def set_connection(self, mgr, lgtv_settings):

"""Called by MainWindow after connect — wires sliders to real SSAP writes."""

from app.tv.connection import ConnectionManager

from app.tv.settings import LGTVSettings

self._mgr: ConnectionManager = mgr

self._lgtv: LGTVSettings = lgtv_settings

# Replace the OLED Light slider call in _build_picture_tab with:

#   lambda v: self._on_write(self._write_oled_light(v))

# Replace the Contrast slider call with:

#   lambda v: self._on_write(self._write_contrast(v))

async def _write_oled_light(self, value: int):

if self._mgr and self._mgr.client:

await self._mgr.client.set_oled_light(value, self._snap.pic_mode)

self._snap.oled_light = value

async def _write_contrast(self, value: int):

if self._mgr and self._mgr.client:

await self._mgr.client.set_contrast(value, self._snap.pic_mode)

self._snap.contrast = value

async def _write_wb_2pt(self):

"""Called after any 2-point WB slider changes — reads all 6 values and writes in one call."""

if self._lgtv:

await self._lgtv.set_white_balance_2pt(

red_gain=self._snap.wb_2pt_red_gain,

green_gain=self._snap.wb_2pt_green_gain,

blue_gain=self._snap.wb_2pt_blue_gain,

red_offset=self._snap.wb_2pt_red_offset,

green_offset=self._snap.wb_2pt_green_offset,

blue_offset=self._snap.wb_2pt_blue_offset,

)

async def _write_cms(self, color: str):

"""Called after any CMS slider changes for the given color."""

if self._lgtv:

await self._lgtv.set_cms_color(

color,

hue=getattr(self._snap, f"cms_{color}_hue"),

saturation=getattr(self._snap, f"cms_{color}_saturation"),

luminance=getattr(self._snap, f"cms_{color}_luminance"),

)

async def _copy_to_other_tv(self, source_ip: str, target_mgr):

"""Copy white balance 2pt settings from this TV to target_mgr."""

if target_mgr and target_mgr.client:

target_lgtv = LGTVSettings(client=target_mgr.client, pic_mode=self._snap.pic_mode)

await target_lgtv.set_white_balance_2pt(

red_gain=self._snap.wb_2pt_red_gain,

green_gain=self._snap.wb_2pt_green_gain,

blue_gain=self._snap.wb_2pt_blue_gain,

red_offset=self._snap.wb_2pt_red_offset,

green_offset=self._snap.wb_2pt_green_offset,

blue_offset=self._snap.wb_2pt_blue_offset,

)

target_mgr.snapshot.wb_2pt_red_gain = self._snap.wb_2pt_red_gain

target_mgr.snapshot.wb_2pt_green_gain = self._snap.wb_2pt_green_gain

target_mgr.snapshot.wb_2pt_blue_gain = self._snap.wb_2pt_blue_gain

All other sliders follow the same pattern: update self._snap.<field>, then call the appropriate self._mgr.client.<bscpylgtv_method>() or self._lgtv.<extension_method>(). The pattern above covers all cases.

☐  Step 2: Update MainWindow to orchestrate discovery and connection

Add to app/ui/main_window.py:

# Add to imports in main_window.py

import asyncio

from app.tv.discovery import discover_tvs

from app.tv.connection import ConnectionManager

from app.tv.settings import LGTVSettings

from app.ui.discovery_panel import DiscoveryPanel

from app.ui.settings_panel import SettingsPanel

# Add to MainWindow.__init__ after self._build_ui():

#   self._managers: dict[str, ConnectionManager] = {}

#   self._setup_discovery()

# Add these methods to MainWindow:

def _setup_discovery(self):

self._discovery_panel = DiscoveryPanel(on_connect=self._connect_to_tv)

self._discovery_panel.tv_selected.connect(self._on_tv_selected)

self.set_content(self._discovery_panel)

def _on_tv_selected(self, ip: str, name: str):

if ip == "__scan__":

asyncio.ensure_future(self._run_scan())

else:

asyncio.ensure_future(self._connect_to_tv(ip, name))

async def _run_scan(self):

self._discovery_panel.set_scanning(True)

tvs = await discover_tvs()

self._discovery_panel.show_discovered(tvs)

self._discovery_panel.set_scanning(False)

async def _connect_to_tv(self, ip: str, name: str):

mgr = ConnectionManager(ip)

try:

await mgr.connect()

except Exception as e:

self._discovery_panel._status_label.setText(f"Connection failed: {e}")

return

self._managers[ip] = mgr

self.update_tv_status(ip, name, connected=True)

if mgr.firmware_warning:

from PyQt6.QtWidgets import QMessageBox

QMessageBox.warning(

self,

"Firmware Warning",

f"webOS {mgr.snapshot.webos_version} detected.\n"

"This firmware version may be incompatible with calibration commands.",

)

lgtv = LGTVSettings(client=mgr.client, pic_mode=mgr.snapshot.pic_mode)

settings = SettingsPanel(

snapshot=mgr.snapshot,

on_write=lambda coro: asyncio.ensure_future(coro),

)

settings.set_connection(mgr, lgtv)

self.set_content(settings)

☐  Step 2: Run all unit tests to confirm nothing is broken

pytest tests/unit/ -v

Expected: All previously passing tests still pass.

☐  Step 3: Create hardware test skeleton

# tests/hardware/test_hardware.py

import pytest

import asyncio

from app.tv.discovery import discover_tvs

from app.tv.connection import ConnectionManager

from app.tv.state import ChipGeneration

TV_IP = "192.168.1.101"  # Update to your C1/C2 IP before running

@pytest.mark.hardware

async def test_discover_finds_tv():

tvs = await discover_tvs(timeout=5.0)

ips = [tv.ip for tv in tvs]

assert TV_IP in ips, f"Expected to find TV at {TV_IP}. Found: {ips}"

@pytest.mark.hardware

async def test_connect_and_detect_model():

mgr = ConnectionManager(TV_IP)

await mgr.connect()

assert mgr.is_connected

assert mgr.snapshot.chip_generation in (ChipGeneration.ALPHA9_GEN4, ChipGeneration.ALPHA9_GEN5)

await mgr.disconnect()

@pytest.mark.hardware

async def test_read_software_info():

mgr = ConnectionManager(TV_IP)

await mgr.connect()

assert mgr.snapshot.webos_version != ""

await mgr.disconnect()

☐  Step 4: Run unit tests one final time

pytest tests/unit/ -v

Expected: All tests pass. Note hardware tests require a real TV:

pytest tests/hardware/ -v -m hardware  # Only run with TV connected

☐  Step 5: Final commit

git add app/ui/main_window.py app/main.py tests/hardware/test_hardware.py

git commit -m "feat: wire discovery and connection to MainWindow — full app flow working"

## Running the Complete App

source .venv/bin/activate

python -m app.main

Expected flow:

1. Window opens with "Connect to TV" discovery panel in the main area

2. Click "Scan Network" — TV list populates with found LG TVs

3. Double-click a TV — pairing PIN appears on TV screen

4. Enter PIN → app connects, sidebar shows "● C1 Connected"

5. Settings panel appears with 5 tabs — all sliders reflect current TV state

6. Adjust a slider — setting writes to TV in real time

## Running Tests

# Unit tests (no TV needed)

pytest tests/unit/ -v

# Hardware tests (TV must be on, update TV_IP in test_hardware.py)

pytest tests/hardware/ -v -m hardware

# Single test

pytest tests/unit/test_connection.py::test_connect_detects_c1_chip -v

Section 3

# Sub-project 2: LUT Upload Pipeline

> For agentic workers: REQUIRED SUB-SKILL: Use superpowers:subagent-driven-development (recommended) or superpowers:executing-plans to implement this plan task-by-task. Steps use checkbox (- [ ]) syntax for tracking.

Goal: Parse 1D and 3D LUT files (.cube, .cal), upload them to LG OLED TVs via bscpylgtv, upload Dolby Vision configuration files, and expose a LUT management UI panel.

Architecture: A LUTParser module converts standard LUT file formats into the numpy array shapes bscpylgtv expects. A LUTUploader service wraps the bscpylgtv calibration upload methods, routing to the correct call based on LUT type and target picture mode. A DolbyVisionConfig module handles the .txt/.cfg DV config upload as a raw SSAP payload. The LUTPanel PyQt6 widget ties it together with a file browser and upload controls.

Tech Stack: Python 3.11+, PyQt6, bscpylgtv, numpy, pytest, pytest-asyncio, pytest-qt

Prerequisites: Sub-project 1 must be complete (ConnectionManager, TVSettingsSnapshot, LGTVSettings available).

## bscpylgtv Upload API Reference

Before implementing, verify these method signatures in the installed bscpylgtv version:

python -c "import bscpylgtv; help(bscpylgtv.WebOsClient.upload_1d_lut)"

python -c "import bscpylgtv; help(bscpylgtv.WebOsClient.upload_3by3_gamut_data)"

python -c "import bscpylgtv; help(bscpylgtv.WebOsClient.upload_3d_lut_bt709)"

python -c "import bscpylgtv; help(bscpylgtv.WebOsClient.upload_3d_lut_bt2020)"

Expected shapes (from bscpylgtv source):

1D LUT: numpy array shape (3, 1024) — rows are R, G, B; values in range [0, 1]

3x3 gamut matrix: numpy array shape (3, 3) — row-major, values are float coefficients

3D LUT (BT.709/BT.2020): numpy array shape (N, N, N, 3) where N is typically 17 or 33 — last axis is RGB output; input domain [0, 1], output domain [0, 1]

If bscpylgtv uses different shapes, adapt the LUTParser output accordingly.

## File Map

| File | Responsibility |
| --- | --- |
| `app/tv/lut.py` | `LUT1D`, `LUT3D` dataclasses; `.cube` parser; `.cal` parser |
| `app/tv/upload.py` | `LUTUploader` — wraps bscpylgtv upload methods, handles SDR/HDR10/DV routing |
| `app/tv/dv_config.py` | `DolbyVisionConfig` — load and upload `.txt`/`.cfg` DV config files |
| `app/ui/lut_panel.py` | `LUTPanel` — file browser, LUT metadata display, upload controls |
| `tests/unit/test_lut.py` | LUT parsing unit tests (no TV, no bscpylgtv) |
| `tests/unit/test_upload.py` | LUTUploader unit tests (mocked bscpylgtv client) |
| `tests/unit/test_dv_config.py` | DV config parsing unit tests |
| `tests/unit/test_lut_panel.py` | LUT panel UI tests (mocked uploader) |
| `tests/hardware/test_lut_hardware.py` | Hardware LUT upload tests (`@pytest.mark.hardware`) |
| `tests/fixtures/test_lut_17.cube` | 17³ identity 3D LUT fixture |
| `tests/fixtures/test_lut_1d.cal` | 1D LUT cal fixture |
| `tests/fixtures/test_dv_config.txt` | DV config fixture (copy from Resources/) |

## Task 1: LUT Data Models + .cube Parser

Files:

Create: app/tv/lut.py

Create: tests/unit/test_lut.py

Create: tests/fixtures/test_lut_17.cube

☐  Step 1: Create the 17³ identity .cube fixture

Create tests/fixtures/test_lut_17.cube:

TITLE "Identity 17x17x17"

LUT_3D_SIZE 17

DOMAIN_MIN 0.0 0.0 0.0

DOMAIN_MAX 1.0 1.0 1.0

Then append 17³ = 4913 lines of R G B data. The identity LUT has output = input, so the entry at grid index (r, g, b) is (r/16, g/16, b/16). Generate with:

python3 -c "

size = 17

print('TITLE \"Identity 17x17x17\"')

print('LUT_3D_SIZE 17')

print('DOMAIN_MIN 0.0 0.0 0.0')

print('DOMAIN_MAX 1.0 1.0 1.0')

for b in range(size):

for g in range(size):

for r in range(size):

print(f'{r/(size-1):.6f} {g/(size-1):.6f} {b/(size-1):.6f}')

" > tests/fixtures/test_lut_17.cube

☐  Step 2: Create the 1D .cal fixture

Create tests/fixtures/test_lut_1d.cal with a minimal identity 1D LUT in ArgyllCMS .cal format:

CAL

DESCRIPTOR "Identity 1D Cal"

ORIGINATOR "test"

NUMBER_OF_FIELDS 4

BEGIN_DATA_FORMAT

TV_R TV_G TV_B TV_in

END_DATA_FORMAT

NUMBER_OF_SETS 3

BEGIN_DATA

0.000000 0.000000 0.000000 0.000000

0.500000 0.500000 0.500000 0.500000

1.000000 1.000000 1.000000 1.000000

END_DATA

☐  Step 3: Write failing tests

# tests/unit/test_lut.py

import numpy as np

import pytest

from pathlib import Path

from app.tv.lut import LUT1D, LUT3D, parse_cube, parse_cal

FIXTURES = Path(__file__).parent.parent / "fixtures"

def test_parse_cube_identity_17():

lut = parse_cube(FIXTURES / "test_lut_17.cube")

assert isinstance(lut, LUT3D)

assert lut.size == 17

assert lut.data.shape == (17, 17, 17, 3)

# identity LUT: output at (8, 8, 8) should be (0.5, 0.5, 0.5)

np.testing.assert_allclose(lut.data[8, 8, 8], [0.5, 0.5, 0.5], atol=1e-4)

def test_parse_cube_domain_clamped():

lut = parse_cube(FIXTURES / "test_lut_17.cube")

assert lut.domain_min == (0.0, 0.0, 0.0)

assert lut.domain_max == (1.0, 1.0, 1.0)

def test_parse_cube_values_in_range():

lut = parse_cube(FIXTURES / "test_lut_17.cube")

assert lut.data.min() >= 0.0

assert lut.data.max() <= 1.0

def test_parse_cal_identity():

lut = parse_cal(FIXTURES / "test_lut_1d.cal")

assert isinstance(lut, LUT1D)

assert lut.data.shape[0] == 3   # R, G, B rows

assert lut.data.shape[1] > 0    # at least one entry

def test_parse_cube_wrong_extension_raises():

with pytest.raises(ValueError, match="Expected .cube"):

parse_cube(Path("file.txt"))

def test_parse_cal_wrong_extension_raises():

with pytest.raises(ValueError, match="Expected .cal"):

parse_cal(Path("file.txt"))

☐  Step 4: Run tests to verify they fail

pytest tests/unit/test_lut.py -v

Expected: ImportError (module does not exist yet)

☐  Step 5: Implement LUT data models and parsers

# app/tv/lut.py

from __future__ import annotations

from dataclasses import dataclass, field

from pathlib import Path

from typing import Tuple

import numpy as np

@dataclass

class LUT1D:

"""1D LUT: shape (3, N) — rows are R, G, B; values in [0, 1]."""

data: np.ndarray          # shape (3, N)

title: str = ""

@dataclass

class LUT3D:

"""3D LUT: shape (N, N, N, 3) — axes are B, G, R input; last axis is RGB output."""

data: np.ndarray          # shape (N, N, N, 3)

size: int = 17

domain_min: Tuple[float, float, float] = (0.0, 0.0, 0.0)

domain_max: Tuple[float, float, float] = (1.0, 1.0, 1.0)

title: str = ""

def parse_cube(path: Path) -> LUT3D:

"""Parse an Adobe .cube 3D LUT file."""

path = Path(path)

if path.suffix.lower() != ".cube":

raise ValueError(f"Expected .cube file, got: {path.suffix}")

title = ""

size = None

domain_min = (0.0, 0.0, 0.0)

domain_max = (1.0, 1.0, 1.0)

data_lines: list[str] = []

with open(path) as f:

for line in f:

line = line.strip()

if not line or line.startswith("#"):

continue

if line.startswith("TITLE"):

title = line.split(None, 1)[1].strip('"')

elif line.startswith("LUT_3D_SIZE"):

size = int(line.split()[1])

elif line.startswith("DOMAIN_MIN"):

parts = line.split()

domain_min = (float(parts[1]), float(parts[2]), float(parts[3]))

elif line.startswith("DOMAIN_MAX"):

parts = line.split()

domain_max = (float(parts[1]), float(parts[2]), float(parts[3]))

else:

data_lines.append(line)

if size is None:

raise ValueError(f"LUT_3D_SIZE not found in {path}")

if len(data_lines) != size ** 3:

raise ValueError(

f"Expected {size**3} data lines, got {len(data_lines)} in {path}"

)

# .cube iterates R fastest, then G, then B

raw = np.array([[float(v) for v in ln.split()] for ln in data_lines],

dtype=np.float32)

# Reshape to (B, G, R, 3) then transpose to (R, G, B, 3) for intuitive indexing

data = raw.reshape(size, size, size, 3).transpose(2, 1, 0, 3)

return LUT3D(data=data, size=size, domain_min=domain_min,

domain_max=domain_max, title=title)

def parse_cal(path: Path) -> LUT1D:

"""Parse an ArgyllCMS .cal 1D LUT file."""

path = Path(path)

if path.suffix.lower() != ".cal":

raise ValueError(f"Expected .cal file, got: {path.suffix}")

title = ""

in_data = False

r_vals, g_vals, b_vals = [], [], []

with open(path) as f:

for line in f:

line = line.strip()

if line.startswith("DESCRIPTOR"):

title = line.split(None, 1)[1].strip('"')

elif line == "BEGIN_DATA":

in_data = True

elif line == "END_DATA":

in_data = False

elif in_data and line:

parts = line.split()

# .cal data format: TV_R TV_G TV_B TV_in (all normalized 0..1)

r_vals.append(float(parts[0]))

g_vals.append(float(parts[1]))

b_vals.append(float(parts[2]))

data = np.array([r_vals, g_vals, b_vals], dtype=np.float32)

return LUT1D(data=data, title=title)

☐  Step 6: Run tests to verify they pass

pytest tests/unit/test_lut.py -v

Expected: 6 tests pass.

☐  Step 7: Commit

git add app/tv/lut.py tests/unit/test_lut.py tests/fixtures/

git commit -m "feat: LUT1D/LUT3D data models with .cube and .cal parsers"

## Task 2: LUT Uploader

Files:

Create: app/tv/upload.py

Create: tests/unit/test_upload.py

☐  Step 1: Write failing tests

# tests/unit/test_upload.py

import numpy as np

import pytest

from unittest.mock import AsyncMock, MagicMock, patch

from app.tv.lut import LUT1D, LUT3D

from app.tv.upload import LUTUploader, LUTTarget

@pytest.fixture

def mock_client():

client = AsyncMock()

client.upload_1d_lut = AsyncMock(return_value={"returnValue": True})

client.upload_3by3_gamut_data = AsyncMock(return_value={"returnValue": True})

client.upload_3d_lut_bt709 = AsyncMock(return_value={"returnValue": True})

client.upload_3d_lut_bt2020 = AsyncMock(return_value={"returnValue": True})

return client

@pytest.fixture

def uploader(mock_client):

return LUTUploader(client=mock_client, pic_mode="expert1")

@pytest.fixture

def identity_1d():

data = np.tile(np.linspace(0, 1, 1024, dtype=np.float32), (3, 1))

return LUT1D(data=data)

@pytest.fixture

def identity_3d():

size = 17

data = np.zeros((size, size, size, 3), dtype=np.float32)

for r in range(size):

for g in range(size):

for b in range(size):

data[r, g, b] = [r / (size - 1), g / (size - 1), b / (size - 1)]

return LUT3D(data=data, size=size)

async def test_upload_1d_lut_calls_client(uploader, mock_client, identity_1d):

await uploader.upload_1d(identity_1d)

mock_client.upload_1d_lut.assert_called_once()

async def test_upload_3d_lut_bt709_calls_client(uploader, mock_client, identity_3d):

await uploader.upload_3d(identity_3d, target=LUTTarget.BT709)

mock_client.upload_3d_lut_bt709.assert_called_once()

async def test_upload_3d_lut_bt2020_calls_client(uploader, mock_client, identity_3d):

await uploader.upload_3d(identity_3d, target=LUTTarget.BT2020)

mock_client.upload_3d_lut_bt2020.assert_called_once()

async def test_upload_3d_wrong_target_raises(uploader, identity_3d):

with pytest.raises(ValueError, match="Unknown LUTTarget"):

await uploader.upload_3d(identity_3d, target="bad_target")

async def test_upload_failure_raises(uploader, mock_client, identity_1d):

mock_client.upload_1d_lut = AsyncMock(return_value={"returnValue": False,

"errorText": "upload failed"})

with pytest.raises(RuntimeError, match="upload failed"):

await uploader.upload_1d(identity_1d)

☐  Step 2: Run tests to verify they fail

pytest tests/unit/test_upload.py -v

Expected: ImportError

☐  Step 3: Implement LUTUploader

# app/tv/upload.py

from __future__ import annotations

from enum import Enum

import numpy as np

from bscpylgtv import WebOsClient

from app.tv.lut import LUT1D, LUT3D

class LUTTarget(str, Enum):

BT709 = "bt709"

BT2020 = "bt2020"

class LUTUploader:

def __init__(self, client: WebOsClient, pic_mode: str = "expert1"):

self._client = client

self.pic_mode = pic_mode

async def upload_1d(self, lut: LUT1D) -> None:

"""Upload 1D LUT. bscpylgtv expects shape (3, 1024) in [0, 1]."""

data = _ensure_1d_shape(lut.data)

result = await self._client.upload_1d_lut(data)

_check_result(result)

async def upload_3d(self, lut: LUT3D, target: LUTTarget) -> None:

"""Upload 3D LUT to the specified color space slot."""

data = lut.data.astype(np.float32)

if target == LUTTarget.BT709:

result = await self._client.upload_3d_lut_bt709(data)

elif target == LUTTarget.BT2020:

result = await self._client.upload_3d_lut_bt2020(data)

else:

raise ValueError(f"Unknown LUTTarget: {target!r}")

_check_result(result)

async def upload_gamut_matrix(self, matrix: np.ndarray) -> None:

"""Upload a 3×3 gamut correction matrix."""

if matrix.shape != (3, 3):

raise ValueError(f"Gamut matrix must be (3, 3), got {matrix.shape}")

result = await self._client.upload_3by3_gamut_data(matrix.astype(np.float32))

_check_result(result)

def _ensure_1d_shape(data: np.ndarray) -> np.ndarray:

"""Interpolate or resample 1D LUT data to shape (3, 1024)."""

if data.shape == (3, 1024):

return data.astype(np.float32)

# Resample each channel to 1024 points via linear interpolation

x_in = np.linspace(0, 1, data.shape[1])

x_out = np.linspace(0, 1, 1024)

out = np.zeros((3, 1024), dtype=np.float32)

for ch in range(3):

out[ch] = np.interp(x_out, x_in, data[ch])

return out

def _check_result(result: dict) -> None:

if not result.get("returnValue", False):

error = result.get("errorText", "unknown error")

raise RuntimeError(f"LUT upload failed: {error}")

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_upload.py -v

Expected: 5 tests pass.

☐  Step 5: Commit

git add app/tv/upload.py tests/unit/test_upload.py

git commit -m "feat: LUTUploader wraps bscpylgtv upload methods for 1D/3D LUT and gamut matrix"

## Task 3: Dolby Vision Config Upload

Files:

Create: app/tv/dv_config.py

Create: tests/unit/test_dv_config.py

Create: tests/fixtures/test_dv_config.txt

☐  Step 1: Create the DV config fixture

Copy the existing DV config as a test fixture:

cp "Resources/(5) - Dolby Vision CFG File/DolbyVision_UserDisplayConfiguration.txt" \

tests/fixtures/test_dv_config.txt

☐  Step 2: Write failing tests

# tests/unit/test_dv_config.py

import pytest

from pathlib import Path

from unittest.mock import AsyncMock

from app.tv.dv_config import DolbyVisionConfig, load_dv_config

FIXTURES = Path(__file__).parent.parent / "fixtures"

def test_load_dv_config_parses_file():

cfg = load_dv_config(FIXTURES / "test_dv_config.txt")

assert isinstance(cfg, DolbyVisionConfig)

assert len(cfg.raw_text) > 0

def test_load_dv_config_wrong_extension_raises():

with pytest.raises(ValueError, match="Expected .txt or .cfg"):

load_dv_config(Path("file.xyz"))

async def test_upload_dv_config_sends_ssap():

mock_client = AsyncMock()

mock_client.request = AsyncMock(return_value={"returnValue": True})

cfg = DolbyVisionConfig(raw_text="[DisplayConfiguration]\nVersion=2\n")

await cfg.upload(mock_client)

mock_client.request.assert_called_once()

uri, payload = mock_client.request.call_args[0]

assert "dolby" in uri.lower() or "externalpq" in uri.lower()

async def test_upload_dv_config_failure_raises():

mock_client = AsyncMock()

mock_client.request = AsyncMock(return_value={"returnValue": False,

"errorText": "not supported"})

cfg = DolbyVisionConfig(raw_text="[DisplayConfiguration]\n")

with pytest.raises(RuntimeError, match="not supported"):

await cfg.upload(mock_client)

☐  Step 3: Run tests to verify they fail

pytest tests/unit/test_dv_config.py -v

Expected: ImportError

☐  Step 4: Implement DV config module

# app/tv/dv_config.py

from __future__ import annotations

from dataclasses import dataclass

from pathlib import Path

# DV config is uploaded via the externalpq SSAP endpoint.

# Verify against bscpylgtv source or LightSpace documentation if this URI changes.

_DV_CONFIG_URI = "ssap://externalpq/setDolbyVisionUserDisplayConfiguration"

@dataclass

class DolbyVisionConfig:

raw_text: str

async def upload(self, client) -> None:

"""Upload the DV config to the TV via SSAP."""

payload = {"data": self.raw_text}

result = await client.request(_DV_CONFIG_URI, payload)

if not result.get("returnValue", False):

error = result.get("errorText", "unknown error")

raise RuntimeError(f"DV config upload failed: {error}")

def load_dv_config(path: Path) -> DolbyVisionConfig:

"""Load a Dolby Vision user display configuration file (.txt or .cfg)."""

path = Path(path)

if path.suffix.lower() not in (".txt", ".cfg"):

raise ValueError(f"Expected .txt or .cfg file, got: {path.suffix}")

return DolbyVisionConfig(raw_text=path.read_text())

☐  Step 5: Run tests to verify they pass

pytest tests/unit/test_dv_config.py -v

Expected: 4 tests pass. If the DV config SSAP URI is wrong (returns False from a real TV), update _DV_CONFIG_URI to match the correct endpoint — check the LightSpace CMS guides in Resources/ for the actual SSAP URI used during DV calibration.

☐  Step 6: Commit

git add app/tv/dv_config.py tests/unit/test_dv_config.py tests/fixtures/test_dv_config.txt

git commit -m "feat: DolbyVisionConfig loads and uploads DV user display configuration via SSAP"

## Task 4: LUT Panel UI

Files:

Create: app/ui/lut_panel.py

Create: tests/unit/test_lut_panel.py

☐  Step 1: Write failing tests

# tests/unit/test_lut_panel.py

import pytest

from unittest.mock import AsyncMock, MagicMock, patch

from PyQt6.QtWidgets import QApplication, QLabel, QPushButton, QComboBox

from app.ui.lut_panel import LUTPanel

from app.tv.lut import LUT3D

import numpy as np

@pytest.fixture(scope="module")

def app():

return QApplication.instance() or QApplication([])

@pytest.fixture

def panel(app):

p = LUTPanel(on_upload=AsyncMock())

yield p

p.close()

def test_panel_creates(panel):

assert panel is not None

def test_panel_has_upload_button(panel):

buttons = [b.text() for b in panel.findChildren(QPushButton)]

assert any("upload" in t.lower() or "Upload" in t for t in buttons)

def test_panel_has_target_selector(panel):

combos = panel.findChildren(QComboBox)

assert len(combos) >= 1

texts = []

for c in combos:

for i in range(c.count()):

texts.append(c.itemText(i))

assert any("BT.709" in t or "709" in t for t in texts)

def test_panel_shows_no_file_initially(panel):

labels = [l.text() for l in panel.findChildren(QLabel)]

assert any("no file" in t.lower() or "select" in t.lower() or "" == t for t in labels)

☐  Step 2: Run tests to verify they fail

pytest tests/unit/test_lut_panel.py -v

Expected: ImportError

☐  Step 3: Implement LUTPanel

# app/ui/lut_panel.py

from __future__ import annotations

from typing import Callable, Optional

from pathlib import Path

from PyQt6.QtWidgets import (

QWidget, QVBoxLayout, QHBoxLayout, QPushButton, QLabel,

QComboBox, QFileDialog, QGroupBox, QGridLayout, QMessageBox

)

from PyQt6.QtCore import Qt

from app.tv.lut import LUT1D, LUT3D, parse_cube, parse_cal

from app.tv.upload import LUTTarget

_STYLE_LABEL = "color: #ccc; font-size: 12px;"

_STYLE_BTN_PRIMARY = (

"background: #4fc3f7; color: #000; padding: 8px 16px; border-radius: 4px; font-weight: bold;"

)

_STYLE_BTN_SECONDARY = (

"background: #2a2a3e; color: #aaa; padding: 6px 12px; border-radius: 4px;"

)

_STYLE_COMBO = (

"background: #1a1a2e; color: #ccc; border: 1px solid #333; padding: 4px 8px; border-radius: 4px;"

)

class LUTPanel(QWidget):

def __init__(self, on_upload: Callable, parent=None):

super().__init__(parent)

self._on_upload = on_upload

self._loaded_lut: Optional[LUT1D | LUT3D] = None

self._lut_path: Optional[Path] = None

self._build_ui()

def _build_ui(self):

layout = QVBoxLayout(self)

layout.setContentsMargins(24, 24, 24, 24)

layout.setSpacing(16)

title = QLabel("LUT Files")

title.setStyleSheet("color: #fff; font-size: 18px; font-weight: bold;")

layout.addWidget(title)

# File selector group

file_group = QGroupBox("LUT File")

file_group.setStyleSheet(

"QGroupBox { color: #aaa; font-size: 12px; border: 1px solid #333; border-radius: 4px; margin-top: 8px; }"

"QGroupBox::title { padding: 0 4px; }"

)

file_layout = QVBoxLayout(file_group)

file_row = QHBoxLayout()

self._file_label = QLabel("No file selected")

self._file_label.setStyleSheet("color: #aaa; font-size: 12px;")

browse_btn = QPushButton("Browse…")

browse_btn.setStyleSheet(_STYLE_BTN_SECONDARY)

browse_btn.clicked.connect(self._on_browse)

file_row.addWidget(self._file_label, 1)

file_row.addWidget(browse_btn)

file_layout.addLayout(file_row)

self._lut_info_label = QLabel("")

self._lut_info_label.setStyleSheet("color: #4fc3f7; font-size: 11px;")

file_layout.addWidget(self._lut_info_label)

layout.addWidget(file_group)

# Upload options group

options_group = QGroupBox("Upload Options")

options_group.setStyleSheet(file_group.styleSheet())

options_layout = QGridLayout(options_group)

options_layout.setSpacing(10)

options_layout.addWidget(QLabel("Color Space Target:").also(

lambda l: l.setStyleSheet(_STYLE_LABEL)) if False else

self._make_label("Color Space Target:"), 0, 0)

self._target_combo = QComboBox()

self._target_combo.setStyleSheet(_STYLE_COMBO)

self._target_combo.addItems(["BT.709 (SDR)", "BT.2020 (HDR10 / HLG)"])

options_layout.addWidget(self._target_combo, 0, 1)

options_layout.addWidget(self._make_label("Picture Mode:"), 1, 0)

self._picmode_combo = QComboBox()

self._picmode_combo.setStyleSheet(_STYLE_COMBO)

self._picmode_combo.addItems(["expert1", "expert2", "cinema", "isf_bright", "isf_dark"])

options_layout.addWidget(self._picmode_combo, 1, 1)

layout.addWidget(options_group)

# DV Config group

dv_group = QGroupBox("Dolby Vision Config")

dv_group.setStyleSheet(file_group.styleSheet())

dv_layout = QVBoxLayout(dv_group)

dv_row = QHBoxLayout()

self._dv_label = QLabel("No file selected")

self._dv_label.setStyleSheet("color: #aaa; font-size: 12px;")

dv_browse_btn = QPushButton("Browse…")

dv_browse_btn.setStyleSheet(_STYLE_BTN_SECONDARY)

dv_browse_btn.clicked.connect(self._on_browse_dv)

dv_row.addWidget(self._dv_label, 1)

dv_row.addWidget(dv_browse_btn)

dv_layout.addLayout(dv_row)

dv_upload_btn = QPushButton("Upload DV Config")

dv_upload_btn.setStyleSheet(_STYLE_BTN_SECONDARY)

dv_upload_btn.clicked.connect(self._on_upload_dv)

dv_layout.addWidget(dv_upload_btn)

layout.addWidget(dv_group)

# Upload button

self._upload_btn = QPushButton("Upload LUT to TV")

self._upload_btn.setStyleSheet(_STYLE_BTN_PRIMARY)

self._upload_btn.setEnabled(False)

self._upload_btn.clicked.connect(self._on_upload_clicked)

layout.addWidget(self._upload_btn)

self._status_label = QLabel("")

self._status_label.setStyleSheet("color: #4fc3f7; font-size: 12px;")

layout.addWidget(self._status_label)

layout.addStretch()

def _make_label(self, text: str) -> QLabel:

lbl = QLabel(text)

lbl.setStyleSheet(_STYLE_LABEL)

return lbl

def _on_browse(self):

path, _ = QFileDialog.getOpenFileName(

self, "Select LUT File", "",

"LUT Files (*.cube *.cal);;Cube (*.cube);;Cal (*.cal)"

)

if not path:

return

try:

p = Path(path)

if p.suffix.lower() == ".cube":

self._loaded_lut = parse_cube(p)

info = f"3D LUT — {self._loaded_lut.size}³ — {p.name}"

elif p.suffix.lower() == ".cal":

self._loaded_lut = parse_cal(p)

info = f"1D LUT — {self._loaded_lut.data.shape[1]} pts — {p.name}"

else:

self._status_label.setText(f"Unsupported file type: {p.suffix}")

return

self._lut_path = p

self._file_label.setText(p.name)

self._lut_info_label.setText(info)

self._upload_btn.setEnabled(True)

except Exception as e:

self._status_label.setText(f"Parse error: {e}")

def _on_browse_dv(self):

path, _ = QFileDialog.getOpenFileName(

self, "Select Dolby Vision Config", "",

"DV Config (*.txt *.cfg)"

)

if path:

self._dv_path = Path(path)

self._dv_label.setText(self._dv_path.name)

def _on_upload_clicked(self):

if self._loaded_lut is None:

return

target_text = self._target_combo.currentText()

target = LUTTarget.BT709 if "709" in target_text else LUTTarget.BT2020

pic_mode = self._picmode_combo.currentText()

self._on_upload(self._loaded_lut, target, pic_mode)

def _on_upload_dv(self):

if not hasattr(self, "_dv_path"):

return

self._on_upload(self._dv_path, None, None)

def set_status(self, text: str, error: bool = False):

color = "#f44336" if error else "#4fc3f7"

self._status_label.setStyleSheet(f"color: {color}; font-size: 12px;")

self._status_label.setText(text)

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_lut_panel.py -v

Expected: 4 tests pass.

☐  Step 5: Commit

git add app/ui/lut_panel.py tests/unit/test_lut_panel.py

git commit -m "feat: LUTPanel UI with file browser, target selector, and upload button"

## Task 5: Hardware LUT Upload Tests

Files:

Create: tests/hardware/test_lut_hardware.py

☐  Step 1: Create hardware test skeleton

# tests/hardware/test_lut_hardware.py

import pytest

import numpy as np

from pathlib import Path

from app.tv.connection import ConnectionManager

from app.tv.upload import LUTUploader, LUTTarget

from app.tv.lut import LUT1D, LUT3D

TV_IP = "192.168.1.101"  # Update before running

FIXTURES = Path(__file__).parent.parent / "fixtures"

@pytest.fixture

async def connected_mgr():

mgr = ConnectionManager(TV_IP)

await mgr.connect()

yield mgr

await mgr.disconnect()

@pytest.mark.hardware

async def test_upload_identity_1d_lut(connected_mgr):

"""Upload an identity 1D LUT — should be a no-op visually."""

data = np.tile(np.linspace(0, 1, 1024, dtype=np.float32), (3, 1))

lut = LUT1D(data=data)

uploader = LUTUploader(client=connected_mgr.client, pic_mode="expert1")

await uploader.upload_1d(lut)

@pytest.mark.hardware

async def test_upload_identity_3d_lut_bt709(connected_mgr):

"""Upload an identity 3D LUT to BT.709 slot — should be a no-op visually."""

size = 17

data = np.zeros((size, size, size, 3), dtype=np.float32)

for r in range(size):

for g in range(size):

for b in range(size):

data[r, g, b] = [r / (size - 1), g / (size - 1), b / (size - 1)]

lut = LUT3D(data=data, size=size)

uploader = LUTUploader(client=connected_mgr.client, pic_mode="expert1")

await uploader.upload_3d(lut, target=LUTTarget.BT709)

@pytest.mark.hardware

async def test_upload_cube_file_from_disk(connected_mgr):

"""Parse a real .cube file and upload it."""

from app.tv.lut import parse_cube

cube_path = FIXTURES / "test_lut_17.cube"

lut = parse_cube(cube_path)

uploader = LUTUploader(client=connected_mgr.client, pic_mode="expert1")

await uploader.upload_3d(lut, target=LUTTarget.BT709)

☐  Step 2: Run all unit tests one final time

pytest tests/unit/ -v

Expected: All previously passing tests still pass.

☐  Step 3: Commit

git add tests/hardware/test_lut_hardware.py

git commit -m "feat: hardware LUT upload integration tests (require real TV)"

## Task 6: Wire LUTPanel into MainWindow

Files:

Modify: app/ui/main_window.py

☐  Step 1: Add LUT panel to navigation

In app/ui/main_window.py, after _setup_discovery() initialization, add:

# In MainWindow.__init__, after self._setup_discovery():

self._lut_panel = LUTPanel(on_upload=self._handle_lut_upload)

Add to imports at top of app/ui/main_window.py:

import asyncio

from app.ui.lut_panel import LUTPanel

from app.tv.upload import LUTUploader, LUTTarget

from app.tv.dv_config import load_dv_config

from app.tv.lut import LUT1D, LUT3D

from pathlib import Path

Add method to MainWindow:

async def _handle_lut_upload(self, lut_or_path, target, pic_mode):

"""Dispatch upload based on type — called by LUTPanel."""

if not self._managers:

self._lut_panel.set_status("No TV connected", error=True)

return

# Upload to all connected TVs

for ip, mgr in self._managers.items():

if not mgr.is_connected:

continue

try:

if isinstance(lut_or_path, Path):

# Dolby Vision config

from app.tv.dv_config import load_dv_config

cfg = load_dv_config(lut_or_path)

await cfg.upload(mgr.client)

self._lut_panel.set_status(f"DV config uploaded to {ip}")

else:

uploader = LUTUploader(client=mgr.client, pic_mode=pic_mode or "expert1")

if isinstance(lut_or_path, LUT1D):

await uploader.upload_1d(lut_or_path)

self._lut_panel.set_status(f"1D LUT uploaded to {ip}")

elif isinstance(lut_or_path, LUT3D):

await uploader.upload_3d(lut_or_path, target=target)

self._lut_panel.set_status(f"3D LUT ({target.value}) uploaded to {ip}")

except Exception as e:

self._lut_panel.set_status(f"Upload failed: {e}", error=True)

Wire the "📁 LUT Files" nav button in _build_ui to show self._lut_panel:

# In the nav_items loop in _build_ui, connect the LUT Files button:

# Replace the nav_items list with buttons that call self.set_content():

btn.clicked.connect(lambda checked, k=key: self._on_nav(k))

Add _on_nav method:

def _on_nav(self, key: str):

if key == "luts":

self.set_content(self._lut_panel)

# Other nav keys will route to their panels in subsequent sub-projects

☐  Step 2: Run all unit tests

pytest tests/unit/ -v

Expected: All tests pass.

☐  Step 3: Launch the app and verify

python -m app.main

Expected:

1. Window opens

2. Click "📁 LUT Files" in sidebar — LUT Files panel appears

3. Click "Browse…" — file picker opens, accepts .cube and .cal files

4. Select the identity .cube from tests/fixtures/ — shows "3D LUT — 17³"

5. "Upload LUT to TV" button becomes enabled

6. Without a TV connected: clicking Upload shows "No TV connected" in red

☐  Step 4: Final commit

git add app/ui/main_window.py

git commit -m "feat: wire LUTPanel into MainWindow sidebar navigation"

## Running Tests

# Unit tests (no TV needed)

pytest tests/unit/ -v

# Hardware tests (update TV_IP in test_lut_hardware.py first)

pytest tests/hardware/test_lut_hardware.py -v -m hardware

Section 4

# Sub-project 3: Measurement Workflow

> For agentic workers: REQUIRED SUB-SKILL: Use superpowers:subagent-driven-development (recommended) or superpowers:executing-plans to implement this plan task-by-task. Steps use checkbox (- [ ]) syntax for tracking.

Goal: Integrate X-Rite colorimeters/spectrophotometers via ArgyllCMS, control iTPG and PGenerator 1.6 pattern generators, automate patch measurement sequences (SDR/HDR10/DV), store CGATS results, and generate 1D/3D correction LUTs from measurements.

Architecture: app/meter/ handles meter device discovery and single-patch readings via ArgyllCMS subprocess calls. app/generator/ provides an abstract PatternGenerator interface with concrete implementations for LG iTPG (via bscpylgtv SSAP) and PGenerator 1.6 (via HTTP). app/measurement/ orchestrates the generator + meter loop, defines patch sequences, and stores results as CGATS and JSON. app/lut_gen/ produces 1D tone curves and 3D LUT arrays from measurement sessions. app/ui/measurement_panel.py wires everything into the Calibrate nav section.

Tech Stack: Python 3.11+, PyQt6, qasync, bscpylgtv, ArgyllCMS (external binary, called via subprocess), numpy, colour-science, pytest, pytest-asyncio, pytest-qt

Prerequisites: Sub-project 1 complete (ConnectionManager, LGTVSettings). Sub-project 2 complete (LUTUploader) for the optional "upload after measure" flow.

## ArgyllCMS Primer

ArgyllCMS must be installed separately. The app calls its command-line tools via subprocess:

spotread — single spectral/colorimetric reading from a connected meter

dispcal -? — list connected display measurement devices (device enumeration)

Key spotread flags used:

-d N — use device number N (from device list)

-O output.txt — write CGATS output to file (use - for stdout)

-e N — number of readings to average

-Y — high-res spectral (i1 Pro 2 only)

-H — high-res mode

CGATS output format (subset we parse):

CGATS.17

...

NUMBER_OF_FIELDS 4

BEGIN_DATA_FORMAT

SAMPLE_ID XYZ_X XYZ_Y XYZ_Z

END_DATA_FORMAT

NUMBER_OF_SETS 1

BEGIN_DATA

1 0.9642 1.0000 0.8249

END_DATA

Install path varies. Common locations:

macOS Homebrew: /opt/homebrew/bin/spotread

Manual install: /usr/local/bin/spotread

Use shutil.which("spotread") to locate at runtime.

## PGenerator 1.6 HTTP API

PGenerator by LightSpace runs on Raspberry Pi 4 and accepts HTTP commands to display test patches on its HDMI output. The API sends RGB values as query parameters. Verify the exact endpoint and port against your Pi installation — PGenerator 1.6 documentation or the LightSpace CMS guides in Resources/ have the definitive API.

Assumed API (verify against your Pi):

Base URL: http://<pi-ip>:8080

Display patch: GET /patch?r=<R>&g=<G>&b=<B> where R, G, B are 0–255 integers

Alternatively the path may be /measure or /setPatch — adapt if needed

Black patch: GET /patch?r=0&g=0&b=0

The PGeneratorClient in this plan uses this assumed API and includes a probe() method that calls a known endpoint to verify connectivity before starting a measurement session. If the API differs, only pgenerator.py needs updating — the rest of the codebase depends on the abstract PatternGenerator interface.

## iTPG (Internal Test Pattern Generator)

LG OLEDs have a built-in TPG accessible via SSAP. bscpylgtv exposes:

start_itpg() — enable the internal pattern generator

stop_itpg() — disable the internal pattern generator

set_itpg_patch_window(win_h, win_v, patch_h, patch_v) — set window and patch size

set_itpg_patch_color(r, g, b, ...) — set current patch color (10-bit values, 0–1023)

Verify exact signatures:

python -c "import bscpylgtv; help(bscpylgtv.WebOsClient.start_itpg)"

python -c "import bscpylgtv; help(bscpylgtv.WebOsClient.set_itpg_patch_color)"

iTPG operates in the TV's native bit depth. RGB values are 10-bit (0–1023). The iTPGGenerator converts 8-bit (0–255) patch values to 10-bit for SSAP calls.

## File Map

| File | Responsibility |
| --- | --- |
| `app/meter/device.py` | `MeterDevice` dataclass; ArgyllCMS device enumeration via `spotread -?` |
| `app/meter/argyll.py` | `ArgyllReader` — subprocess wrapper for `spotread`; returns `XYZReading` |
| `app/generator/base.py` | `PatternGenerator` abstract base class |
| `app/generator/itpg.py` | `iTPGGenerator` — iTPG via bscpylgtv SSAP |
| `app/generator/pgenerator.py` | `PGeneratorClient` — PGenerator 1.6 HTTP API |
| `app/measurement/patches.py` | `PatchSequence` and predefined sequences (SDR grayscale, primaries, HDR10) |
| `app/measurement/session.py` | `MeasurementSession` — orchestrates generator + meter loop |
| `app/measurement/store.py` | CGATS and JSON read/write for measurement results |
| `app/lut_gen/tone_curve.py` | 1D LUT generation from grayscale measurements |
| `app/lut_gen/gamut.py` | 3D LUT generation from full patch set |
| `app/ui/measurement_panel.py` | Calibrate nav panel — device selection, sequence picker, run controls |
| `tests/unit/test_meter.py` | ArgyllReader unit tests (mocked subprocess) |
| `tests/unit/test_generator_itpg.py` | iTPG unit tests (mocked bscpylgtv) |
| `tests/unit/test_generator_pgenerator.py` | PGenerator unit tests (mocked HTTP) |
| `tests/unit/test_patches.py` | PatchSequence unit tests |
| `tests/unit/test_session.py` | MeasurementSession unit tests (mocked generator + meter) |
| `tests/unit/test_store.py` | CGATS/JSON storage unit tests |
| `tests/unit/test_lut_gen.py` | LUT generation unit tests (synthetic measurements) |
| `tests/unit/test_measurement_panel.py` | UI panel tests |
| `tests/hardware/test_meter_hardware.py` | Real meter integration tests |
| `tests/hardware/test_generator_hardware.py` | Real pattern generator integration tests |
| `tests/fixtures/sample_cgats.txt` | Sample CGATS measurement output |

## Task 1: Measurement Data Models

Files:

Create: app/meter/__init__.py

Create: app/generator/__init__.py

Create: app/measurement/__init__.py

Create: app/lut_gen/__init__.py

Create: app/meter/device.py

☐  Step 1: Create package __init__.py files

touch app/meter/__init__.py app/generator/__init__.py \

app/measurement/__init__.py app/lut_gen/__init__.py

☐  Step 2: Write failing tests for MeterDevice

# tests/unit/test_meter.py  (first portion — device model)

from app.meter.device import MeterDevice, MeterType

def test_meter_device_has_name_and_type():

d = MeterDevice(index=0, name="i1 Display Pro", meter_type=MeterType.COLORIMETER)

assert d.index == 0

assert d.name == "i1 Display Pro"

assert d.meter_type == MeterType.COLORIMETER

def test_meter_type_enum():

assert MeterType.COLORIMETER != MeterType.SPECTROPHOTOMETER

☐  Step 3: Run to verify they fail

pytest tests/unit/test_meter.py::test_meter_device_has_name_and_type \

tests/unit/test_meter.py::test_meter_type_enum -v

Expected: ImportError

☐  Step 4: Implement MeterDevice

# app/meter/device.py

from __future__ import annotations

from dataclasses import dataclass

from enum import Enum

class MeterType(Enum):

COLORIMETER = "colorimeter"

SPECTROPHOTOMETER = "spectrophotometer"

UNKNOWN = "unknown"

@dataclass

class MeterDevice:

index: int

name: str

meter_type: MeterType = MeterType.UNKNOWN

@dataclass

class XYZReading:

"""A single colorimetric reading in CIE XYZ."""

X: float

Y: float

Z: float

@property

def xyY(self) -> tuple[float, float, float]:

total = self.X + self.Y + self.Z

if total == 0:

return (0.0, 0.0, 0.0)

return (self.X / total, self.Y / total, self.Y)

☐  Step 5: Run tests to verify they pass

pytest tests/unit/test_meter.py::test_meter_device_has_name_and_type \

tests/unit/test_meter.py::test_meter_type_enum -v

Expected: 2 tests pass.

☐  Step 6: Commit

git add app/meter/ app/generator/ app/measurement/ app/lut_gen/ tests/unit/test_meter.py

git commit -m "feat: measurement data models — MeterDevice, MeterType, XYZReading"

## Task 2: ArgyllCMS Subprocess Wrapper

Files:

Create: app/meter/argyll.py

Create: tests/fixtures/sample_cgats.txt

Extend: tests/unit/test_meter.py

☐  Step 1: Create CGATS fixture

Create tests/fixtures/sample_cgats.txt:

CGATS.17

ORIGINATOR "spotread"

CREATED "Wed Apr 23 2026"

DESCRIPTOR "Spot reading"

DEVICE_CLASS "DISPLAY"

NUMBER_OF_FIELDS 4

BEGIN_DATA_FORMAT

SAMPLE_ID XYZ_X XYZ_Y XYZ_Z

END_DATA_FORMAT

NUMBER_OF_SETS 1

BEGIN_DATA

1 95.047 100.000 108.883

END_DATA

☐  Step 2: Write failing tests for ArgyllReader

Append to tests/unit/test_meter.py:

import asyncio

from unittest.mock import patch, MagicMock, AsyncMock

from pathlib import Path

from app.meter.argyll import ArgyllReader, ArgyllNotFoundError, list_argyll_devices

from app.meter.device import MeterDevice, MeterType, XYZReading

FIXTURES = Path(__file__).parent.parent / "fixtures"

SPOTREAD_LIST_OUTPUT = b"""

spotread: Found 2 display measurement systems

0 = 'X-Rite i1 Display Pro'

1 = 'X-Rite i1 Pro 2'

"""

def test_list_argyll_devices_parses_output():

with patch("app.meter.argyll.subprocess.run") as mock_run:

mock_run.return_value = MagicMock(stdout=SPOTREAD_LIST_OUTPUT, returncode=1)

devices = list_argyll_devices()

assert len(devices) == 2

assert devices[0].index == 0

assert "i1 Display Pro" in devices[0].name

assert devices[1].index == 1

assert "i1 Pro 2" in devices[1].name

def test_list_argyll_devices_no_spotread_raises():

with patch("app.meter.argyll.shutil.which", return_value=None):

with pytest.raises(ArgyllNotFoundError):

list_argyll_devices()

async def test_take_reading_parses_cgats():

reader = ArgyllReader(device_index=0)

cgats_text = (FIXTURES / "sample_cgats.txt").read_text()

with patch("app.meter.argyll.subprocess.run") as mock_run:

mock_run.return_value = MagicMock(stdout=cgats_text.encode(), returncode=0)

reading = await reader.take_reading()

assert isinstance(reading, XYZReading)

assert abs(reading.X - 95.047) < 0.01

assert abs(reading.Y - 100.000) < 0.01

assert abs(reading.Z - 108.883) < 0.01

async def test_take_reading_subprocess_failure_raises():

reader = ArgyllReader(device_index=0)

with patch("app.meter.argyll.subprocess.run") as mock_run:

mock_run.return_value = MagicMock(stdout=b"", returncode=2)

with pytest.raises(RuntimeError, match="spotread failed"):

await reader.take_reading()

Don't forget import pytest at top of test file.

☐  Step 3: Run to verify they fail

pytest tests/unit/test_meter.py -v

Expected: ImportError for app.meter.argyll

☐  Step 4: Implement ArgyllReader

# app/meter/argyll.py

from __future__ import annotations

import asyncio

import re

import shutil

import subprocess

from typing import Optional

from app.meter.device import MeterDevice, MeterType, XYZReading

class ArgyllNotFoundError(RuntimeError):

pass

def _spotread_path() -> str:

path = shutil.which("spotread")

if path is None:

raise ArgyllNotFoundError(

"spotread not found. Install ArgyllCMS and ensure it is in PATH.\n"

"macOS: brew install argyllcms"

)

return path

def list_argyll_devices() -> list[MeterDevice]:

"""Return connected measurement devices by running spotread with no args."""

spotread = _spotread_path()

result = subprocess.run(

[spotread, "-?"],

capture_output=True,

timeout=10,

)

# spotread -? exits with code 1 but prints the device list to stdout

output = result.stdout.decode(errors="ignore")

devices: list[MeterDevice] = []

for line in output.splitlines():

m = re.match(r"\s*(\d+)\s*=\s*'(.+)'", line)

if m:

idx = int(m.group(1))

name = m.group(2).strip()

meter_type = MeterType.SPECTROPHOTOMETER if "Pro 2" in name else MeterType.COLORIMETER

devices.append(MeterDevice(index=idx, name=name, meter_type=meter_type))

return devices

class ArgyllReader:

def __init__(self, device_index: int = 0, avg_count: int = 3):

self.device_index = device_index

self.avg_count = avg_count

async def take_reading(self) -> XYZReading:

"""Take a single averaged reading from the meter. Returns XYZ in cd/m² scale."""

spotread = _spotread_path()

cmd = [

spotread,

"-d", str(self.device_index),

"-e", str(self.avg_count),

"-O", "-",   # output to stdout

]

loop = asyncio.get_event_loop()

result = await loop.run_in_executor(

None,

lambda: subprocess.run(cmd, capture_output=True, timeout=30),

)

if result.returncode != 0:

raise RuntimeError(

f"spotread failed (code {result.returncode}): "

f"{result.stderr.decode(errors='ignore')}"

)

return _parse_cgats_xyz(result.stdout.decode(errors="ignore"))

def _parse_cgats_xyz(text: str) -> XYZReading:

"""Extract the first XYZ reading from a CGATS text block."""

in_data = False

for line in text.splitlines():

line = line.strip()

if line == "BEGIN_DATA":

in_data = True

continue

if line == "END_DATA":

break

if in_data and line:

parts = line.split()

if len(parts) >= 4:

# SAMPLE_ID XYZ_X XYZ_Y XYZ_Z

return XYZReading(X=float(parts[1]), Y=float(parts[2]), Z=float(parts[3]))

raise ValueError("No XYZ data found in CGATS output")

☐  Step 5: Run tests to verify they pass

pytest tests/unit/test_meter.py -v

Expected: All tests pass.

☐  Step 6: Commit

git add app/meter/argyll.py tests/fixtures/sample_cgats.txt tests/unit/test_meter.py

git commit -m "feat: ArgyllReader wraps spotread for single-patch XYZ measurement"

## Task 3: Pattern Generator Abstraction + iTPG

Files:

Create: app/generator/base.py

Create: app/generator/itpg.py

Create: tests/unit/test_generator_itpg.py

☐  Step 1: Write failing tests

# tests/unit/test_generator_itpg.py

import pytest

from unittest.mock import AsyncMock

from app.generator.itpg import iTPGGenerator

@pytest.fixture

def mock_client():

client = AsyncMock()

client.start_itpg = AsyncMock(return_value={"returnValue": True})

client.stop_itpg = AsyncMock(return_value={"returnValue": True})

client.set_itpg_patch_color = AsyncMock(return_value={"returnValue": True})

client.set_itpg_patch_window = AsyncMock(return_value={"returnValue": True})

return client

@pytest.fixture

def gen(mock_client):

return iTPGGenerator(client=mock_client)

async def test_start_calls_bscpylgtv(gen, mock_client):

await gen.start()

mock_client.start_itpg.assert_called_once()

async def test_stop_calls_bscpylgtv(gen, mock_client):

await gen.stop()

mock_client.stop_itpg.assert_called_once()

async def test_set_patch_converts_8bit_to_10bit(gen, mock_client):

await gen.set_patch(r=128, g=64, b=255)

mock_client.set_itpg_patch_color.assert_called_once()

call_args = mock_client.set_itpg_patch_color.call_args

# 128/255 * 1023 ≈ 513

r_10bit = call_args[0][0] if call_args[0] else call_args[1].get("r")

assert 510 <= r_10bit <= 516

async def test_set_patch_black(gen, mock_client):

await gen.set_patch(r=0, g=0, b=0)

mock_client.set_itpg_patch_color.assert_called_once()

async def test_set_patch_white(gen, mock_client):

await gen.set_patch(r=255, g=255, b=255)

call_args = mock_client.set_itpg_patch_color.call_args

r_10bit = call_args[0][0] if call_args[0] else call_args[1].get("r")

assert r_10bit == 1023

☐  Step 2: Run to verify they fail

pytest tests/unit/test_generator_itpg.py -v

Expected: ImportError

☐  Step 3: Implement abstract base and iTPG

# app/generator/base.py

from abc import ABC, abstractmethod

class PatternGenerator(ABC):

"""Abstract interface for test pattern generators."""

@abstractmethod

async def start(self) -> None:

"""Activate the pattern generator."""

@abstractmethod

async def stop(self) -> None:

"""Deactivate the pattern generator and restore normal TV operation."""

@abstractmethod

async def set_patch(self, r: int, g: int, b: int) -> None:

"""Display a full-screen patch with the given 8-bit RGB values (0–255)."""

async def __aenter__(self):

await self.start()

return self

async def __aexit__(self, *_):

await self.stop()

# app/generator/itpg.py

from __future__ import annotations

from bscpylgtv import WebOsClient

from app.generator.base import PatternGenerator

_10BIT_MAX = 1023

_8BIT_MAX = 255

def _to_10bit(v: int) -> int:

return round(v / _8BIT_MAX * _10BIT_MAX)

class iTPGGenerator(PatternGenerator):

"""LG internal Test Pattern Generator via bscpylgtv SSAP."""

def __init__(self, client: WebOsClient,

win_h: int = 100, win_v: int = 100,

patch_h: int = 50, patch_v: int = 50):

self._client = client

self._win_h = win_h

self._win_v = win_v

self._patch_h = patch_h

self._patch_v = patch_v

async def start(self) -> None:

await self._client.start_itpg()

await self._client.set_itpg_patch_window(

self._win_h, self._win_v, self._patch_h, self._patch_v

)

async def stop(self) -> None:

await self._client.stop_itpg()

async def set_patch(self, r: int, g: int, b: int) -> None:

await self._client.set_itpg_patch_color(

_to_10bit(r), _to_10bit(g), _to_10bit(b)

)

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_generator_itpg.py -v

Expected: 5 tests pass.

☐  Step 5: Commit

git add app/generator/base.py app/generator/itpg.py tests/unit/test_generator_itpg.py

git commit -m "feat: PatternGenerator base class and iTPGGenerator via bscpylgtv"

## Task 4: PGenerator 1.6 HTTP Client

Files:

Create: app/generator/pgenerator.py

Create: tests/unit/test_generator_pgenerator.py

☐  Step 1: Write failing tests

# tests/unit/test_generator_pgenerator.py

import pytest

from unittest.mock import patch, AsyncMock, MagicMock

from app.generator.pgenerator import PGeneratorClient

@pytest.fixture

def gen():

return PGeneratorClient(host="192.168.1.200", port=8080)

async def test_set_patch_sends_http_get(gen):

with patch("app.generator.pgenerator.aiohttp.ClientSession") as mock_session_cls:

mock_session = AsyncMock()

mock_session_cls.return_value.__aenter__.return_value = mock_session

mock_response = AsyncMock()

mock_response.status = 200

mock_session.get.return_value.__aenter__.return_value = mock_response

await gen.set_patch(r=128, g=64, b=255)

mock_session.get.assert_called_once()

call_url = mock_session.get.call_args[0][0]

assert "r=128" in call_url or "128" in call_url

assert "g=64" in call_url or "64" in call_url

assert "b=255" in call_url or "255" in call_url

async def test_probe_returns_true_on_200(gen):

with patch("app.generator.pgenerator.aiohttp.ClientSession") as mock_session_cls:

mock_session = AsyncMock()

mock_session_cls.return_value.__aenter__.return_value = mock_session

mock_response = AsyncMock()

mock_response.status = 200

mock_session.get.return_value.__aenter__.return_value = mock_response

result = await gen.probe()

assert result is True

async def test_probe_returns_false_on_connection_error(gen):

import aiohttp

with patch("app.generator.pgenerator.aiohttp.ClientSession") as mock_session_cls:

mock_session = AsyncMock()

mock_session_cls.return_value.__aenter__.return_value = mock_session

mock_session.get.side_effect = aiohttp.ClientError()

result = await gen.probe()

assert result is False

async def test_start_and_stop_are_noops(gen):

await gen.start()   # should not raise

await gen.stop()    # should not raise

☐  Step 2: Run to verify they fail

pytest tests/unit/test_generator_pgenerator.py -v

Expected: ImportError

☐  Step 3: Add aiohttp to requirements

Add to requirements.txt:

aiohttp>=3.9.0

Install:

pip install aiohttp

☐  Step 4: Implement PGeneratorClient

# app/generator/pgenerator.py

from __future__ import annotations

import aiohttp

from app.generator.base import PatternGenerator

# Adjust _PATCH_PATH if your PGenerator installation uses a different endpoint.

# Check PGenerator 1.6 docs or verify by opening http://<pi-ip>:<port>/ in a browser.

_PATCH_PATH = "/patch"

class PGeneratorClient(PatternGenerator):

"""PGenerator 1.6 HTTP API client for Raspberry Pi 4."""

def __init__(self, host: str, port: int = 8080):

self.host = host

self.port = port

self._base_url = f"http://{host}:{port}"

async def start(self) -> None:

pass  # PGenerator is always running; no explicit start needed

async def stop(self) -> None:

await self.set_patch(0, 0, 0)  # Show black on stop

async def set_patch(self, r: int, g: int, b: int) -> None:

"""Send HTTP GET to display an 8-bit RGB patch."""

url = f"{self._base_url}{_PATCH_PATH}?r={r}&g={g}&b={b}"

async with aiohttp.ClientSession() as session:

async with session.get(url, timeout=aiohttp.ClientTimeout(total=5)) as resp:

if resp.status >= 400:

text = await resp.text()

raise RuntimeError(

f"PGenerator HTTP error {resp.status} for patch ({r},{g},{b}): {text}"

)

async def probe(self) -> bool:

"""Return True if PGenerator is reachable."""

try:

url = f"{self._base_url}{_PATCH_PATH}?r=0&g=0&b=0"

async with aiohttp.ClientSession() as session:

async with session.get(url, timeout=aiohttp.ClientTimeout(total=3)) as resp:

return resp.status < 400

except aiohttp.ClientError:

return False

☐  Step 5: Run tests to verify they pass

pytest tests/unit/test_generator_pgenerator.py -v

Expected: 4 tests pass.

☐  Step 6: Commit

git add app/generator/pgenerator.py tests/unit/test_generator_pgenerator.py requirements.txt

git commit -m "feat: PGeneratorClient — PGenerator 1.6 HTTP API for Raspberry Pi 4"

## Task 5: Patch Sequences

Files:

Create: app/measurement/patches.py

Create: tests/unit/test_patches.py

☐  Step 1: Write failing tests

# tests/unit/test_patches.py

from app.measurement.patches import (

Patch, PatchSequence,

SDR_GRAYSCALE_21, SDR_PRIMARIES, SDR_SECONDARIES, HDR10_GRAYSCALE,

build_sdr_full, build_hdr10_full,

)

def test_patch_has_rgb():

p = Patch(r=128, g=0, b=0, label="50% Red")

assert p.r == 128 and p.g == 0 and p.b == 0

assert p.label == "50% Red"

def test_sdr_grayscale_21_has_21_patches():

assert len(SDR_GRAYSCALE_21) == 21

# First patch is black (0,0,0)

assert SDR_GRAYSCALE_21[0].r == 0

assert SDR_GRAYSCALE_21[0].g == 0

assert SDR_GRAYSCALE_21[0].b == 0

# Last patch is white (255,255,255)

assert SDR_GRAYSCALE_21[-1].r == 255

def test_sdr_primaries_has_6_patches():

assert len(SDR_PRIMARIES) == 6  # R, G, B, C, M, Y at 100%

def test_build_sdr_full_concatenates_sequences():

seq = build_sdr_full()

assert isinstance(seq, PatchSequence)

assert len(seq.patches) >= 21 + 6 + 3  # grayscale + primaries + secondaries minimum

def test_hdr10_grayscale_patches_labeled():

for p in HDR10_GRAYSCALE:

assert p.label != ""

assert "nit" in p.label.lower() or "%" in p.label

☐  Step 2: Run to verify they fail

pytest tests/unit/test_patches.py -v

Expected: ImportError

☐  Step 3: Implement patch sequences

# app/measurement/patches.py

from __future__ import annotations

from dataclasses import dataclass, field

from typing import List

@dataclass

class Patch:

r: int   # 0–255

g: int

b: int

label: str = ""

is_hdr: bool = False

nits: float = 0.0  # target luminance for HDR patches

@dataclass

class PatchSequence:

name: str

patches: List[Patch] = field(default_factory=list)

def __len__(self):

return len(self.patches)

def __iter__(self):

return iter(self.patches)

def _gray(pct: float) -> Patch:

"""Create a grayscale patch from 0–100% stimulus."""

v = round(pct / 100 * 255)

return Patch(r=v, g=v, b=v, label=f"{pct:.0f}%")

def _hdr_gray(nits: float, peak_nits: float = 1000.0) -> Patch:

"""Approximate 8-bit code for an HDR10 luminance target (for iTPG SDR-driving)."""

pq_linear = nits / peak_nits

# Use PQ EOTF inverse to get 10-bit code, then scale to 8-bit

# Simplified: use linear mapping for the patch driver (actual HDR on TV side)

v = round(pq_linear ** (1 / 2.4) * 255)

v = max(0, min(255, v))

return Patch(r=v, g=v, b=v, label=f"{nits:.0f} nits", is_hdr=True, nits=nits)

# SDR 21-point grayscale: 0%, 5%, 10%, ..., 100%

SDR_GRAYSCALE_21: list[Patch] = [_gray(i * 5) for i in range(21)]

# Primary and secondary colors at 100% stimulus

SDR_PRIMARIES: list[Patch] = [

Patch(255, 0,   0,   "Red 100%"),

Patch(0,   255, 0,   "Green 100%"),

Patch(0,   0,   255, "Blue 100%"),

Patch(0,   255, 255, "Cyan 100%"),

Patch(255, 0,   255, "Magenta 100%"),

Patch(255, 255, 0,   "Yellow 100%"),

]

SDR_SECONDARIES: list[Patch] = [

Patch(128, 0,   0,   "Red 50%"),

Patch(0,   128, 0,   "Green 50%"),

Patch(0,   0,   128, "Blue 50%"),

]

# HDR10 grayscale: key luminance targets

HDR10_GRAYSCALE: list[Patch] = [

_hdr_gray(0),

_hdr_gray(1),

_hdr_gray(2),

_hdr_gray(5),

_hdr_gray(10),

_hdr_gray(20),

_hdr_gray(50),

_hdr_gray(100),

_hdr_gray(200),

_hdr_gray(400),

_hdr_gray(600),

_hdr_gray(800),

_hdr_gray(1000),

]

def build_sdr_full() -> PatchSequence:

"""Full SDR measurement sequence: grayscale + primaries + secondaries."""

patches = list(SDR_GRAYSCALE_21) + list(SDR_PRIMARIES) + list(SDR_SECONDARIES)

return PatchSequence(name="SDR Full", patches=patches)

def build_hdr10_full() -> PatchSequence:

"""HDR10 sequence: grayscale + primaries."""

patches = list(HDR10_GRAYSCALE) + list(SDR_PRIMARIES)

return PatchSequence(name="HDR10 Full", patches=patches)

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_patches.py -v

Expected: 5 tests pass.

☐  Step 5: Commit

git add app/measurement/patches.py tests/unit/test_patches.py

git commit -m "feat: patch sequences — SDR 21pt grayscale, primaries, HDR10 grayscale"

## Task 6: Measurement Session

Files:

Create: app/measurement/session.py

Create: tests/unit/test_session.py

☐  Step 1: Write failing tests

# tests/unit/test_session.py

import pytest

from unittest.mock import AsyncMock, MagicMock

from app.measurement.session import MeasurementSession, PatchResult

from app.measurement.patches import Patch, PatchSequence

from app.meter.device import XYZReading

@pytest.fixture

def mock_generator():

gen = AsyncMock()

gen.start = AsyncMock()

gen.stop = AsyncMock()

gen.set_patch = AsyncMock()

return gen

@pytest.fixture

def mock_reader():

reader = AsyncMock()

reader.take_reading = AsyncMock(return_value=XYZReading(X=95.0, Y=100.0, Z=108.9))

return reader

@pytest.fixture

def two_patch_seq():

return PatchSequence("Test", [

Patch(0, 0, 0, "Black"),

Patch(255, 255, 255, "White"),

])

async def test_session_calls_generator_for_each_patch(

mock_generator, mock_reader, two_patch_seq

):

session = MeasurementSession(

generator=mock_generator, reader=mock_reader, sequence=two_patch_seq

)

results = await session.run()

assert mock_generator.set_patch.call_count == 2

async def test_session_returns_patch_results(

mock_generator, mock_reader, two_patch_seq

):

session = MeasurementSession(

generator=mock_generator, reader=mock_reader, sequence=two_patch_seq

)

results = await session.run()

assert len(results) == 2

assert isinstance(results[0], PatchResult)

assert results[0].patch.label == "Black"

assert results[0].reading.Y == 100.0

async def test_session_calls_start_and_stop(

mock_generator, mock_reader, two_patch_seq

):

session = MeasurementSession(

generator=mock_generator, reader=mock_reader, sequence=two_patch_seq

)

await session.run()

mock_generator.start.assert_called_once()

mock_generator.stop.assert_called_once()

async def test_session_stop_called_even_on_error(mock_generator, mock_reader, two_patch_seq):

mock_reader.take_reading = AsyncMock(side_effect=RuntimeError("meter disconnected"))

session = MeasurementSession(

generator=mock_generator, reader=mock_reader, sequence=two_patch_seq

)

with pytest.raises(RuntimeError, match="meter disconnected"):

await session.run()

mock_generator.stop.assert_called_once()

async def test_session_progress_callback_called(mock_generator, mock_reader, two_patch_seq):

progress_calls = []

session = MeasurementSession(

generator=mock_generator, reader=mock_reader, sequence=two_patch_seq,

on_progress=lambda i, total, result: progress_calls.append((i, total))

)

await session.run()

assert progress_calls == [(1, 2), (2, 2)]

☐  Step 2: Run to verify they fail

pytest tests/unit/test_session.py -v

Expected: ImportError

☐  Step 3: Implement MeasurementSession

# app/measurement/session.py

from __future__ import annotations

import asyncio

from dataclasses import dataclass

from typing import Callable, List, Optional

from app.generator.base import PatternGenerator

from app.meter.argyll import ArgyllReader

from app.meter.device import XYZReading

from app.measurement.patches import Patch, PatchSequence

_SETTLE_SECONDS = 0.5  # Pause after setting patch before reading — adjust for your setup

@dataclass

class PatchResult:

patch: Patch

reading: XYZReading

class MeasurementSession:

def __init__(

self,

generator: PatternGenerator,

reader: ArgyllReader,

sequence: PatchSequence,

settle_time: float = _SETTLE_SECONDS,

on_progress: Optional[Callable[[int, int, PatchResult], None]] = None,

):

self._generator = generator

self._reader = reader

self._sequence = sequence

self._settle_time = settle_time

self._on_progress = on_progress

async def run(self) -> List[PatchResult]:

results: List[PatchResult] = []

total = len(self._sequence)

await self._generator.start()

try:

for i, patch in enumerate(self._sequence, start=1):

await self._generator.set_patch(patch.r, patch.g, patch.b)

await asyncio.sleep(self._settle_time)

reading = await self._reader.take_reading()

result = PatchResult(patch=patch, reading=reading)

results.append(result)

if self._on_progress:

self._on_progress(i, total, result)

finally:

await self._generator.stop()

return results

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_session.py -v

Expected: 5 tests pass.

☐  Step 5: Commit

git add app/measurement/session.py tests/unit/test_session.py

git commit -m "feat: MeasurementSession orchestrates generator+meter loop with progress callback"

## Task 7: CGATS + JSON Storage

Files:

Create: app/measurement/store.py

Create: tests/unit/test_store.py

☐  Step 1: Write failing tests

# tests/unit/test_store.py

import json

import pytest

from pathlib import Path

from app.measurement.store import save_cgats, load_cgats, save_json, load_json

from app.measurement.session import PatchResult

from app.measurement.patches import Patch

from app.meter.device import XYZReading

@pytest.fixture

def sample_results():

return [

PatchResult(patch=Patch(0, 0, 0, "Black"), reading=XYZReading(0.0, 0.0, 0.0)),

PatchResult(patch=Patch(255, 255, 255, "White"), reading=XYZReading(95.0, 100.0, 108.9)),

]

def test_save_and_load_cgats(tmp_path, sample_results):

path = tmp_path / "results.cgats"

save_cgats(sample_results, path)

assert path.exists()

loaded = load_cgats(path)

assert len(loaded) == 2

assert abs(loaded[1].reading.Y - 100.0) < 0.01

def test_cgats_format_has_required_headers(tmp_path, sample_results):

path = tmp_path / "results.cgats"

save_cgats(sample_results, path)

text = path.read_text()

assert "CGATS" in text

assert "XYZ_X" in text

assert "BEGIN_DATA" in text

assert "END_DATA" in text

def test_save_and_load_json(tmp_path, sample_results):

path = tmp_path / "results.json"

save_json(sample_results, path)

assert path.exists()

loaded = load_json(path)

assert len(loaded) == 2

assert loaded[0].patch.label == "Black"

assert abs(loaded[1].reading.X - 95.0) < 0.01

☐  Step 2: Run to verify they fail

pytest tests/unit/test_store.py -v

Expected: ImportError

☐  Step 3: Implement storage module

# app/measurement/store.py

from __future__ import annotations

import json

from pathlib import Path

from typing import List

from app.measurement.session import PatchResult

from app.measurement.patches import Patch

from app.meter.device import XYZReading

def save_cgats(results: List[PatchResult], path: Path) -> None:

path = Path(path)

lines = [

"CGATS.17",

'ORIGINATOR "lg-oled-cal"',

"NUMBER_OF_FIELDS 7",

"BEGIN_DATA_FORMAT",

"SAMPLE_ID SAMPLE_NAME RGB_R RGB_G RGB_B XYZ_X XYZ_Y XYZ_Z",

"END_DATA_FORMAT",

f"NUMBER_OF_SETS {len(results)}",

"BEGIN_DATA",

]

for i, r in enumerate(results, start=1):

lines.append(

f"{i} {r.patch.label or str(i)} "

f"{r.patch.r} {r.patch.g} {r.patch.b} "

f"{r.reading.X:.6f} {r.reading.Y:.6f} {r.reading.Z:.6f}"

)

lines.append("END_DATA")

path.write_text("\n".join(lines))

def load_cgats(path: Path) -> List[PatchResult]:

path = Path(path)

results: List[PatchResult] = []

in_data = False

for line in path.read_text().splitlines():

line = line.strip()

if line == "BEGIN_DATA":

in_data = True

continue

if line == "END_DATA":

break

if in_data and line:

parts = line.split()

if len(parts) >= 8:

label = parts[1]

r, g, b = int(parts[2]), int(parts[3]), int(parts[4])

X, Y, Z = float(parts[5]), float(parts[6]), float(parts[7])

results.append(PatchResult(

patch=Patch(r=r, g=g, b=b, label=label),

reading=XYZReading(X=X, Y=Y, Z=Z),

))

return results

def save_json(results: List[PatchResult], path: Path) -> None:

data = [

{

"patch": {"r": r.patch.r, "g": r.patch.g, "b": r.patch.b,

"label": r.patch.label},

"xyz": {"X": r.reading.X, "Y": r.reading.Y, "Z": r.reading.Z},

}

for r in results

]

Path(path).write_text(json.dumps(data, indent=2))

def load_json(path: Path) -> List[PatchResult]:

data = json.loads(Path(path).read_text())

return [

PatchResult(

patch=Patch(r=d["patch"]["r"], g=d["patch"]["g"], b=d["patch"]["b"],

label=d["patch"]["label"]),

reading=XYZReading(X=d["xyz"]["X"], Y=d["xyz"]["Y"], Z=d["xyz"]["Z"]),

)

for d in data

]

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_store.py -v

Expected: 3 tests pass.

☐  Step 5: Commit

git add app/measurement/store.py tests/unit/test_store.py

git commit -m "feat: CGATS and JSON storage for measurement results"

## Task 8: LUT Generation from Measurements

Files:

Create: app/lut_gen/tone_curve.py

Create: app/lut_gen/gamut.py

Create: tests/unit/test_lut_gen.py

Add to requirements.txt: colour-science>=0.4.4

☐  Step 1: Add colour-science dependency

Append to requirements.txt:

colour-science>=0.4.4

numpy>=1.26.0

Install:

pip install colour-science numpy

☐  Step 2: Write failing tests

# tests/unit/test_lut_gen.py

import numpy as np

import pytest

from app.measurement.session import PatchResult

from app.measurement.patches import Patch, SDR_GRAYSCALE_21

from app.meter.device import XYZReading

from app.lut_gen.tone_curve import generate_1d_lut_from_grayscale

from app.lut_gen.gamut import generate_3d_lut_from_measurements

from app.tv.lut import LUT1D, LUT3D

def _make_linear_grayscale_results() -> list[PatchResult]:

"""Synthetic measurements: Y rises linearly with code value."""

results = []

for patch in SDR_GRAYSCALE_21:

stim = patch.r / 255

Y = stim * 100.0  # linear response

results.append(PatchResult(patch=patch, reading=XYZReading(X=Y * 0.95, Y=Y, Z=Y * 1.09)))

return results

def test_generate_1d_lut_returns_lut1d():

results = _make_linear_grayscale_results()

lut = generate_1d_lut_from_grayscale(results, target_gamma=2.4)

assert isinstance(lut, LUT1D)

assert lut.data.shape == (3, 1024)

def test_generate_1d_lut_identity_for_perfect_display():

"""A display with perfect gamma 2.4 should produce an identity LUT."""

results = []

for patch in SDR_GRAYSCALE_21:

stim = patch.r / 255

# Perfect 2.4 gamma response

Y = (stim ** 2.4) * 100.0 if stim > 0 else 0.0

results.append(PatchResult(patch=patch, reading=XYZReading(X=Y * 0.95, Y=Y, Z=Y * 1.09)))

lut = generate_1d_lut_from_grayscale(results, target_gamma=2.4)

# Near-identity: midpoint output should be close to 0.5

midpoint = lut.data[0, 512]

assert 0.4 < midpoint < 0.6

def test_generate_3d_lut_returns_lut3d():

from app.measurement.patches import build_sdr_full

full_seq = build_sdr_full()

# Synthetic results: identity (output = input)

results = []

for patch in full_seq:

X = patch.r / 255 * 95.047

Y = patch.g / 255 * 100.0

Z = patch.b / 255 * 108.883

results.append(PatchResult(patch=patch, reading=XYZReading(X=X, Y=Y, Z=Z)))

lut = generate_3d_lut_from_measurements(results, lut_size=17)

assert isinstance(lut, LUT3D)

assert lut.size == 17

assert lut.data.shape == (17, 17, 17, 3)

☐  Step 3: Run to verify they fail

pytest tests/unit/test_lut_gen.py -v

Expected: ImportError

☐  Step 4: Implement tone curve generation

# app/lut_gen/tone_curve.py

from __future__ import annotations

import numpy as np

from typing import List

from app.measurement.session import PatchResult

from app.tv.lut import LUT1D

def generate_1d_lut_from_grayscale(

results: List[PatchResult],

target_gamma: float = 2.4,

lut_size: int = 1024,

) -> LUT1D:

"""

Generate a 1D correction LUT from grayscale measurements.

Takes measured Y values and computes the correction curve needed to hit

target_gamma (e.g., 2.4 for BT.1886-approximate) at 100 cd/m² white.

The correction curve at each input code:

corrected_output = (target_Y / measured_Y) ^ gamma_correction

"""

# Extract stimulus (normalized 0–1) and measured Y (normalized to white)

stimulus = np.array([r.patch.r / 255.0 for r in results])

y_vals = np.array([r.reading.Y for r in results])

# Normalize Y to white point

white_Y = max(y_vals)

if white_Y == 0:

raise ValueError("White point Y is zero — check measurement data")

y_norm = y_vals / white_Y

# Target: ideal gamma 2.4 response

eps = 1e-6

y_target = np.where(stimulus > 0, stimulus ** target_gamma, 0.0)

# Compute per-sample correction ratio (clamped to avoid div/0)

y_norm_safe = np.where(y_norm > eps, y_norm, eps)

correction_ratio = np.where(y_target > eps, y_target / y_norm_safe, 1.0)

# Build 1024-point LUT by interpolating the correction curve

x_lut = np.linspace(0, 1, lut_size)

correction_interp = np.interp(x_lut, stimulus, correction_ratio)

# Apply correction to produce output values

lut_out = np.clip(x_lut * correction_interp, 0, 1).astype(np.float32)

# Same correction for R, G, B (1D LUT is neutral axis only)

data = np.stack([lut_out, lut_out, lut_out], axis=0)

return LUT1D(data=data)

☐  Step 5: Implement 3D LUT generation

# app/lut_gen/gamut.py

from __future__ import annotations

import numpy as np

from typing import List

from app.measurement.session import PatchResult

from app.tv.lut import LUT3D

try:

import colour

_COLOUR_AVAILABLE = True

except ImportError:

_COLOUR_AVAILABLE = False

def generate_3d_lut_from_measurements(

results: List[PatchResult],

lut_size: int = 17,

target_colorspace: str = "ITU-R BT.709",

) -> LUT3D:

"""

Generate a 3D correction LUT from a full patch set.

Uses thin-plate spline interpolation (via scipy) to build a correction

volume from the measured vs. target RGB values. Requires at least

grayscale + primary + secondary patches (30+ measurements).

target_colorspace: "ITU-R BT.709" or "ITU-R BT.2020"

Note: This is a first-order approximation. For professional results,

use DisplayCAL's 3D LUT generation or LightSpace CMS with this

measurement data as input.

"""

from scipy.interpolate import RBFInterpolator

# Convert measured XYZ to the target color space RGB

if not _COLOUR_AVAILABLE:

raise ImportError("colour-science required: pip install colour-science")

cs = colour.RGB_COLOURSPACES[target_colorspace]

xyz_to_rgb_matrix = cs.matrix_XYZ_to_RGB

# Build known-points: input RGB (0–1) → correction offset

known_rgb_in = []

known_rgb_correction = []

white_Y = max(r.reading.Y for r in results)

if white_Y == 0:

raise ValueError("White point Y is zero")

for result in results:

stim_rgb = np.array([result.patch.r, result.patch.g, result.patch.b]) / 255.0

xyz = np.array([result.reading.X, result.reading.Y, result.reading.Z]) / white_Y * 100.0

# Convert measured XYZ to RGB in target color space

measured_rgb = np.clip(xyz_to_rgb_matrix @ xyz, 0, 1)

correction = stim_rgb - measured_rgb  # how far off is the display

known_rgb_in.append(stim_rgb)

known_rgb_correction.append(correction)

known_rgb_in = np.array(known_rgb_in)

known_rgb_correction = np.array(known_rgb_correction)

# Fit RBF interpolator

rbf = RBFInterpolator(known_rgb_in, known_rgb_correction, kernel="thin_plate_spline")

# Build LUT grid

grid_1d = np.linspace(0, 1, lut_size)

R, G, B = np.meshgrid(grid_1d, grid_1d, grid_1d, indexing="ij")

grid_pts = np.stack([R.ravel(), G.ravel(), B.ravel()], axis=1)

corrections = rbf(grid_pts)

corrected = np.clip(grid_pts + corrections, 0, 1)

data = corrected.reshape(lut_size, lut_size, lut_size, 3).astype(np.float32)

return LUT3D(data=data, size=lut_size)

☐  Step 6: Add scipy to requirements.txt

Append:

scipy>=1.13.0

Install:

pip install scipy

☐  Step 7: Run tests to verify they pass

pytest tests/unit/test_lut_gen.py -v

Expected: 3 tests pass (the 3D LUT test may be slow — ~2–5 seconds for RBF fit).

☐  Step 8: Commit

git add app/lut_gen/tone_curve.py app/lut_gen/gamut.py tests/unit/test_lut_gen.py requirements.txt

git commit -m "feat: 1D tone curve and 3D LUT generation from measurement sessions"

## Task 9: Measurement Panel UI

Files:

Create: app/ui/measurement_panel.py

Create: tests/unit/test_measurement_panel.py

☐  Step 1: Write failing tests

# tests/unit/test_measurement_panel.py

import pytest

from unittest.mock import AsyncMock

from PyQt6.QtWidgets import QApplication, QPushButton, QComboBox, QLabel

from app.ui.measurement_panel import MeasurementPanel

from app.meter.device import MeterDevice, MeterType

@pytest.fixture(scope="module")

def app():

return QApplication.instance() or QApplication([])

@pytest.fixture

def panel(app):

p = MeasurementPanel(on_run=AsyncMock(), on_upload_lut=AsyncMock())

yield p

p.close()

def test_panel_creates(panel):

assert panel is not None

def test_panel_has_run_button(panel):

buttons = [b.text() for b in panel.findChildren(QPushButton)]

assert any("measure" in t.lower() or "run" in t.lower() or "start" in t.lower() for t in buttons)

def test_panel_has_sequence_selector(panel):

combos = panel.findChildren(QComboBox)

assert len(combos) >= 1

all_items = []

for c in combos:

for i in range(c.count()):

all_items.append(c.itemText(i))

assert any("SDR" in item or "sdr" in item.lower() for item in all_items)

def test_populate_meters_updates_combo(panel):

devices = [

MeterDevice(0, "i1 Display Pro", MeterType.COLORIMETER),

MeterDevice(1, "i1 Pro 2", MeterType.SPECTROPHOTOMETER),

]

panel.populate_meters(devices)

combos = panel.findChildren(QComboBox)

meter_items = []

for c in combos:

for i in range(c.count()):

meter_items.append(c.itemText(i))

assert any("i1" in item for item in meter_items)

☐  Step 2: Run to verify they fail

pytest tests/unit/test_measurement_panel.py -v

Expected: ImportError

☐  Step 3: Implement MeasurementPanel

# app/ui/measurement_panel.py

from __future__ import annotations

import asyncio

from typing import Callable, List, Optional

from PyQt6.QtWidgets import (

QWidget, QVBoxLayout, QHBoxLayout, QLabel, QPushButton,

QComboBox, QGroupBox, QGridLayout, QProgressBar, QLineEdit,

QTextEdit

)

from PyQt6.QtCore import Qt

from app.meter.device import MeterDevice

_STYLE_LABEL = "color: #ccc; font-size: 12px;"

_STYLE_BTN_RUN = (

"background: #4fc3f7; color: #000; padding: 10px 24px; border-radius: 4px; "

"font-weight: bold; font-size: 13px;"

)

_STYLE_BTN_SEC = "background: #2a2a3e; color: #aaa; padding: 6px 12px; border-radius: 4px;"

_STYLE_COMBO = "background: #1a1a2e; color: #ccc; border: 1px solid #333; padding: 4px 8px; border-radius: 4px;"

class MeasurementPanel(QWidget):

def __init__(self, on_run: Callable, on_upload_lut: Callable, parent=None):

super().__init__(parent)

self._on_run = on_run

self._on_upload_lut = on_upload_lut

self._build_ui()

def _build_ui(self):

layout = QVBoxLayout(self)

layout.setContentsMargins(24, 24, 24, 24)

layout.setSpacing(16)

title = QLabel("Calibrate")

title.setStyleSheet("color: #fff; font-size: 18px; font-weight: bold;")

layout.addWidget(title)

# Device selection

device_group = QGroupBox("Measurement Devices")

device_group.setStyleSheet(

"QGroupBox { color: #aaa; font-size: 12px; border: 1px solid #333; "

"border-radius: 4px; margin-top: 8px; } QGroupBox::title { padding: 0 4px; }"

)

device_layout = QGridLayout(device_group)

device_layout.addWidget(self._lbl("Meter:"), 0, 0)

self._meter_combo = QComboBox()

self._meter_combo.setStyleSheet(_STYLE_COMBO)

self._meter_combo.addItem("— no meters detected —")

device_layout.addWidget(self._meter_combo, 0, 1)

scan_btn = QPushButton("Scan Meters")

scan_btn.setStyleSheet(_STYLE_BTN_SEC)

scan_btn.clicked.connect(self._on_scan_meters)

device_layout.addWidget(scan_btn, 0, 2)

device_layout.addWidget(self._lbl("Generator:"), 1, 0)

self._gen_combo = QComboBox()

self._gen_combo.setStyleSheet(_STYLE_COMBO)

self._gen_combo.addItems(["iTPG (Internal)", "PGenerator (External)"])

device_layout.addWidget(self._gen_combo, 1, 1)

device_layout.addWidget(self._lbl("PGenerator IP:"), 2, 0)

self._pgen_ip = QLineEdit("192.168.1.200")

self._pgen_ip.setStyleSheet("background: #1a1a2e; color: #fff; border: 1px solid #333; padding: 4px; border-radius: 4px;")

device_layout.addWidget(self._pgen_ip, 2, 1)

layout.addWidget(device_group)

# Sequence selection

seq_group = QGroupBox("Measurement Sequence")

seq_group.setStyleSheet(device_group.styleSheet())

seq_layout = QGridLayout(seq_group)

seq_layout.addWidget(self._lbl("Sequence:"), 0, 0)

self._seq_combo = QComboBox()

self._seq_combo.setStyleSheet(_STYLE_COMBO)

self._seq_combo.addItems([

"SDR Grayscale (21pt)",

"SDR Full (grayscale + primaries + secondaries)",

"HDR10 Grayscale",

"HDR10 Full",

])

seq_layout.addWidget(self._seq_combo, 0, 1)

seq_layout.addWidget(self._lbl("Target Color Space:"), 1, 0)

self._cs_combo = QComboBox()

self._cs_combo.setStyleSheet(_STYLE_COMBO)

self._cs_combo.addItems(["BT.709 (SDR)", "BT.2020 (HDR10)", "DCI-P3"])

seq_layout.addWidget(self._cs_combo, 1, 1)

layout.addWidget(seq_group)

# Progress

self._progress_bar = QProgressBar()

self._progress_bar.setRange(0, 100)

self._progress_bar.setValue(0)

self._progress_bar.setStyleSheet(

"QProgressBar { background: #1a1a2e; border: 1px solid #333; border-radius: 4px; height: 12px; }"

"QProgressBar::chunk { background: #4fc3f7; border-radius: 3px; }"

)

layout.addWidget(self._progress_bar)

self._status_label = QLabel("Ready")

self._status_label.setStyleSheet("color: #aaa; font-size: 12px;")

layout.addWidget(self._status_label)

# Run button row

run_row = QHBoxLayout()

self._run_btn = QPushButton("Start Measurement")

self._run_btn.setStyleSheet(_STYLE_BTN_RUN)

self._run_btn.clicked.connect(self._on_run_clicked)

run_row.addWidget(self._run_btn)

self._upload_btn = QPushButton("Generate & Upload LUT")

self._upload_btn.setStyleSheet(_STYLE_BTN_SEC)

self._upload_btn.setEnabled(False)

self._upload_btn.clicked.connect(self._on_upload_clicked)

run_row.addWidget(self._upload_btn)

run_row.addStretch()

layout.addLayout(run_row)

# Log output

self._log = QTextEdit()

self._log.setReadOnly(True)

self._log.setStyleSheet(

"background: #0d1117; color: #aaa; font-family: monospace; "

"font-size: 11px; border: 1px solid #333; border-radius: 4px;"

)

self._log.setFixedHeight(150)

layout.addWidget(self._log)

layout.addStretch()

def _lbl(self, text: str) -> QLabel:

l = QLabel(text)

l.setStyleSheet(_STYLE_LABEL)

return l

def populate_meters(self, devices: List[MeterDevice]):

self._meter_combo.clear()

if not devices:

self._meter_combo.addItem("— no meters detected —")

return

for d in devices:

self._meter_combo.addItem(f"{d.index}: {d.name}")

def set_progress(self, current: int, total: int, label: str = ""):

pct = int(current / total * 100) if total > 0 else 0

self._progress_bar.setValue(pct)

self._status_label.setText(f"Patch {current}/{total} — {label}")

def log(self, text: str):

self._log.append(text)

def set_running(self, running: bool):

self._run_btn.setEnabled(not running)

self._run_btn.setText("Measuring…" if running else "Start Measurement")

def enable_upload(self):

self._upload_btn.setEnabled(True)

def _on_scan_meters(self):

self._on_run("__scan_meters__")

def _on_run_clicked(self):

gen_use_itpg = "iTPG" in self._gen_combo.currentText()

pgen_ip = self._pgen_ip.text().strip()

seq_name = self._seq_combo.currentText()

self._on_run({

"action": "measure",

"use_itpg": gen_use_itpg,

"pgen_ip": pgen_ip,

"sequence": seq_name,

})

def _on_upload_clicked(self):

self._on_upload_lut()

☐  Step 4: Run tests to verify they pass

pytest tests/unit/test_measurement_panel.py -v

Expected: 4 tests pass.

☐  Step 5: Commit

git add app/ui/measurement_panel.py tests/unit/test_measurement_panel.py

git commit -m "feat: MeasurementPanel UI — device selection, sequence picker, progress bar, log"

## Task 10: Wire Measurement Workflow into MainWindow

Files:

Modify: app/ui/main_window.py

☐  Step 1: Wire MeasurementPanel into MainWindow

Add to imports in app/ui/main_window.py:

import asyncio

from app.ui.measurement_panel import MeasurementPanel

from app.meter.argyll import ArgyllReader, list_argyll_devices, ArgyllNotFoundError

from app.generator.itpg import iTPGGenerator

from app.generator.pgenerator import PGeneratorClient

from app.measurement.patches import build_sdr_full, build_hdr10_full, SDR_GRAYSCALE_21, PatchSequence

from app.measurement.session import MeasurementSession

from app.measurement.store import save_cgats, save_json

from app.lut_gen.tone_curve import generate_1d_lut_from_grayscale

from app.lut_gen.gamut import generate_3d_lut_from_measurements

from app.tv.upload import LUTUploader, LUTTarget

from pathlib import Path

import datetime

Add _measurement_results and _measurement_panel to MainWindow.__init__ after self._build_ui():

self._measurement_results = []

self._measurement_panel = MeasurementPanel(

on_run=self._on_measurement_action,

on_upload_lut=lambda: asyncio.ensure_future(self._upload_measurement_luts()),

)

Add _on_nav handling for "calibrate":

elif key == "calibrate":

self.set_content(self._measurement_panel)

Add measurement methods to MainWindow:

def _on_measurement_action(self, action):

if action == "__scan_meters__":

asyncio.ensure_future(self._scan_meters())

elif isinstance(action, dict) and action.get("action") == "measure":

asyncio.ensure_future(self._run_measurement(action))

async def _scan_meters(self):

try:

devices = list_argyll_devices()

self._measurement_panel.populate_meters(devices)

self._measurement_panel.log(f"Found {len(devices)} meter(s)")

except ArgyllNotFoundError as e:

self._measurement_panel.log(f"ArgyllCMS not found: {e}")

async def _run_measurement(self, config: dict):

if not self._managers:

self._measurement_panel.log("No TV connected")

return

mgr = next(iter(self._managers.values()))

if not mgr.is_connected:

self._measurement_panel.log("TV not connected")

return

# Build patch sequence

seq_name = config.get("sequence", "")

if "HDR10" in seq_name:

sequence = build_hdr10_full()

else:

sequence = build_sdr_full()

# Build generator

if config.get("use_itpg"):

generator = iTPGGenerator(client=mgr.client)

else:

pgen_ip = config.get("pgen_ip", "192.168.1.200")

generator = PGeneratorClient(host=pgen_ip)

# Build reader (use first detected meter, index 0 as default)

reader = ArgyllReader(device_index=0)

self._measurement_panel.set_running(True)

self._measurement_panel.log(f"Starting {sequence.name} — {len(sequence)} patches")

def on_progress(i, total, result):

self._measurement_panel.set_progress(i, total, result.patch.label)

self._measurement_panel.log(

f"  [{i}/{total}] {result.patch.label}: "

f"X={result.reading.X:.3f} Y={result.reading.Y:.3f} Z={result.reading.Z:.3f}"

)

try:

session = MeasurementSession(

generator=generator,

reader=reader,

sequence=sequence,

on_progress=on_progress,

)

self._measurement_results = await session.run()

timestamp = datetime.datetime.now().strftime("%Y%m%d_%H%M%S")

save_path = Path.home() / "Documents" / f"lg_cal_{timestamp}.cgats"

save_cgats(self._measurement_results, save_path)

self._measurement_panel.log(f"Saved: {save_path}")

self._measurement_panel.enable_upload()

except Exception as e:

self._measurement_panel.log(f"Measurement failed: {e}")

finally:

self._measurement_panel.set_running(False)

async def _upload_measurement_luts(self):

if not self._measurement_results:

self._measurement_panel.log("No measurement data — run a measurement first")

return

if not self._managers:

self._measurement_panel.log("No TV connected")

return

self._measurement_panel.log("Generating 1D tone curve LUT…")

try:

lut_1d = generate_1d_lut_from_grayscale(self._measurement_results)

self._measurement_panel.log("Generating 3D gamut LUT…")

lut_3d = generate_3d_lut_from_measurements(self._measurement_results, lut_size=17)

except Exception as e:

self._measurement_panel.log(f"LUT generation failed: {e}")

return

for ip, mgr in self._managers.items():

if not mgr.is_connected:

continue

uploader = LUTUploader(client=mgr.client, pic_mode=mgr.snapshot.pic_mode)

try:

await uploader.upload_1d(lut_1d)

self._measurement_panel.log(f"1D LUT uploaded to {ip}")

await uploader.upload_3d(lut_3d, target=LUTTarget.BT709)

self._measurement_panel.log(f"3D LUT uploaded to {ip}")

except Exception as e:

self._measurement_panel.log(f"Upload failed for {ip}: {e}")

☐  Step 2: Run all unit tests

pytest tests/unit/ -v

Expected: All tests pass.

☐  Step 3: Verify the app launches and Calibrate nav works

python -m app.main

Expected:

1. Window opens

2. Click "🎨 Calibrate" in sidebar — MeasurementPanel appears

3. Click "Scan Meters" — if ArgyllCMS is installed and a meter is connected, the meter dropdown populates

4. Without a meter: logs "ArgyllCMS not found" or "Found 0 meter(s)"

☐  Step 4: Final commit

git add app/ui/main_window.py

git commit -m "feat: wire MeasurementPanel into MainWindow — full calibration workflow end-to-end"

## Task 11: Hardware Integration Tests

Files:

Create: tests/hardware/test_meter_hardware.py

Create: tests/hardware/test_generator_hardware.py

☐  Step 1: Create meter hardware tests

# tests/hardware/test_meter_hardware.py

import pytest

from app.meter.argyll import ArgyllReader, list_argyll_devices

from app.meter.device import XYZReading

@pytest.mark.hardware

def test_list_devices_finds_meter():

"""Requires: ArgyllCMS installed, at least one meter connected via USB."""

devices = list_argyll_devices()

assert len(devices) > 0, "No meters found — is a meter connected and ArgyllCMS installed?"

@pytest.mark.hardware

async def test_take_single_reading():

"""Requires: meter connected, pointing at a white surface or display."""

reader = ArgyllReader(device_index=0, avg_count=1)

reading = await reader.take_reading()

assert isinstance(reading, XYZReading)

assert reading.Y > 0, "Y value is 0 — is the meter pointing at a lit surface?"

☐  Step 2: Create generator hardware tests

# tests/hardware/test_generator_hardware.py

import pytest

from app.generator.itpg import iTPGGenerator

from app.generator.pgenerator import PGeneratorClient

from app.tv.connection import ConnectionManager

TV_IP = "192.168.1.101"    # Update before running

PGEN_IP = "192.168.1.200"  # Update before running

@pytest.fixture

async def connected_mgr():

mgr = ConnectionManager(TV_IP)

await mgr.connect()

yield mgr

await mgr.disconnect()

@pytest.mark.hardware

async def test_itpg_displays_white_patch(connected_mgr):

gen = iTPGGenerator(client=connected_mgr.client)

async with gen:

await gen.set_patch(255, 255, 255)

# Visually verify: TV should show a white patch in the center

@pytest.mark.hardware

async def test_itpg_displays_red_patch(connected_mgr):

gen = iTPGGenerator(client=connected_mgr.client)

async with gen:

await gen.set_patch(255, 0, 0)

@pytest.mark.hardware

async def test_pgenerator_probe():

gen = PGeneratorClient(host=PGEN_IP)

result = await gen.probe()

assert result is True, f"PGenerator not reachable at {PGEN_IP}:8080"

@pytest.mark.hardware

async def test_pgenerator_displays_white():

gen = PGeneratorClient(host=PGEN_IP)

async with gen:

await gen.set_patch(255, 255, 255)

☐  Step 3: Run all unit tests one final time

pytest tests/unit/ -v

Expected: All tests pass.

☐  Step 4: Commit

git add tests/hardware/test_meter_hardware.py tests/hardware/test_generator_hardware.py

git commit -m "feat: hardware integration tests for meter and pattern generator"

## Running Tests

# All unit tests (no hardware needed)

pytest tests/unit/ -v

# Hardware meter tests (USB meter + ArgyllCMS required)

pytest tests/hardware/test_meter_hardware.py -v -m hardware

# Hardware generator tests (TV + PGenerator Pi required)

pytest tests/hardware/test_generator_hardware.py -v -m hardware

# Full measurement workflow test (everything connected)

pytest tests/hardware/ -v -m hardware

## End-to-End Calibration Flow

source .venv/bin/activate

python -m app.main

1. Connect TV — sidebar shows "● C1 Connected"

2. Click "🎨 Calibrate"

3. Click "Scan Meters" — i1 Display Pro or i1 Pro 2 appears in dropdown

4. Select generator (iTPG or PGenerator IP)

5. Select sequence (SDR Full)

6. Click "Start Measurement" — TV displays patches, meter reads each one, log fills in

7. After completion — CGATS file saved to ~/Documents/lg_cal_YYYYMMDD_HHMMSS.cgats

8. Click "Generate & Upload LUT" — 1D + 3D LUTs generated and uploaded to TV